| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0085 |
| --- | --- |

| Title: | Non-EE2: Direct block vector (DBV) mode extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Myungoh Hong Jangwon Choi Naeri Park Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung-Hwan Kim 10680 Treena St, San Diego, CA 92131, USA | Tel: Email: | +82-2-6912-6477 myungoh.hong@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes direct block vector (DBV) mode extension. Firstly, the number of locations to find a block vector are extended from 5 to 17 keeping the existing search range. Also, DBV candidate list is composed of block vector of collocated luma blocks that is coded with IBC or IntraTMP mode, and DBV candidate list is sorted in ascending order of SAD cost. The best one of DBV candidate list is used to apply block prediction for chroma. Experimental results of the proposed method for class F and class TGM are reported as follows:

For AI configuration:

F: -0.08%/-0.19%/-0.09%, 100.1%(EncT), 100.1%(DecT),

TGM: -0.10%/-0.15%/-0.24%, 99.3%(EncT), 99.4%(DecT).

For RA configuration:

F: -0.10%/0.06%/-0.49%, 100.4%(EncT), 100.0%(DecT),

TGM: -0.03%/-0.05%/-0.02%, 99.7%(EncT), 99.6%(DecT).

Introduction

The DBV mode [1] is a method to improve the coding efficiency of chroma intra prediction for screen content. For chroma components, when chroma dual tree is activated in intra slices, if one of the luma blocks (the following five locations in Fig. 1) is coded with IBC mode or IntraTMP mode, its block vector bvL is used and scaled to derive chroma block vector bvC. Here, the scaling factor depends on the chroma subsampling format. As depicted in Fig. 2, by using the position of the current chroma block (xCb, yCb) and the derived bvC, the corresponding offset position (xCb+ bvC[0] , yCb + bvC[1]) is determined, and a block copy prediction is performed.

Figure 1 5 locations in reconstructed luma samples

Figure 2 prediction process of DBV mode

Proposed methods

2 extensions are introduced on top of DBV mode derivation process in ECM-9.0

a) Construction of DBV candidate list

In order to support sufficient block vector candidates which could result in more accurate block vector, the number of locations to find block vector that is coded with IBC mode or IntraTMP mode are extended from 5 (C, TL, TR, BL and BR) to 17 as shown in Fig. 3. Please note that the existing search range for block vector prediction is not changed.

The 17 locations are as follow:

{

(0, 0), (W/2, 0), (W-1, 0),

(W/4, H/4), (W/2, H/4), (3*W/4, H/4),

(0, H/2), (W/4, H/2), (W/2, H/2), (3*W/4, H/2), (W-1, H/2),

(W/4, 3xH/4), (W/2, 3xH/4), (3xW/4, 3xH/4),

(0, H-1), (W/2, H-1), (W-1, H-1)

}

Figure 3 17 locations in reconstructed luma samples

After composing DBV candidate list, the candidates are sorted in ascending order of SAD cost using a template. If a template of at least one block vector in DBV candidate list is not available, DBV candidate list is not sorted. And if collocated luma block center position that is coded with IBC mode or IntraTMP mode is the same to chroma block center position after scaling, DBV candidate list is also not sorted.

After sorting DBV candidate list, a block vector bvL with the first index in DBV candidate list is scaled to derive chroma block vector bvC. By using the position of the current chroma block (xCb, yCb) and the derived bvC, the corresponding offset position (xCb+ bvC[0] , yCb + bvC[1]) is determined, and a block copy prediction is performed as shown in Fig. 2.

b) Signalling of DBV mode

If one or more luma blocks in 17 locations (as shown in Fig. 3) is coded with IBC mode or IntraTMP mode, a DBV flag is signaled as in ECM-9.0 to indicate whether the CU is coded as DBV mode or not. No additional signaling method is newly introduced and CABAC context for the DBV flag is also unchanged.

Experimental results

The proposed method is tested on top of ECM-9.0 under the common test conditions [2].

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.07% | 0.00% | 99.6% | 99.6% |
| Class A2 | -0.03% | -0.07% | 0.03% | 99.7% | 99.2% |
| Class B | 0.00% | 0.00% | 0.08% | 99.9% | 99.4% |
| Class C | -0.01% | -0.04% | 0.08% | 99.8% | 99.5% |
| Class E | -0.01% | -0.07% | 0.17% | 99.7% | 99.0% |
| Overall | -0.01% | -0.04% | 0.07% | 99.8% | 99.4% |
| Class F | -0.08% | -0.19% | -0.09% | 100.1% | 100.1% |
| Class TGM | -0.10% | -0.15% | -0.24% | 99.3% | 99.4% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.30% | -0.02% | 100.7% | 100.2% |
| Class A2 | -0.04% | 0.02% | 0.11% | 100.9% | 100.4% |
| Class B | 0.01% | -0.05% | 0.06% | 99.9% | 100.0% |
| Class C | 0.02% | -0.14% | -0.11% | 100.8% | 100.1% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.12% | 0.01% | 100.5% | 100.1% |
| Class F | -0.10% | 0.06% | -0.49% | 100.4% | 100.0% |
| Class TGM | -0.03% | -0.05% | -0.02% | 99.7% | 99.6% |

* ParkRunning3 qp22 result is copied from anchor

Conclusions

Direct block vector (DBV) mode extension is presented in this contribution. It is recommended to study the proposed DBV mode extension in the next round of EE.

References

- J.-Y Huo, X. Hao, Y.-Z Ma, F.-Z Yang, M. Li, L. Zhang, J. Ren, “EE2-3.1: Direct block vector mode for chroma prediction”, JVET-AC0071, January 2023.

- M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AD2017, April 2023.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).