| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0088-v1 |
| --- | --- |

| Title: | AHG9: A study on Generative Face Video SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Han Boon Teo Jing Yuan Thong Karlekar Jayashree Chong Soon Lim Kiyofumi Abe | Tel: Email: | +65-6550-5417 hanboon.teo@sg.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

A generative face video SEI message was presented in JVET-AC0088 at the 29th JVET meeting for compression of talking-face videos at ultra low bitrate, where at the 30th JVET meeting JVET-AD0051 further expanded its scope to encompass multiple facial representations. This contribution demonstrates the effectiveness of 3-D facial landmarks for generating talking-face videos. This document further highlights the framework’s independence of number of landmarks used for generation, and its interoperability of landmark representations across various systems.

# Introduction

Face re-enactment refers to the process of mapping expressions and pose of source persons to target persons, while simultaneously ensuring that the identity and attributes of the target person are being preserved. Face re-enactment techniques are currently adopted in a variety of applications, including video conferencing, social media as well as the gaming industry.

With the rise in popularity of generative networks, numerous works [1][2][3] have utilized models such as generative adversarial networks (GAN) to create talking-face videos. To enhance the photo-realistic aspect of the generated outputs, various methods such as projection of image features into latent spaces or extraction of motion representations have been introduced in recent works.

JVET-AC0088 [4] leveraged on the extraction of facial parameters of a 3D Morphable Model (3DMM) [5] from video frames, from which a generative face video SEI message was proposed to allow for the transmission of these facial parameters. Building on the proposed SEI message, JVET-AD0051 [6] expanded its scope to encompass multiple facial representations: 2-D landmarks, 2-D keypoints, 3-D keypoints and facial semantics.

This contribution taps on retrieving interpretable 3-D landmarks from video frames, from which these landmarks will be transformed and transmitted in the bitstream. This contribution aims to demonstrate how the architecture allows independence of the number of 3-D landmarks used to represent the facial attributes, thereby according flexibility and interoperability across systems with differing complexities, as different systems are not restricted to a specific number of landmarks.

In this contribution, all landmarks would refer to 3-D landmark representations unless otherwise specified.

# Exploration on Visual Quality

## Face Re-enactment using 3-D Landmarks

In this architecture, one or more source images are first encoded into the bitstream using the conventional video codec methods such as VVC, HEVC or AVC, from which the person-specific identity information will be extracted from the decoded source images at the decoder. For each subsequent video frame, geometric attributes like landmarks will be extracted, transformed, and encoded into the bitstream as SEI messages, from which these geometric attributes will be decoded and, together with the identity information and geometric attributes extracted from the source images, fed into the generative network to produce the subsequent frames of the talking-face video. Such landmarks allow for easy visualization of the facial attributes, and can be modified easily to produce the desired emotions and facial expressions.

To allow for compatibility with current codec standards, which require one picture data per access unit, a picture unit of the minimum allowed resolution can be included in each access unit that contains only SEI messages. This picture unit may be a unit of constant values for all pixels, to serve as a dummy picture data.

Note that only the geometric attributes are required to be extracted and transmitted for every frame, whilst the unique identity information can be extracted from the source image and thereafter universally reused during the generation of the entire video. By doing so, this significantly reduces the volume of data transferred between parties as minimal details are required to be transmitted per frame, thereby minimizing the bandwidth required.

Figure 1 illustrates a high-level overview of this model architecture.

Figure 1: Overview of Model Architecture for Face Re-enactment using 3-D Landmarks, Pictures obtained from [7]

## Impact of Landmark Representations on Re-enactment

This section analyzes the impact of landmark representations on the quality of the output reconstructed frames. Videos from the Voxceleb2 [7] dataset was used in this study.

Various frameworks [8][9] were employed to obtain face landmarks for all frames of each test video, from which subsets of these landmarks were extracted. These formed the landmark representations that were used for evaluation.

Figure 2 showcases a frame of a sample video, with landmarks from [8] rendered onto the face. Figure 3 displays some of the landmark representations that were used for evaluation.

Figure 2: (a) A Frame of a Sample Test Video, (b) 478 Landmarks Rendered on Face

Figure 3: Different Number of Landmarks extracted from Fig. 2(a) and Rendered onto a white frame – (a) 478, (b) 128, (c) 78

Subsequently, face parameters were extracted from different regions of landmarks. Some of the face parameters include head pose, and movements of the head and the core central face region. By extracting these face parameters, this leads to a reduction of the bitrate for transmission of geometric attributes information.

An example of the average number of bits to code each type of data during the transmission of 478 landmarks is displayed in Table 1. Note that there are no significant changes in the structure of data transmission across experiments of varying number of landmarks. Further, the number of bits for dummy picture data is set as 40 [6].

Table 1: Average Number of Bits for Each Type of Data per Frame

Sample outputs of the same-identity re-enactment for 3 test videos based on 478 3-D, 128 3-D and 78 3-D landmarks are highlighted in Figures 4, 5 and 6 below.

Figure 4: An Example of Qualitative Evaluation using different number of 3-D Landmarks for Same-identity reconstruction.

Figure 5: A Second Example of Qualitative Evaluation using different number of 3-D Landmarks for Same-identity reconstruction.

Figure 6: A Third Example of Qualitative Evaluation using different number of 3-D Landmarks for Same-identity reconstruction.

From the above study, it can be observed that the outputs generated from the model using different number of landmarks have similar visual quality as compared to the videos compressed using VVC at QP32. This demonstrates the versatility of the approach to cater for varying number of landmarks and hence entails the possibility of changing landmark detectors or number of landmarks in future iterations without significant impact to the generated outputs. The approach is also flexible in catering for different landmark representations among encoders and decoders, thereby providing compatibility across devices.

In addition, by comparing generated outputs against videos compressed using VVC at the same bitrate, it can be observed that the visual quality of generated outputs outperforms that of VVC at the same bitrate, highlighting the efficacy of the approach for transmissions at ultra low bitrates.

# Conclusion

The above studies demonstrate the effectiveness of utilizing 3-D landmarks for face re-enactment, and the presented approach is flexible against varying number of 3-D landmarks. This highlights the possible compatibility of face re-enactment across different devices adopting different landmark representations.

# References

- T. C. Wang, A. Mallya, and M. Y. Liu, “One-Shot Free-View Neural Talking-Head Synthesis for Video Conferencing”, IEEE/CVF Computer Vision and Pattern Recognition (CVPR), pp. 10039 – 10049, 2021.

- E. Zakharov, A. Ivakhnenko, A. Shysheya, and V. Lempitsky, “Fast Bi-layer Neural Synthesis of One-shot Realistic Head Avatars”, European Conference on Computer Vision (ECCV), pp. 524 – 540, 2020.

- E. Burkov, I. Pasechnik, A. Grigorev, and V. Lempitsky, “Neural Head Reenactment with Latent Pose Descriptors”, IEEE/CVF Computer Vision and Pattern Recognition (CVPR), pp. 13786 – 13795, 2020.

- B. Chen, J. Chen, S. R. Wang, Y. Ye, and S. Q. Wang, “AHG9: Generative Face Video SEI Message”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-AC0088, 2023.

- V. Blanz, and T. Vetter, “A Morphable Model For The Synthesis Of 3D Faces”, International Conference on Computer Graphics and Interactive Techniques (SIGGRAPH), pp. 187 – 194, 1999.

- B. Chen, J. Chen, Y. Ye, and S. Q. Wang, “AHG9: Common SEI Message of Generative Face Video”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-AD0051, 2023.

- J. S. Chung, A. Nagrani, and A. Zisserman, “VoxCeleb2: Deep Speaker Recognition”, Interspeech, 2018.

- I. Grischenko, A. Ablavatski, Y. Kartynnik, K. Raveendran, and M. Grundmann, “Attention Mesh: High-fidelity Face Mesh Prediction in Real-time”, CVPR Workshop on Computer Vision for Augmented and Virtual Reality (CV4ARVR), 2020.

- A. Bulat, and G. Tzimiropoulos, “How far are we from solving the 2D & 3D Face Alignment problem? (and a dataset of 230,000 3D facial landmarks)”, IEEE International Conference on Computer Vision (ICCV), pp. 1021 – 1030, 2017.

# Patent rights declaration(s)

Panasonic may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).