| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0097-v2 |
| --- | --- |

| Title: | AHG12: On the cross-component merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Vadim Seregin, Marta Karczewicz | Email: | hanhuang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to remove the template based offset update process for the cross-component merge candidate.

On top of ECM-9.0, the average BD-rate impact, encoder and decoder run times are summarized as follows {Y, U, V, EncT, DecT}:

AI: -0.01%, 0.01, 0.00%, 98.8%, 97.0%; class TGM -0.39%, -1.01%, -1.02%, 99.4%, 99.7%

RA: -0.02%, 0.00%, -0.22%, 99.8%, 99.9%

# Introduction

In the cross-component merge (CCMerge) mode in ECM-9.0, cross-component model parameters can be inherited from its spatial adjacent and non-adjacent neighbors, or default models. The chroma offset is further updated by applying the model to the template of a current block and calculating the average difference between the generated predictor and the reconstruction signal.  However, such design is contradictory to the assumption that the correlation between luma and chroma components in the current block is the same to a spatial adjacent or non-adjacent neighboring block instead of the template.

# Proposal

In this contribution, it is proposed to remove the template based offset update process for the cross-component merge candidate.

# Simulation results

The proposed change was implemented on top of ECM-9.0 and test results under common test conditions are shown in Table 1.

Table 1 Test results of proposed method under CTC

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | -0.08% | 97.9% | 96.8% |
| Class A2 | 0.00% | -0.20% | 0.06% | 99.1% | 94.5% |
| Class B | 0.01% | 0.09% | -0.04% | 99.2% | 99.0% |
| Class C | -0.02% | 0.01% | 0.07% | 97.3% | 92.3% |
| Class E | -0.02% | 0.04% | -0.02% | 100.6% | 103.2% |
| Overall | -0.01% | 0.01% | 0.00% | 98.8% | 97.0% |
| Class D | -0.01% | 0.01% | 0.07% | 97.1% | 89.8% |
| Class F | 0.01% | -0.10% | -0.14% | 98.9% | 96.7% |
| Class TGM | -0.39% | -1.01% | -1.02% | 99.4% | 99.7% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.06% | -0.56% | 99.3% | 98.2% |
| Class A2 | -0.02% | -0.25% | -0.12% | 100.2% | 101.7% |
| Class B | -0.01% | 0.19% | -0.19% | 101.0% | 101.3% |
| Class C | -0.01% | -0.09% | -0.06% | 98.5% | 98.1% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.00% | -0.22% | 99.8% | 99.9% |
| Class D | 0.01% | -0.28% | -0.22% | 98.6% | 98.6% |
| Class F | -0.03% | -0.21% | -0.20% | 101.2% | 100.9% |
| Class TGM |  |  |  |  |  |

# Patent rights declaration(s)

Qualcomm Incorporated does not have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).