| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0098-v2 |
| --- | --- |

| Title: | AHG12: On the chroma DBV mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Hongtao Wang, Vadim Seregin, Marta Karczewicz | Email: | hanhuang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to enable chroma DBV mode in single-tree configuration and to remove the template matching based refinement in chroma DBV mode. The simulation on top of ECM-9.0 reports the following results {Y, U, V, EncT, DecT}:

AI: -0.01%, -0.03, 0.02%, 100%, 95%

RA:

AI (single tree): -0.19%, -0.24%, -0.19%, 100%, 100%

# Problem statement

In the ECM-9.0, chroma DBV (Direct block vector) mode is only enabled when the current slice is intra slice and uses dual-tree block partitioning. In the case of single tree block partitioning, the chroma component does not have an option to perform intra block copy even when the luma component chooses block copy mode, i.e. IntraTMP.

The chroma DBV mode inherits a block vector from the collocated luma block, and then a template matching refinement process is performed to adjust the block vector.

# Proposal

In this contribution, it is proposed to enable chroma DBV mode in single-tree partitioning configuration. When the luma component is coded by IntraTMP mode, chroma coding can choose from DBV mode and cross-component prediction modes, the regular chroma intra prediction modes (DM, Planar, DC, and angular intra prediction modes) are disabled.  Note that when luma component is not coded by IntraTMP, the DBV mode is disabled (as there is no block vector from the collocated luma block).

It is also proposed to remove the template matching based refinement process in DBV mode.

# Simulation results

The proposed change was implemented on top of ECM-9.0 and test results are provided under common test conditions in Table 1, and for a single tree (DualITree is set to 0) in Table 2.

Table 1 Test results of proposed method under CTC

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.02% | 100.6% | 88.8% |
| Class A2 | -0.01% | -0.14% | 0.01% | 100.5% | 96.9% |
| Class B | 0.00% | -0.01% | 0.03% | 100.2% | 94.7% |
| Class C | -0.01% | -0.03% | 0.01% | 97.6% | 91.7% |
| Class E | -0.01% | 0.01% | 0.03% | 101.6% | 101.9% |
| Overall | -0.01% | -0.03% | 0.02% | 99.9% | 94.5% |
| Class D | 0.01% | -0.07% | -0.03% | 98.2% | 92.4% |
| Class F | -0.04% | -0.34% | -0.33% | 101.8% | 101.8% |
| Class TGM | 0.05% | -0.14% | -0.13% | 98.1% | 94.3% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.07% | 0.09% | 97.4% | 97.2% |
| Class C | -0.01% | -0.16% | -0.08% | 96.0% | 96.3% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.00% | -0.55% | -0.03% | 95.4% | 95.6% |
| Class F | -0.09% | 0.06% | -0.07% | 98.2% | 99.4% |
| Class TGM |  |  |  |  |  |

Table 2 Test results with DualITree set to 0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 DualITree off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.25% | -0.06% | 101.3% | 99.5% |  |
| Class A2 | -0.26% | -0.11% | 0.11% | 100.3% | 100.6% |  |
| Class B | -0.20% | -0.15% | -0.26% | 100.2% | 100.4% |  |
| Class C | -0.18% | -0.07% | 0.03% | 100.0% | 100.3% |  |
| Class E | -0.26% | -0.74% | -0.80% | 99.9% | 100.6% |  |
| Overall | -0.19% | -0.24% | -0.19% | 100.3% | 100.3% |  |
| Class D | -0.10% | -0.18% | -0.18% | 99.8% | 100.2% |  |
| Class F | -0.02% | 0.00% | 0.10% | 99.3% | 100.6% |  |
| Class TGM | -0.12% | -0.01% | 0.00% | 97.8% | 101.7% |  |

Table 3 shows the test result of simple enable chroma DBV in single tree for a single tree all intra configuration (DualITree is set to 0)

Table 3 Test results of simply enable DBV in single tree with DualITree set to 0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 DualITree off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.12% | 0.00% | 101.3% | 101.3% |
| Class A2 |  |  |  |  |  |
| Class B | -0.18% | -0.12% | -0.35% | 101.3% | 110.7% |
| Class C | -0.12% | -0.09% | -0.05% | 103.4% | 115.3% |
| Class E | -0.19% | -0.66% | -0.91% | 105.4% | 110.2% |
| Overall |  |  |  |  |  |
| Class D | -0.08% | -0.13% | -0.16% | 103.0% | 106.5% |
| Class F | 0.00% | -0.03% | -0.11% | 100.2% | 99.3% |
| Class TGM | -0.10% | -0.03% | -0.06% | 100.8% | 101.1% |

Table 4 shows the performance impact of removing template matching refinement process in Chroma DBV.

Table 4 Performance impact of removing template matching refinement process in chroma DBV

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.02% | 96.6% | 96.9% |
| Class A2 | -0.01% | -0.14% | 0.01% | 97.7% | 97.7% |
| Class B | 0.00% | -0.01% | 0.03% | 98.4% | 100.9% |
| Class C | -0.01% | -0.03% | 0.01% | 101.6% | 113.9% |
| Class E | -0.01% | 0.01% | 0.03% | 99.6% | 100.3% |
| Overall | -0.01% | -0.03% | 0.02% | 98.9% | 102.3% |
| Class D | 0.01% | -0.07% | -0.03% | 101.4% | 112.7% |
| Class F | -0.04% | -0.34% | -0.33% | 92.4% | 84.6% |
| Class TGM | 0.05% | -0.14% | -0.13% | 92.5% | 81.1% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.02% | -0.01% | 97.1% | 97.5% |
| Class A2 | -0.02% | -0.13% | -0.03% | 100.0% | 102.9% |
| Class B | -0.01% | 0.01% | 0.10% | 99.9% | 100.1% |
| Class C | -0.01% | -0.02% | 0.07% | 102.2% | 102.4% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.03% | 0.04% | 100.0% | 100.7% |
| Class D | 0.02% | -0.46% | 0.29% | 103.1% | 102.3% |
| Class F | -0.06% | 0.12% | -0.14% | 96.1% | 91.3% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).