| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0107-v2 |
| --- | --- |

| Title: | AHG8: Improvements of the BD-rate model using monotonic curve-fitting method |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Haiqiang Wang, Xiang Pan, Zizheng Liu, Xiaozhong Xu, Shan Liu | Email: | {walleewang, xavierxpan, zizhengliu } @tencent.com { xiaozhongxu, shanl} @global.tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution provides improvements of the BD-rate model used in current Excel templates. Specifically, we propose a monotonic fitted curve to replace the PCHIP (Piecewise Cubic Hermit Interpolation) curve for BD-rate and BD-metric calculation, such as BD-PSNR, BD-mAP, BD-MOTA, etc. The fitted curve guarantees monotonicity even if the input points are nonmonotonic, thus makes it possible to calculate BD-rate and BD-metric values in various testing conditions.

V2: change part of the statistics in Table 2, 3, and 4. Fix some figures on TVD AI condition.

Introduction

The BD-PSNR and BD-rate measurements [1] have been used by the video standardization community to compare the performance of codecs and tools herein. An extended version of BD-Rate/BD-PSNR proposed in [2] has been used in several WGs. It is explicitly required that the data points should be monotonic to derive valid values. We notice that, in most cases, the collected bitrate and PSNR data are monotonic and the calculation of BD-rate and BD-PSNR is feasible for evaluating the performance of codec and tools.

More recently, learning based modules, such as pre-processing, encoding, decoding and post-processing techniques are in the scope of exploration and standardization of some AhGs in WG4 and WG5. Unlike the conventional video codec, some machine task specific metrics, like mAP for object detection and MOTA for object tracking used in WG4 VCM exploration and standardization, are introduced as performance evaluation metrics. A long-standing issue that has been raised during the investigation relates to the non-monotonic behavior of reporting data. In previous studies, it was frequently reported that the mAP or MOTA values were not monotonic. As a result, the current excel template for VCM could not give valid values in such cases.

In VCM standardization conducted in WG4, it is also observed that the metric values in terms of mAP and MOTA are noisy. The noise of metric value boils down to several major aspects: 1) measurement error due to limited accuracy of the object detection or object tracking methods currently used in the VCM framework, 2) cross-platform discrepancy when inferenced with the same implementation and bit-exact bitstream due to different hardware architectures or floating-point precision, and 3) numerical difference as the reference software still updates regularly.

Fig. 1 gives an example of noisy observations on the Traffic sequence for the object detection task. Each x denotes a data point of one QP while the points with circles represent those selected as anchor points. The QP range evaluated in Fig. 1 is [14, 63]. It is expected to reduce the amount of randomness in a metric value with a more robust BD-rate calculation method to compare the performance of different solutions.

Fig 1. mAP vs. bitrate scatter plot of the Traffic sequence for object detection task.

Several efforts have been made to address this issue, including selecting points in the Pareto front, using a subset of 5 or 4 points in the low, middle or high bitrate regions, updating QP selections to make sure at least the anchor points are monotonic, etc. With these point selection methods, there is a higher probability that the selected points are monotonic and reporting with current excel template is feasible. However, it is unclear whether these methods would increase or decrease the uncertainty of BD-rate, which is the primary metric to compare the performance of different solutions.

Monotonic curve-fitting solution

For both of video coding for human vision and machine vision, one fundamental assumption about the Rate-Distortion curve is that the distortion value should be monotonically decreasing as the bitrate increases. Meanwhile, the distortion reaches saturation point when the bitrate becomes high enough. For example, the perceived quality has undistinguishable difference with its reference for human vision or the evaluated machine task accuracy is close to its reference for machine vision. Similarly, the phenomenon could also be observed for the very low bitrate region.

Fig. 2 Illustration of the Four Parameter Logistic curve

2.1 Target curve

We suggest using the Four Parameter Logistic (4PL) curve to model the nonlinear relationship between bitrate and a variety of distortion metrics. It is recommended in ITU-R BT.500 [3] to capture the relationship between subjective and objective quality scores of videos. Fig 2. gives an example of the 4PL curve shape using conceptual bitrate and PSNR data.  It follows a sigmoidal, or “s”-shaped curve that fits our assumption well. It is almost linear within a specific range of bitrate where the distortion metric value varies rapidly. Beyond this range, the response quickly saturates and reaches minimum and maximum. The logistic function could be described as

fx=a+b-a1+exp[-x-cd]

where a, b, c and d are the parameters to control the shape of the curve. Intuitively, a and b are the maximum and minimum values of f(x) when the bitrate approaches positive infinity or zero, respectively. c determines the displacement of the curve along the bitrate axis. d represents the steepness of the curve that determines how quickly f(x) rises or falls.

2.2 Curve fitting

Let (x_i, y_i) for i in [1, 2, .., N] denote the bitrate and metric value of a data point. N=6 is the number of QP points used in current configuration. The fitted curve could be obtained by solving a typical nonlinear curve-fitting problem. Parameters a, b, c, and d could be obtained by minimizing least square error between observed values and fitted values with any suitable optimization method.

mina,b,c, and di in [1, 2, .., N]yi-f(xi)2

Once the parameters of logistic curve are obtained, we use the same procedure to calculate BD-rate and BD-metric value. To be specific, for the metrics used in VCM, BD-mAP and BD-MOTA evaluate difference of integral along x-axis between fitted curve of anchor and test. Similarly, BD-rate mAP and BD-rate MOTA evaluate difference of integral along y-axis between fitted curve of anchor and test.

Experimental results

We tested the proposed metric, namely mBD-rate mAP, mBD-rate MOTA, mBD-mAP, and mBD-MOTA using data from CE 4.2 which was adopted in the 142 MPEG meeting. The protocol is straightforward:

- Take bitrate and mAP/MOTA data directly from CE 4.2 report.

- Conduct curve fitting using 6 points for anchor and test, respectively.

- Count number of valid values on a per method per dataset-config basis.

- Calculate mBD-mAP and mBD-MOTA for SFU and TVD datasets, respectively.

- Calculate mBD-rate mAP and mBD-rate MOTA for SFU and TVD datasets, respectively.

- Compare mBD-rate mAP versus BD-rate mAP 6 points, BD-rate Pareto mAP, also mBD-mAP versus BD-mAP for SFU dataset.

- Compare mBD-rate MOTA versus BD-rate MOTA 6 points, BD-rate Pareto MOTA, also mBD- MOTA versus BD-MOTA for TVD dataset.

Comparison results are given in Table 1 and Table 2 for the SFU dataset, and Table 3 and Table 4 for the TVD dataset, respectively.

Table 1. Number of valid values obtained from each method, 13 in total for SFU dataset.

|  | BD-rate |  |  | BD-mAP |  |
| --- | --- | --- | --- | --- | --- |
|  |  | BDExtend |  |  | BDExtend |
|  | mBD | 6 points | pareto | mBD | 6 points |
| SFU-AI | 11 | 7 | 13 | 13 | 10 |
| SFU-LD | 13 | 8 | 13 | 13 | 13 |
| SFU-RA | 13 | 8 | 13 | 13 | 13 |

*BDExtend data was obtained from reporting excel template

Table 2. Comparison of BD-rate mAP and BD-mAP values between proposed (mBD) and existing (BDExtend) implementations on the SFU dataset. Diff column indicates average of absolute difference from sequences that valid values are available. The % symbol represents the percentage symbol of BD-rate, not relative difference.

|  | Diff of BD-rate mAP |  | Diff of BD-mAP |
| --- | --- | --- | --- |
|  | mBD vs. BDExtend (6 points) | mBD vs. BDExtend (Pareto) | mBD vs. BDExtend (6 points) |
| SFU-AI | 5.56% | 4.30% | 1.27 |
| SFU-LD | 4.23% | 4.54% | 0.27 |
| SFU-RA | 3.83% | 4.40% | 0.21 |

*BDExtend data was obtained from reporting excel template

Table 3. Number of valid values obtained from each method, 7 in total for TVD dataset.

|  | BD-rate MOTA |  |  | BD- MOTA |  |
| --- | --- | --- | --- | --- | --- |
|  |  | BDExtend |  |  | BDExtend |
|  | mBD | 6 points | pareto | mBD | 6 points |
| TVD-AI | 4 | 0 | 4 | 2 | 2 |
| TVD-LD | 7 | 1 | 7 | 7 | 7 |
| TVD-RA | 7 | 1 | 7 | 7 | 7 |

*BDExtend data was obtained from reporting excel template

Table 4. Comparison of BD-rate and BD-mAP values between proposed (mBD) and existing (BDExtend) implementations on the TVD dataset. Diff column indicates average of absolute difference from sequences that valid values are available. The % symbol represents the percentage symbol of BD-rate, not relative difference.

|  | Diff of BD-rate MOTA |  | Diff of BD-MOTA |
| --- | --- | --- | --- |
|  | mBD vs. BDExtend (6 points) | mBD vs. BDExtend (Pareto) | mBD vs. BDExtend (6 points) |
| TVD-AI | N/A | N/A | N/A |
| TVD-LD | 0.016% | 5.44% | 0.316 |
| TVD-RA | 0.282% | 0.887% | 0.327 |

*BDExtend data was obtained from reporting excel template

Visualization of fitting results on the SFU dataset.

|  | RA | LD | AI |
| --- | --- | --- | --- |
| Traffic |  |  |  |
| ParkScene |  |  |  |
| Cactus |  |  |  |
| BaskballDrive |  |  |  |
| BQTerrace |  |  |  |
| BasketballDrill |  |  |  |
| BQMall |  |  |  |
| PartyScene |  |  |  |
| RaceHorsesC |  |  |  |
| BasketballPass |  |  |  |
| BQSquare |  |  |  |
| BlowingBubbles |  |  |  |
| RaceHorsesD |  |  |  |

Visualization of fitting results on the TVD dataset.

|  | RA | LD | AI |
| --- | --- | --- | --- |
| TVD-01_1 |  |  |  |
| TVD-01_2 |  |  |  |
| TVD-01_3 |  |  |  |
| TVD-02_1 |  |  |  |
| TVD-03_1 |  |  |  |
| TVD-03_2 |  |  |  |
| TVD-03_3 |  |  |  |

Comparison with BDExtend

We compare the accuracy of the proposed method with the existing solution in this section. As there is no ground truth of RD curve which either method could compare with, we compare the fitting error of mBD with the interpolation error of BDExtend method. The procedure is summarized as the following.

- For each sequence in SFU, encode every QP in RA mode and hybrid codec is used. Then evaluating the accuracy in terms of mAP for object detection task. The encoding QP range is [QP_min, QP_max], where QP_min and QP_max are the minimum and maximum QP value in current anchor QP set, respectively.

- Conduct PCHIP interpolation to get interpolated curve, CI.

- Conduct the proposed curve fitting method to get fitter curve, CF.

- For every QP within [QP_min, QP_max] but not in the anchor QP set, we evaluate the horizontal error from current point to CI and CF. Interpolation error or fitting error play a similar role of the integral difference used to calculate BD-rate.

- We compare average of absolute difference of interpolation error and fitting error for each sequence.

It is worth noting that anchor points were not taken into consideration when interpolation error is evaluated. Furthermore, points that have a mAP value above or below the fitted curve were not used to get fitting error. Both PCHIP and mBD were implemented with Python in this section.

Fig. 3 Overlay scatter plots of PCHIP interpolation and mBD curve fitting results. The data is obtained on the SFU dataset, run with VCM-RS 0.4, RA configuration and hybrid inner codec.

Fig. 3 gives interpolated curve and fitting curve for every sequence in the SFU set except Kimono. Text box at the right bottom corner gives average of absolute interpolation error and fitting error. It was further summarized in Table 3 for easy comparison.

Table 3. Comparison of interpolation error with fitting error. Column 2 and 3 indicates average of absolute error for BDExtend and mBD methods, respectively. N/A in column 2 is due to non-monotonical anchor results. A negative value in Gain/loss column indicates mBD has lower error than mBD

| Sequence | BDExtend (6 points) | mBD (6 points) | Gain/loss |
| --- | --- | --- | --- |
| Traffic | 0.1158 | 0.1105 | -0.0053 |
| ParkScene | 0.1288 | 0.0836 | -0.0452 |
| Cactus | N/A | 0.1392 |  |
| BaskballDrive | 0.0335 | 0.0643 | 0.0308 |
| BQTerrace | 0.0946 | 0.0719 | -0.0227 |
| BasketballDrill | 0.0377 | 0.0523 | 0.0146 |
| BQMall | 0.0486 | 0.0460 | -0.0026 |
| PartyScene | 0.0946 | 0.1065 | 0.0119 |
| RaceHorsesC | 0.0687 | 0.0692 | 0.0005 |
| BasketballPass | 0.0914 | 0.0984 | 0.007 |
| BQSquare | 0.1286 | 0.0907 | -0.0379 |
| BlowingBubbles | 0.0546 | 0.0652 | 0.0106 |
| RaceHorsesD | 0.1198 | 0.1085 | -0.0113 |
| AVG |  |  | -0.00413 |

Conclusions

It is observed that the proposed method has the following advantages: 1) It has a higher probability to give valid value than BDExtend (6 points) method. 2) It is more transparent and consistent as all 6 points are used than the BDExtend (pareto) method. 3) The fitted curve is comparable or a little bit closer to the underlying RD curve than BDExtend (6 points) method.

It is suggested to adopt the proposed mBD-mAP, mBD-MOTA, mBD-rate mAP, and mBD-rate MOTA calculation methods for VCM standardization.

References

- Bjontegaard, Gisle. "Calculation of average PSNR differences between RD-curves." ITU SG16 Doc. VCEG-M33 (2001).

- Tourapis, A. M., D. Singer, Y. Su, and K. Mammou. "BD-Rate/BD-PSNR excel extensions." ISO/IEC JTC1/SC29/WG11 M41482 (2017).

[3] Series, B. T. "Methodology for the subjective assessment of the quality of television pictures." Recommendation ITU-R BT 500 (2012): 500-13.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).