| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0115-v1 |
| --- | --- |

| Title: | Non-EE2: Unified intra CC-models parameters precision |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Franck Galpin, Karam Naser, Fabrice Le Léannec | Email: | philippe.bordes@interdigital.com, karam.naser@interdigital.com, franck.galpin@interdigital.com, fabrice.leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In ECM-9.0, several intra coding tools use cross-component (CC) prediction. However, some of these tools use 64-bits precision for storage and applying CC-models, while others use 32-bits. It is proposed to unify the CC-model parameters to use 32-bits precision. It is reported that the proposed change does not impact the RD performance of ECM-9.0: AI 0.00% -0.01% 0.01%, RA xx% xx% xx%.

# Proposal

Several coding tools of ECM are based on cross-component (CC) prediction. The CC-models allow predicting chroma samples from reconstructed luma samples. The CC-models are computed with reconstructed luma and chroma samples.

In ECM, the storage precision of the CC-model parameters differs depending on the type of CC-model:

- CCLM: integer 32-bits params

- GLM: integer 32-bits params

- CCCM: integer 64-bits params (CCCM, GLCCCM, NSCCCM, MDFCCCM)

In cross-component prediction (CCP) merge mode [1], the CC-models are transformed into 64-bits and next inherited.

It is proposed to unify the CC-model parameters using reduced 32-bits representation for storage and convolution. The derived 64-bits parameters are transformed into 32-bits. Next, when applying the CC-model, the convolution is made with 32-bits parameters and the accumulation uses 32-bits sum only. Using fewer number of bits decreases memory bandwidth and allows effective SIMD implementation.

# Experimental results

The proposal has been implemented in ECM-9.0 and tested with CTC-AI/RA conditions:

Table 1: Performance results of proposal in AI (32-bits).

| AI | Test vs ECM-9.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,00% | -0,03% | -0,03% | 100% | 100% |
| Class A2 | -0,01% | 0,00% | 0,02% | 100% | 100% |
| Class B | 0,00% | 0,00% | 0,01% | 100% | 100% |
| Class C | 0,00% | -0,02% | 0,02% | 100% | 100% |
| Class E | 0,00% | 0,00% | 0,03% | 100% | 100% |
| Overall | 0,00% | -0,01% | 0,01% | 100% | 100% |
| Class D | 0,00% | 0,03% | 0,00% | 100% | 100% |
| Class F | 0,02% | -0,01% | 0,01% | 100% | 100% |
| Class TGM | 0,00% | -0,08% | -0,05% | 100% | 100% |

Table 2: Performance results of proposal in RA (32-bits).

| RA | Test vs ECM-9.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,04% | 0,08% | 0,10% | 100% | 100% |
| Class A2 |  |  |  |  |  |
| Class B | 0,00% | -0,11% | 0,09% | 100% | 100% |
| Class C | 0,02% | -0,02% | -0,09% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0,02% | -0,12% | -0,22% | 100% | 100% |
| Class F | 0,02% | -0,02% | -0,08% | 100% | 100% |
| Class TGM | 0,02% | 0,02% | 0,00% | 100% | 100% |

One also tested use of CCCM using 16-bits params (CCCM, GLCCCM, NSCCCM, MDFCCCM) with CTC-AI conditions:

Table 3: Performance results of proposal in AI (16-bits).

| AI | Test vs ECM-9.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,00% | 0,05% | 0,02% | 100% | 100% |
| Class C | 0,00% | -0,04% | -0,01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0,02% | -0,07% | 0,02% | 100% | 100% |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

# Conclusion

It is proposed to unify the CC-model parameters using reduced 32-bits representation for storage and convolution. It is reported that the proposed change does not impact the RD performance of ECM-9.0.

It is proposed to use 32-bits representation for all the CC-model parameters in ECM.

# References

- Kai Zhang, Li Zhang, Zhipin Deng, “EE2-1.6: Non-local cross-component prediction and cross-component merge mode,” document JVET-AD0188, JVET 30th Meeting, Antalya, TR, 21–28 April 2023.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).