| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0129-v1 |
| --- | --- |

| Title: | AHG12 : Template-based CIIP weight derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Menghu Jia Yali Hu Ying Gao Shaowei Xie Cheng Huang | Tel: Email: | {jia.menghu, hu.yali, gao.ying, xie.shaowei, huang.cheng5 }@zte.com.cn |
| Source: | ZTE Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to derive the inter weight and intra weight of CIIP based on template matching cost. Given a CIIP coded CU, the TM cost values are calculated with different CIIP weights setting. Then, the CIIP weight with minimum TM cost value is selected to fusion intra prediction and inter prediction of CIIP. It is reported that on top of ECM-9.0, the overall coding performance impact are as follows:

- LDB: xx%, xx%, xx%

- RA: xx%, xx%, xx%

Introduction

In ECM-9.0, the weight derivation of CIIP includes three strategies, as shown in the table below. When PDPC is not enabled, the weight of CIIP is derived by neighboring blocks and intra prediction mode. TM derived weights are used in both the case of inter prediction weighting, i.e. BCW, and the case of intra prediction weighting, i.e. TIMD. CIIP, as a weighted intra prediction and inter prediction, can also use TM to derive weights.

Table 1. the weight derivation of CIIP of ECM9.0

| Ciip pdpc | Intra mode | Condition | Weight |
| --- | --- | --- | --- |
| 1 | dirMode=planar | Location:(x,y) | Weight depends on sample position |
| 0 | dirMode is derived by TIMD | dirMode=planar or dirMode = DC or width<4 or height<4 | Weight depends on the code mode of neighboring blocks |
|  |  | dirMode < DIA_IDX | Horizontal partitioning of coding blocks and applying fixed weights |
|  |  | others | Vertical partitioning of coding blocks and applying fixed weights |

Proposed methods

In this contribution, it is proposed to derive CIIP weight based on template matching cost.

- Obtain the intra prediction and inter prediction of the template. For the intra part of CIIP,Tintra the intra prediction of template are obtained by using intra mode of CIIP. For the inter part of CIIP, Tinterthe inter prediction of template are obtained by using inter mode of CIIP.

- Fuse the intra prediction and inter prediction of the template. For each group of weights{winter, wintra)={(1,3),(2,2),(3,1)}, use this weights to fusion the intra prediction and inter prediction of the template, and obtain the final predictionTpred of the template:

Tpred=winter∗Tinter+wintra∗Tintra+2≫2

Figure 1 The prediction process of CIIP

- Calculate the template cost for each set of weights.The template cost is obtained by calculating the SAD values of the predicted samples and reconstructed samples of the template.

- Select the weights corresponding to the template with the smallest SAD as the weights for CIIP.

The template cost is calculated based on the Y component and applies to all components. If PDPC is not enabled, the method introduced in this contribution will be enabled by default,and does not increase additional bit occupation.

Simulation results

This section reports the simulation results of the proposed method, according to common test conditions [1].

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.00% | 0.05% |  |  |
| Class C | 0.00% | -0.07% | 0.08% |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.06% | -0.07% | -0.08% |  |  |
| Class F |  |  |  |  |  |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.12% | -0.64% | -0.10% |  |  |
| Class E | 0.02% | 0.47% | 1.49% |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.07% | 0.70% | -0.89% |  |  |
| Class F |  |  |  |  |  |

Discussions

This contribution utilizes template information to derive the weights of CIIP, which is a straightforward extension of TIMD and TM-BCW. It is recommended for further study.

References

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Feb. 2022.

Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).