| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0121_v1 |
| --- | --- |

| Title: | Non-EE2: Luma Residual Taps in Chroma-ALF and CCALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Zhipin Deng1, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com zhipin.deng@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

In the current adaptive loop filter (ALF) design of ECM-9.0, luma residual values are stored and used in the Luma-ALF. In this contribution, luma residual values are further introduced into Chroma-ALF and CCALF.

On top of ECM-9.0, simulation results of the proposed method are reported as below:

RA: *0.02%, -0.56%, -0.67%, 99.8%, 100.0%.

LB: %, %, %, %, %.

Introduction

In ALF of ECM-9.0, a Chroma-ALF filter uses a diamond 9x9 filter shape with 20 taps in total, as shown in figure 2-1. All these taps take the spatial chroma reconstruction as input.

In CCALF of ECM-9.0, an online-trained CCALF filter uses a cross-liked filter shape with 25 taps in total, as shown in figure 1-2. All these taps take the spatial luma reconstruction as input.

Figure 2-1 The Currently Used Filter Shape of Chroma ALF.

Figure 2-2 The Currently Used Filter Shape of CCALF.

Proposed Method

The proposed methods are shown in Figure 3-1 and Figure 3-2. All the spatial based taps in Chroma-ALF and CCALF are kept unchanged.

For Chroma-ALF, only one luma residual tap is added. It takes the down-sampled luma residual as input.

For CCALF, five luma residual taps in a cross 3x3 shape are added. The extended taps take the co-located and neighbouring luma residual values as input.

The coefficient signaling mechanism is the same as the current design in ECM. The proposed method is always enabled without any filter shape switching.

Figure 3-1 The Newly Introduced Filter Shape for Chroma ALF.

Figure 3-2 The Newly Introduced Filter Shape for CCALF.

Simulation Results

The proposed method was implemented on top of the ECM-9.0 [1]. Simulations were performed following the common test conditions [2]. More detailed results could be found in the attached excel sheets. The results compared to ECM-9.0 are shown in TABLE 1. Please note that the proposed method is disabled in AI configuration.

TABLE 1 Performance of the Proposed Method

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.02% | -0.76% | -0.66% | 100.2% | 99.5% |  |  |  |  |  |
| Class C | 0.00% | -0.15% | 0.08% | 99.7% | 100.6% | 0.04% | -1.53% | -1.20% | 99.7% | 100.1% |
| Class E |  |  |  |  |  | -0.03% | -2.74% | -2.97% | 100.2% | 99.7% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.01% | 0.08% | 0.33% | 100.5% | 101.9% | 0.01% | -1.28% | -1.05% | 99.8% | 102.4% |
| Class F | -0.04% | -0.05% | -0.04% | 99.9% | 100.2% |  |  |  |  |  |

Conclusions

In this contribution, two luma residual-based chroma improvements for Chroma-ALF and CCALF are proposed. The proposed methods simply extend the usage of luma residuals into Chroma-ALF and CCALF. The proposed methods can achieve promising chroma coding performance with negligible encoding and decoding time increase. It is recommended to include the proposed method in the next round of EE2.

Reference

- ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-9.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.

Patent Rights Declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).