| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0122-v2 |
| --- | --- |

| Title: | AHG10: GOP-based RPR and scalable coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The purpose with the proposal is to fix so that GOP-based RPR works together with scalable coding in the VVC reference software. The fix consists of some updates to the PPS usage in VTM for GOP-based RPR and RPR in general. It is also suggested to include layer-wise control of GOP-based RPR such that it for example only can be used in the base layer. The contribution also provides compression performance of scalable coding with and without GOP-based RPR in the base layer for the case of 1.5x scalability for some candidate VT sequences.

Objective performance (BDR) versus VTM-20.2+ commit 85ea835777eb:

Impact of GOP-based RPR in base layer for 1.5x: Y -0.07%, U -0.65%, V -0.84%

It’s recommended to adopt the fix for GOP-based RPR such that RPR also can be used for scalable coding.

In v2 a fix for intra period was made for CorsairJoy was made.

Thanks to NHK for cross-check.

# Introduction

It has previously been shown that GOP-based RPR can provide objective and subjective benefits compared to encoding in source resolution for single layer coding at low bitrates. It is therefore of interest to also have this possibility for scalable coding.

# Proposal

The fix consists of including a unique PPS id for the scalable layers which are coded in a reduced resolution.

It is also suggested to add control of GOP-based RPR layer wise, for example only in base layer.

# Results

To check that the fix works GOP-based RPR is used in base layer in the test and compared to scalable coding when DMVREncMvSelect is enabled to avoid artifacts from DMVR. This is enabled by default when GOP-based RPR is used. The software is made on top of commit 85ea835777eb [1].

|  | Y | U | V |
| --- | --- | --- | --- |
| CorsairJoy_3840x2160_25p_420_10 | 0,04% | -4,09% | -1,86% |
| Fortnite_Part1_3840x2160_60fps_ | -0,30% | 0,01% | -0,26% |
| Procession_3840x2160_60p_420_10 | 0,31% | 0,69% | -3,21% |
| TallBuildings2_3840x2160_30fps_ | 0,00% | 0,00% | 0,00% |
| Marathon2_3840x2160_30fps_10bit | -0,27% | -0,41% | -0,49% |
| DrivingPOV3_3840x2160_60fps_10b | -0,29% | -0,72% | -0,07% |
| MountainBay2_3840x2160_30fps_42 | 0,00% | 0,00% | 0,00% |
| Average | -0,07% | -0,65% | -0,84% |

# Conclusion

It is suggested to update the VVC reference software such that RPR also works for scalable coding and that control of GOP-based RPR can be made layer-wise.

# References

[1] VTM-20.2 master, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/master (commit 85ea835777eb)

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).