| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0136-v2 |
| --- | --- |

| Title: | Non-EE2: Fix on TM-based reordering for affine MMVD mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Donghyun Kim Woong Lim Jongho Kim Sung-Chang Lim Jin Soo Choi 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5369 kimddng@etri.re.kr woong.lim@etri.re.kr pooney@etri.re.kr sclim@etri.re.kr jschoi@etri.re.kr |
| Source: | Electronics and Telecommunications Research Institute (ETRI) |  |  |

_____________________________

- Abstract

This contribution addresses a bug in the implementation of template matching (TM)-based reordering for affine merge with motion vector difference (MMVD) mode in ECM-9.0. In the affine MMVD mode, the refinement positions for the base candidate motion are reordered based on template costs calculated using the sum of absolute difference (SAD). However, during the reordering process, there is an issue where the incorrect reference picture index is used when calculating the picture order count (POC) of the reference picture from list 1, leading to inaccuracies in template SAD cost calculations. To resolve this issue, the proposed fix involves replacing the reference picture index of list 0 with the correct reference picture index of list 1 in "sortAffineMergeCandidates" function. Experimental results reportedly shows that the proposed fix has a negligible impact on coding performance in the RA case. Therefore, it is recommended to incorporate the proposed fix in the next version of ECM.

Introduction

In ECM-9.0, the affine merge with motion vector difference (MMVD) mode extends the concept of MMVD to affine merge mode by applying MMVD offset to the control point motion vectors (CPMVs) of a base candidate motion [1-2]. In the affine MMVD mode, all possible MMVD refinement positions for the base candidate motion are reordered based on the template cost, which is calculated using the sum of absolute difference (SAD). After reordering, only the top 1/2 refinement positions with the smallest template SAD costs are retained.

When the MMVD offset is applied to both the CPMVs of list 0 and list 1 in a bi-predictive base candidate motion, the MMVD offset is added to the CPMVs of list 0. And, for list 1, the refinement of CPMVs depends on whether both reference pictures from list 0 and list 1 are from the past or the future. This condition utilizes the picture order count (POC) of the reference picture from list 0 and the POC of the reference picture from list 1. Based on this condition, either the MMVD offset or its negation is added to the CPMVs of list 1.

However, there is an issue in the current implementation of ECM-9.0 where the reference picture index of list 0 is incorrectly used instead of the reference picture index of list 1 when calculating the POC of the reference picture from list 1. As a result, this leads to inaccuracies in the calculation of template SAD costs. To address this issue, it is proposed to fix the bug by using the correct reference picture index of list 1 to calculate the POC of the reference picture from list 1 during the reordering process.

Proposed Fix

In this contribution, it is proposed to fix a bug in ECM-9.0 related to template matching (TM)-based reordering for affine MMVD mode. The proposed fix aims to resolve inaccuracies in the calculation of template SAD costs by changing the reference index used to calculate the POC of the reference picture from list 1 in the reordering process.

Specifically, the fix involves modifying only one digit index in the source code of ECM-9.0. The necessary change involves replacing the reference picture index of list 0 with the reference picture index of list 1 in "sortAffineMergeCandidates" function. The relevant portion of the source code is as follows:

void InterPrediction::sortAffineMergeCandidates(PredictionUnit pu, AffineMergeCtx& affMrgCtx, uint32_t * affMmvdLUT, int16_t afMMVDIdx, bool fromStart) { ... //Store if (pu.interDir == 3) { if (dirIdx < 8) { posList0 = fPosPosition1 + 1; posList1 = 0; load0 = false; load1 = (fPosPosition1 != 0) ¦¦ (stepIdx != 0 && stepIdx != stepCutoff ); } else if (dirIdx < 16) { posList0 = 0; posList1 = fPosPosition1 + 1; load0 = (fPosPosition1 != 0) ¦¦ (stepIdx != 0 && stepIdx != stepCutoff ); load1 = false; } else { const int poc0 = pu.cu->slice->getRefPOC(REF_PIC_LIST_0, mvfMmvd[0][0].refIdx); #if FIX const int poc1 = pu.cu->slice->getRefPOC(REF_PIC_LIST_1, mvfMmvd[1][0].refIdx); #else const int poc1 = pu.cu->slice->getRefPOC(REF_PIC_LIST_1, mvfMmvd[0][0].refIdx); #endif * mvfMmvd[LX][cpIdx] const int currPoc = pu.cu->slice->getPOC(); if ((poc1 - currPoc)*(poc0 - currPoc) > 0) { posList0 = fPosPosition1 + 1; posList1 = fPosPosition1 + 1; load0 = true; load1 = true; } else { posList0 = fPosPosition1 + 1; posList1 = (fPosPosition1 ^ 1) + 1; load0 = true; load1 = true; } } } ... }

As shown in Figure 1, an incorrect MMVD offset can be applied to the CPMVs of list 1 in ECM-9.0. To address this problem, the proposed fix involves modifying the calculation of the picture order count (POC) of the reference picture from list 1 by replacing the reference picture index of list 0 with the reference picture index of list 1 during the reordering process. This fix ensures that the correct MMVD offset is applied to the CPMVs of list 1.

Figure 1. Example of the proposed fix

Experimental Results

The experiments were conducted using ECM-9.0 software [3] under JVET ECM common test conditions [4]. And, the experiments were performed on the platform of Intel Xeon Gold 6256 (3.6GHz) and 256GB RAM with GCC 9.4.0 compiler under Ubuntu 20.04.4 LTS.

The experimental results of the proposed fix are summarised in Table I. Based on the experimental results, the RA case shows negligible impact on coding performance. Furthermore, in the LDB case, the proposed fix does not result in any coding performance impact, since both reference pictures from list 0 and list 1 are always from the past. And, more detailed results can be found in accompanied Excel file.

Table I. Experimental result of the proposed fix

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.05% | 0.11% | 100.1% | 100.3% |
| Class A2 | -0.03% | -0.01% | -0.03% | 99.9% | #DIV/0! |
| Class B | 0.01% | -0.09% | -0.03% | 99.3% | 100.1% |
| Class C | 0.00% | -0.11% | -0.04% | 99.7% | 99.8% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.07% | -0.01% | 99.7% | #DIV/0! |
| Class D | 0.00% | -0.06% | -0.03% | 100.1% | 99.8% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Class C | 0.00% | 0.00% | 0.00% | 96.7% | 100.4% |
| Class E | 0.00% | 0.00% | 0.00% | 97.2% | 103.9% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Class D | 0.00% | 0.00% | 0.00% | 99.7% | 101.0% |

Conclusions

This contribution addresses a bug in the TM-based reordering for affine MMVD mode in ECM-9.0. The bug is related to the incorrect usage of the reference picture index when calculating the POC of the reference picture from list 1. The proposed fix involves replacing the reference picture index of list 0 with the reference picture index of list 1. Therefore, it is recommended to incorporate the proposed fix in the next version of ECM.

References

- M. Salehifar, Y. He, K. Zhang, N. Zhang, and L. Zhang, "On MMVD and Affine MMVD Extension," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-Z0142, 26th Meeting, by teleconference, Apr. 2022.

- G. Laroche, P. Onno, Y. Wang, M. Salehifar, K. Zhang, Y. He, N. Zhang, Z. Deng, L. Zhang, "EE2-2.6: Combination tests of Test 2.3, Test 2.4 and Test 2.5," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-AA0093, 27th Meeting, by teleconference, Jul. 2022.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-9.0

- M. Karczewicz and Y. Ye, "Common test conditions and evaluation procedures for enhanced compression tool testing," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-AD2017, 30th Meeting, Antalya, TR, Apr. 2023.

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).