| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0140-v2 |
| --- | --- |

| Title: | Non-EE2: LIC extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Kirill Suverov | Tel: Email: | afilippov@ofinno.com vrufitskiy@ofinno.com |
| Source: | Ofinno LLC |  |  |

_____________________________

# Abstract

The contribution proposes an additional method of filtering (interpolated) reference samples for inter predicted blocks. This method is complementary to LIC that utilizes 2-parametric linear model whereas the proposed technique involves high-order models like done in CCCM. On top of ECM-9.0, the following BD-rate gain and run-time results are reportedly obtained for the proposed method {Y, U, V, EncT, DecT}: {-0.XX%, -0.XX%, -0.XX%, 10X.X%, 10X.X%} and {-0.XX%, -0.XX%, -0.XX%, 10X.X%, 10X.X%} for RA and LDB configurations, respectively.

# Introduction

In ECM-9.0, a PU may be processed by local illumination compensation (LIC). The value of predicted sample is obtained by calculating linear model parameters and applying these parameters to interpolated reference samples.

LIC process is shown in Figure 1.

Figure 1. LIC reference samples processing.

# Proposed method

In addition to the LIC process, it is proposed to derive coefficients of a filter that is applied to the interpolated reference samples to get predicted sample values. An additional flag is used to switch between LIC and the proposed method.

When proposed method is selected, a filter is applied as shown in Figure 2.

Figure 2. Processing of reference samples according to the proposed method

A set of coefficients and an offset are derived from the template samples as exemplarily shown in Figure 3.

Figure 3. Template reference samples of a reference block.

Obtaining coefficients of the filter and an offset is performed similarly to CCCM method [1]. For a position of a filter window (shown in dashed line in Figure 3) in a reference samples template, the corresponding value of a reconstructed samples in the template neighboring to the current block is used.

# Simulation results

The proposed method was implemented on top of ECM-9.0 and are tested under CTC for enhanced compression tool testing [2] with Random-Access and Low-Delay B configurations. The simulation results are shown in Table 1.

Table 1. Simulation results of the proposed method in ECM-9.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.15% | -0.39% | -0.22% | 102.3% | 100.9% |
| Class C | -0.17% | -0.24% | -0.19% | 102.9% | 100.2% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.06% | -0.07% | -0.08% | 102.2% | 100.7% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | -0.18% | -0.27% | -0.32% | 103.0% | 100.6% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.17% | -0.22% | -0.24% | 104.0% | 100.8% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

This contribution proposes an additional method of filtering (interpolated) reference samples for inter predicted blocks. We recommend this technique for studying in the next round of EE2.

# References

- M. Coban, R.-L. Liao, K. Naser, J. Ström, and L. Zhang, “Algorithm description of Enhanced Compression Model 9 (ECM 9),” JVET-AD2025, July 2023.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AD2017, April 2022.

# Patent rights declaration(s)

Ofinno LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).