| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0142-v1 |
| --- | --- |

| Title: | AHG9: On derivation of NNPF input pictures and the value of nnpfc_purpose |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Jia, Ye-Kui Wang, Jizheng Xu, Li Zhang 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | wei.jia@bytedance.com yekui.wang@bytedance.com xujizheng@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes, three on the latest interface text of using the NNPFC SEI message in JVET-AD2005-v1 for the derivation of input pictures to an NNPF, one on the value of nnpfc_purpose in the semantics of the NNPFC SEI message in JVET-AD2006-v1:

- As an asserted bug fix, in the following sentence, replace "numInputPics − j − 1" with "numInputPics − 1":

When numInputPics is greater than 1, the following applies for each value of i in the range of j + 1 to numInputPics − j − 1, inclusive, in increasing order of i: ...

- As an asserted bug fix, in the following sentence, replace "inputPic" with "resampledPic":

If sourceWidth is equal to CroppedWidth and sourceHeight is equal to CroppedHeight, inputPic is set to be the same as sourcePic.

- As an asserted bug fix, condition the following constraint on "when pictureRateUpsamplingFlag is equal to 1", because currently this constraint can also be applied for cases where pictureRateUpsamplingFlag is equal to 0:

It is a requirement of bitstream conformance that num_interpolated_pics[ i − 1 ] shall not be greater than 0.

- As an asserted bug fix, specify that all NNPFC SEI messages with a particular value of nnpfc_id within a CLVS shall have the same value of nnpfc_purpose.

# Bug fixes on derivation of NNPF input pictures

## Introduction

The process of deriving input pictures is currently specified as follows.

–	If pictureRateUpsamplingFlag is equal to 1, the NNPFA SEI message that activated this NNPF has nnpfa_persistence_flag equal to 1, nnpfc_interpolated_pics[ i ] is greater than 0 only for a single value of i that is greater than 0, and currPic is the last picture of the bitstream in output order that has nuh_layer_id equal to currLayerId, the variable numPostRoll is set equal to the value of i such that nnpfc_interpolated_pics[ i ] is greater than 0, and the variable numInferences is set equal to 1 + numPostRoll.

–	Otherwise, the variable numInferences is set equal to 1.

For each value of j in the range of 0 to numInferences − 1, inclusive, the following applies:

–	The arrays inputPic[ i ] and inputPresentFlag[ i ] for i in the range of 0 to numInputPics − 1, inclusive, representing all the input pictures and the presence of input pictures within the bitstream, respectively, are specified as follows:

–	When j is greater than 0, for each value of k in the range of 0 to j − 1, inclusive, inputPic[ k ] is set to be currPic and inputPresentFlag[ k ] is set equal to 0.

–	The j-th input picture, inputPic[ j ], is set equal to be currPic and inputPresentFlag[ j ] is set equal to 1.

–	When numInputPics is greater than 1, the following applies for each value of i in the range of j + 1 to numInputPics − j − 1, inclusive, in increasing order of i: ...

However, the current value range of i when numInputPics is greater than 1 may lead to early termination of the loop, such that when numInputPics is greater than 1 and numInferences is greater than 1, some of the input pictures are not derived. For example, when numInputPics is equal to 4 and j is equal to 1, i would be in the range of j + 1, i.e. 2 to numInputPics − j − 1, i.e. 2. Therefore, inputPic[3] can never be derived. A detailed analysis of this issue with a concrete example is provided below in Subsection 1.2.

In addition, in the following sentence, "inputPic" should be "resampledPic":

–	If sourceWidth is equal to CroppedWidth and sourceHeight is equal to CroppedHeight, inputPic is set to be the same as sourcePic.

Furthermore, the following constraint should be conditioned on "when pictureRateUpsamplingFlag is equal to 1", because currently this constraint can also be applied for cases where pictureRateUpsamplingFlag is equal to 0:

–	It is a requirement of bitstream conformance that num_interpolated_pics[ i − 1 ] shall not be greater than 0.

## Example and analysis

An analysis of the first issue with a concrete example before and after making the first proposed change (i.e., replacing "numInputPics − j − 1 " with "numInputPics − 1") is provided.

Figure 1 An exmaple of interpolating a picture

To verify the proposed method, for example, as shown in Fig 1,

Suppose pictureRateUpsamplingFlag = 1, numInputPics = 4
nnpfc_interpolated_pics[ 0/1/2 ] = 0 1 0
CurrPic: pic8 (last pic in the bitstream)
Input pics: pic8, pic6, pic4, pic2
numPostRoll = 1
numInferences = 2

Let's first assume that there is no frame packing arrangement (FPA) SEI message at all.

//Before modifying "numInputPics − j − 1" to "numInputPics − 1":

//Loop for j = [0, numInferences − 1] = [0, 1]
j = 0
	inputPic[ 0 ] = currPic = pic8, inputPresentFlag[ 0 ] = 1

	//Loop for i = [j + 1, numInputPics − j − 1] = [1, 3]
		i = 1
			prevPic = pic6, thus inputPic[ 1 ] = pic6 and inputPresentFlag[ 1 ] = 1
		i = 2
			prevPic = pic4, thus inputPic[ 2 ] = pic4 and inputPresentFlag[ 2 ] = 1
		i = 3
			prevPic = pic2, thus inputPic[ 3 ] = pic2 and inputPresentFlag[ 3 ] = 1
	//Loop for i = [0, numInputPics − 1] = [0, 3]
		i = 0
			sourcePic = inputPic[ i ] = inputPic[ 0 ]
			resampledPic = sourcePic
		i = 1
			sourcePic = inputPic[ i ] = inputPic[ 1 ]
			resampledPic = sourcePic
		i = 2
			sourcePic = inputPic[ i ] = inputPic[ 2 ]
			resampledPic = sourcePic
		i = 3
			sourcePic = inputPic[ i ] = inputPic[ 3 ]
			resampledPic = sourcePic

j = 1
	k = 0, inputPic[ 0 ] = currPic = pic8, inputPresentFlag[ 0 ] = 0
	inputPic[ 1 ] = currPic = pic8, inputPresentFlag[ 1 ] = 1
	//Loop for i = [j + 1, numInputPics − j − 1] = [2, 2] = 2
		i = 2
			since inputPic[ 1 ] = pic8, so prevPic = pic6,
			therefore inputPic[ 2 ] = pic6, inputPresentFlag[ 2 ] = 1
		//As we can see, i = 3 is missing, with inputPic[ 3 ] = pic4, inputPresentFlag[ 3 ] = 1
	//Loop for i = [0, numInputPics − 1] = [0, 3]
		i = 0
			sourcePic = inputPic[ i ] = inputPic[ 0 ] or 0
			resampledPic = sourcePic
		i = 1
			sourcePic = inputPic[ i ] = inputPic[ 1 ]
			resampledPic = sourcePic
		i = 2
			sourcePic = inputPic[ i ] = inputPic[ 2 ]
			resampledPic = sourcePic
		i = 3
			sourcePic = inputPic[ i ] = inputPic[ 3 ]
			resampledPic = sourcePic

//After modifying "numInputPics − j − 1" to "numInputPics − 1":

//Loop for j = [0, numInferences − 1] = [0, 1]
j = 0
	inputPic[ 0 ] = currPic = pic8, inputPresentFlag[ 0 ] = 1

	//Loop for i = [j + 1, numInputPics − j − 1] = [1, 3]
		i = 1
			prevPic = pic6, thus inputPic[ 1 ] = pic6 and inputPresentFlag[ 1 ] = 1
		i = 2
			prevPic = pic4, thus inputPic[ 2 ] = pic4 and inputPresentFlag[ 2 ] = 1
		i = 3
			prevPic = pic2, thus inputPic[ 3 ] = pic2 and inputPresentFlag[ 3 ] = 1
	//Loop for i = [0, numInputPics − 1] = [0, 3]
		i = 0
			sourcePic = inputPic[ i ] = inputPic[ 0 ]
			resampledPic = sourcePic
		i = 1
			sourcePic = inputPic[ i ] = inputPic[ 1 ]
			resampledPic = sourcePic
		i = 2
			sourcePic = inputPic[ i ] = inputPic[ 2 ]
			resampledPic = sourcePic
		i = 3
			sourcePic = inputPic[ i ] = inputPic[ 3 ]
			resampledPic = sourcePic

j = 1
	k = 0, inputPic[ 0 ] = currPic = pic8, inputPresentFlag[ 0 ] = 0
	inputPic[ 1 ] = currPic = pic8, inputPresentFlag[ 1 ] = 1
	//Loop for i = [j + 1, numInputPics − j − 1] = [2, 3]
		i = 2
			since inputPic[ 1 ] = pic8, so prevPic = pic6,
			therefore inputPic[ 2 ] = pic6, inputPresentFlag[ 2 ] = 1
		//With the above change of "numInputPics − j − 1" to "numInputPics − 1",
			// now i = 3 is not missing, it is correct!
		i = 3
			since inputPic[ 2 ] = pic6, so prevPic = pic4,
			therefore inputPic[ 3 ] = pic4, inputPresentFlag[ 3 ] = 1
	//Loop for i = [0, numInputPics − 1] = [0, 3]
		i = 0
			sourcePic = inputPic[ i ] = inputPic[ 0 ] or 0
			resampledPic = sourcePic
		i = 1
			sourcePic = inputPic[ i ] = inputPic[ 1 ]
			resampledPic = sourcePic
		i = 2
			sourcePic = inputPic[ i ] = inputPic[ 2 ]
			resampledPic = sourcePic
		i = 3
			sourcePic = inputPic[ i ] = inputPic[ 3 ]
			resampledPic = sourcePic

## Proposed text changes

The proposed text changes are provided in this section, where additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts.

### D.12.11	Use of the neural network post-filter characteristics SEI message

…

The variable numInferences is derived as follows:

–	If pictureRateUpsamplingFlag is equal to 1, the NNPFA SEI message that activated this NNPF has nnpfa_persistence_flag equal to 1, nnpfc_interpolated_pics[ i ] is greater than 0 only for a single value of i that is greater than 0, and currPic is the last picture of the bitstream in output order that has nuh_layer_id equal to currLayerId, the variable numPostRoll is set equal to the value of i such that nnpfc_interpolated_pics[ i ] is greater than 0 and the variable numInferences is set equal to 1 + numPostRoll.

–	Otherwise, the variable numInferences is set equal to 1.

For each value of j in the range of 0 to numInferences − 1, inclusive, the following applies:

–	The arrays inputPic[ i ] and inputPresentFlag[ i ] for i in the range of 0 to numInputPics − 1, inclusive, representing all the input pictures and the presence of input pictures within the bitstream, respectively, are specified as follows:

–	When j is greater than 0, for each value of k in the range of 0 to j − 1, inclusive, inputPic[ k ] is set to be currPic and inputPresentFlag[ k ] is set equal to 0.

–	The j-th input picture, inputPic[ j ], is set equal to be currPic and inputPresentFlag[ j ] is set equal to 1.

–	When numInputPics is greater than 1, the following applies for each value of i in the range of j + 1 to numInputPics − j − 1 numInputPics − 1, inclusive, in increasing order of i:

–	If pictureRateUpsamplingFlag is equal to 1, currPic is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and a particular value of fp_current_frame_is_frame0_flag, and there is a cropped decoded output picture prevPic that is the last picture in output order among all cropped decoded output pictures that have nuh_layer_id equal to currLayerId, precede inputPic[ i − 1 ] in output order, and are associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag, inputPic[ i ] is set to be prevPic and inputPresentFlag[ i ] is set equal to 1.

–	Otherwise, if pictureRateUpsamplingFlag is equal to 0 and there is a picture prevPic that is the last picture in output order among all cropped decoded output pictures that have nuh_layer_id equal to currLayerId and precede inputPic[ i − 1 ] in output order, inputPic[ i ] is set to be prevPic and inputPresentFlag[ i ] is set equal to 1.

–	Otherwise, if pictureRateUpsamplingFlag is equal to 0 and there is a picture prevPic that is the last picture in output order among all cropped decoded output pictures that have nuh_layer_id equal to currLayerId and precede inputPic[ i − 1 ] in output order, inputPic[ i ] is set to be prevPic and inputPresentFlag[ i ] is set equal to 1.

–	Otherwise, if currPic is not associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and there is a cropped decoded output picture prevPic that is the last picture in output order among all cropped decoded output pictures that have nuh_layer_id equal to currLayerId and precede inputPic[ i − 1 ] in output order, inputPic[ i ] is set to be prevPic and inputPresentFlag[ i ] is set equal to 1.

–	Otherwise, the following applies:

–	inputPic[ i ] is set to be the same picture as inputPic[ i − 1 ] and inputPresentFlag[ i ] is set equal to 0.

–	It is a requirement of bitstream conformance that, when pictureRateUpsamplingFlag is equal to 1, num_interpolated_pics[ i − 1 ] shall not be greater than 0.

–	For purposes of interpretation of the NNPFC SEI message, the following variables are specified:

–	…

–	Otherwise (numInputPics is greater than 1), the following applies:

–	The variable sourceWidth is set equal to the value of pps_pic_width_in_luma_samples − ‌SubWidthC * ( pps_conf_win_left_offset + pps_conf_win_right_offset ) for sourcePic.

–	The variable sourceHeight is set equal to the value of pps_pic_height_in_luma_samples − ‌SubHeightC * ( pps_conf_win_top_offset + pps_conf_win_bottom_offset ) for sourcePic.

–	If sourceWidth is equal to CroppedWidth and sourceHeight is equal to CroppedHeight, inputPic resampledPic is set to be the same as sourcePic.

–	Otherwise (sourceWidth is not equal to CroppedWidth or sourceHeight is not equal to CroppedHeight), the following applies:

–	There shall be an NNPF, hereafter referred to as the super resolution NNPF, that is defined by at least one NNPFC SEI message, is activated by an NNPFA SEI message for sourcePic, and has nnpfc_purpose equal to 4, nnpfc_pic_width_in_luma_samples equal to CroppedWidth and nnpfc_pic_height_in_luma_samples equal to CroppedHeight.

–	resampledPic is set to be the output of the neural-network inference of the super resolution NNPF with sourcePic being an input.

…

# On the value of nnpfc_purpose

## Introduction

The following constraint is specified when nnpfc_base_flag is equal to 0:

When an NNPFC SEI message nnpfcCurr is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, is not a repetition of the first NNPFC SEI message with that particular nnpfc_id (i.e., the value of nnpfc_base_flag is equal to 0), and the value of nnpfc_property_present_flag is equal to 1, the following constraints apply:

–	The value of nnpfc_purpose in the NNPFC SEI message shall be the same as the value of nnpfc_purpose in the first NNPFC SEI message, in decoding order, that has that particular nnpfc_id value within the current CLVS.

However, the constraint above is only specified when nnpfc_property_present_flag is equal to 1. When nnpfc_property_present_flag is equal to 0, the value of nnpfc_purpose in the NNPFC SEI message is not constrained, which is wrong.

## Proposed text changes

The proposed text changes are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts.

      - Neural-network post-filter characteristics SEI message semantics

...

nnpfc_purpose indicates the purpose of the NNPF as specified in Table 20, where ( nnpfc_purpose & bitMask ) not equal to 0 indicates that the NNPF has the purpose associated with the bitMask value in Table 20. When nnpfc_purpose is greater than 0 and ( nnpfc_purpose & bitMask ) is equal to 0, the purpose associated with the bitMask value is not applicable to the NNPF. When nnpfc_pupose is equal to 0, the NNPF may be used as determined by the application.

All NNPFC SEI messages with a particular value of nnpfc_id within a CLVS shall have the same value of nnpfc_purpose.

The value of nnpfc_purpose shall be in the range of 0 to 63, inclusive, in bitstreams conforming to this edition of this document. Values of 64 to 65 535, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 64 to 65 535, inclusive.

...

nnpfc_property_present_flag equal to 1 specifies that syntax elements related to the filter purpose, input formatting, output formatting, and complexity are present. nnpfc_property_present_flag equal to 0 specifies that no syntax elements related to the filter purpose, input formatting, output formatting, and complexity are present.

When nnpfc_base_flag is equal to 1, nnpfc_property_present_flag shall be equal to 1.

When nnpfc_property_present_flag is equal to 0, the values of all syntax elements that may be present only when nnpfc_property_present_flag is equal to 1 are inferred to be equal to their corresponding syntax elements, respectively, in the NNPFC SEI message that contains the base NNPF for which this SEI message provides an update.

When an NNPFC SEI message nnpfcCurr is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, is not a repetition of the first NNPFC SEI message with that particular nnpfc_id (i.e., the value of nnpfc_base_flag is equal to 0), and the value of nnpfc_property_present_flag is equal to 1, the following constraints apply:

–	The value of nnpfc_purpose in the NNPFC SEI message shall be the same as the value of nnpfc_purpose in the first NNPFC SEI message, in decoding order, that has that particular nnpfc_id value within the current CLVS.

–	The values of syntax elements following nnpfc_property_present_flag and preceding nnpfc_complexity_info_present_flag, in decoding order, in the NNPFC SEI message shall be the same as the values of corresponding syntax elements in the first NNPFC SEI message, in decoding order, that has that particular nnpfc_id value within the current CLVS.

–	Either nnpfc_complexity_info_present_flag shall be equal to 0 or both nnpfc_complexity_info_present_flag shall be equal to 1 in the first NNPFC SEI message, in decoding order, that has that particular nnpfc_id value within the current CLVS (denoted as nnpfcBase below) and all the following apply:

…

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).