| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0148-v2 |
| --- | --- |

| Title: | Non-EE2: Affine subblock BDOF refinement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi Zhang, Han Huang, Jian-Liang Lin, Vadim Seregin, Marta Karczewicz (Qualcomm) | Tel: Email: | zhizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to apply BDOF to refine subblock MV for an affine coded block. When an affine coded block meets the BDOF enabling condition and it is decided to do subblock MC, BDOF is applied to refine each subblock MV and do sample adjustments. The refined MVs are stored for future block MV prediction. The proposed method is also applied to SbTMVP mode.

It also proposes the AMVP-Merge mode for affine block. When a block is decided to be AMVP-Merge mode, a flag is signalled to further indicate the block is an affine block or not.

The proposed method was implemented on top of ECM-9.0 and the test results are as follows:

BDOF affine subblock MV refinement and AMVP-Merge mode for affine:

RA : -0.19% (Y), -0.21% (U), -0.13% (V), 107.4% (EncT), 104.3% (DecT)

BDOF affine subblock MV refinement:

RA : -0.14% (Y), -0.17% (U), -0.07% (V), 103.0% (EncT), 104.4% (DecT)

# Introduction

When a non-affine coded block meets BDOF condition, BDOF subblock MV refinement and sample adjustment is applied to the block.

An affine coded block will do subblock MC when OBMC is applied, the MV of each subblock is derived from the affine model and the luma sample is refined by applying PROF.

AMVP-Merge mode is a bi-pred mode where one predictor is signaled as AMVP mode, and the other predictor is derived from merge list without signalling.

In ECM-9.0, BDOF and AMVP-merge mode are not applied to an affine coded block.

# Proposal

This contribution proposed to apply BDOF subblock MV refinement and sample adjustment to an affine coded block when the block meets the BDOF condition and the block is determined to use subblock MC, e.g., OBMC is applied. It also proposes to apply BDOF to SbTMVP coded block, when the entire or a subarea of the block meets the BDOF condition.

An affine coded block derives MVs for each 4×4 subblock from the affine model. The BDOF process starts with the 4×4 subblocks grouping with identical MVs. When the grouped subblock size is less than 256, BDOF MV refinement is processed in 4×4 subblock grid, and otherwise in 8×8 subblock grid.

The BDOF enabling condition is same as ECM-9.0, e.g., two reference pictures have equal POC distance to the current picture, and equal weight prediction.

It also proposes the AMVP-Merge mode for affine blocks. Like the AMVP-Merge mode in ECM-9.0, it signals reference picture list index, reference picture index and MVP index to indicate the AMVP predictor. The merge candidate is derived by using ARMC reordering. A flag is signalled to indicate an AMVP-merge block is affine coded block or not when the block size is equal or greater than 8×8 and the current picture is not a low-delay picture.

# Simulation results

## BDOF refine affine subblock MV and AMVP-Merge mode for affine:

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.14% | -0.11% | 107.8% | 102.7% |
| Class A2 | -0.34% | -0.29% | -0.28% | 107.3% | 106.9% |
| Class B | -0.16% | -0.17% | -0.04% | 107.5% | 104.6% |
| Class C | -0.21% | -0.25% | -0.15% | 107.0% | 103.1% |
| Class E |  |  |  |  |  |
| Overall | -0.19% | -0.21% | -0.13% | 107.4% | 104.3% |
| Class D | -0.26% | -0.18% | -0.32% | 106.5% | 104.7% |
| Class F | -0.13% | -0.13% | -0.09% | 106.1% | 103.5% |
| Class TGM | 0.00% | -0.06% | -0.08% | 104.3% | 100.8% |

## BDOF refine affine subblock MV:

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.19% | -0.04% | 103.2% | 102.4% |
| Class A2 | -0.30% | -0.23% | -0.20% | 103.7% | 106.6% |
| Class B | -0.10% | -0.08% | -0.03% | 102.9% | 104.5% |
| Class C | -0.18% | -0.22% | -0.05% | 102.5% | 104.2% |
| Class E |  |  |  |  |  |
| Overall | -0.14% | -0.17% | -0.07% | 103.0% | 104.4% |
| Class D | -0.18% | -0.35% | -0.22% | 102.4% | 105.3% |
| Class F | -0.07% | -0.11% | 0.01% | 102.6% | 103.1% |
| Class TGM | 0.02% | 0.00% | -0.04% | 101.7% | 100.6% |

# References

[1] ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).