| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0150-v3 |
| --- | --- |

| Title: | EE2-3.7 RPR with new filters scale factor 1.25x and 1.33x |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson-Allendes Sachin Deshpande | Tel: Email: | +46721882169 samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This input document reports the results of applying new RPR filters for scale factors 1.25x and 1.33x, which is tested in EE2-3.7a and EE2-3.7b. New filters are used for luma, chroma and affine. The same filters are used for both 1.25x and 1.33x. The reported average BD-Rate difference (Y/U/V) for EE2-3.7a (1.25x) is -0.77%/-0.64%-0.55% for PSNR1 and -0.76%/-0.40%-0.40% for PSNR2. The reported average BD-Rate difference (Y/U/V) for EE2-3.7b is -2.09%/-1.24%-1.21% for PSNR1 and -2.02%/-1.35%-1.27% for PSNR2. In the first version of this document the average numbers for EE2-3.7b (1.33x) did not include BasketballDrive and BQTerrace, as these results were still awaited. These average numbers for EE2-3.7b (1.33x) have been updated in the second version of this document to include all sequences. In the third version of this document, the final missing test case has been added (ArenaOfValor QP22 in Class F for 1.33x).

# Introduction

The introduction section of this document is replicated from the original proposal JVET-AD0169 [1].

Compared to previous video coding standards such as AVC and HEVC, the VVC standard introduces Reference Picture Resampling (RPR) as a tool for adaptively changing resolution within a Coded Layer Video Sequence (CLVS). Two additional sets of filters are included in the Motion Compensation (MC) in VVC; one that is optimized for 1.5x scaling and one that is optimized for 2x scaling. However, reduced resolution for all scale factors from 1x to 2x are supported by the VVC standard.

The scale factors 1.5x and 2x represent scaling between common resolutions, for example 1080p to 720p and 2160p to 1080p, respectively. These scale factors represent a sample count reduction of 56% and 75%, respectively.

In many practical cases, such as in Adaptive Bitrate Streaming (ABR) the resolution difference between different representations will be smaller than 1.5x. The HLS authoring specification from Apple [2], includes for example steps from 720p to 540p (1.33x), 1440p to 1080p (1.33x) and 540p to 432p (1.25x). The Netflix bitrate ladder presented in the “Per-Title Encode Optimization” blog post [3] includes steps from 384p to 288p (1.33x) and 480p to 384p (1.25x).

The scale factors 1.33x and 1.25x represent a sample count reduction of 44% and 36%, respectively.

In VVC, the MC filters for 1.5x will be used when the RPR scale factor is 1.33x, and the regular (1x) MC filters will be used when the RPR scale factor is 1.25x. This leads to suboptimal performance; 1.33x will use RPR filters that are too soft and 1.25x will use RPR filters that are too sharp. It is proposed to introduce one additional set of RPR filters that provide good performance for both these scale factors.

It can also be noted that a recent JVET contribution, JVET-AC0096 [4], proposes to test RPR with 1.25x scaling in addition to 1.5x and 2x scaling. Furthermore, the draft verification test plan for VVC multilayer coding, JVET-AC2021 [5], includes 1.33x scaling in addition to 1.5x and 2x scaling and in JVET-AC0145 [6] (experiments in preparation of dual-layer VVC visual tests), two out of four test cases for spatial scalability uses the scale factor 1.33x. However, for those multilayer coding use cases, RPR is only used for upsampling (and not for downsampling).

It is asserted that adding RPR filters for scale factors in the range 1.1x to 1.35x will improve the performance and help in investigating the efficiency of coding tools for a wider range of test conditions.

# Current RPR operation in ECM

Both VTM and ECM supports encoding a subset of the pictures of a video sequence in reduced resolution. This process involves three major steps, as illustrated in Figure 1:

- Non-normative downsampling

- ECM compression (including RPR when picture sizes are different)

- Non-normative upsampling.

This input document proposes changes for step 2 only (for scale factors in the range from 1.1x to 1.35x).

Figure 1. Overview of reduced resolution coding in ECM.

As can be seen in Figure 1, PSNR2 is calculated using the original picture and the picture that has been reconstructed and upsampled. PSNR1 is calculated at the coded resolution.

# Proposed changes to RPR

In order to improve the performance for scaling ratios 1.25x and 1.33x it is proposed to introduce an additional set of RPR filters consisting of three tables (respectively for luma, for chroma and for affine). It is proposed to use these RPR filters when the scaling ratio is between 1.1x and 1.35x (in alignment with the adopted non-normative downsampling filters from JVET-AD0169 [1]).

In alignment with the current RPR filters in ECM, it is proposed to use 8-bit precision for the filters with filter lengths 12, 6, 10, for luma, chroma, and affine, respectively. The proposed filters are constructed with a kaiser-windowed sinc function with kaiser beta equal to 7 and scaling ratio 1.35 (in alignment with the adopted non-normative downsampling filters from JVET-AD0169 [1]). The 10-tap affine filter is constructed from the 12-tap luma filter by moving (adding) the outermost coefficients one step closer to the filter center.

luma:

{  -1,   0,   7, -28,  55, 190,  55, -28,   7,   0,  -1,   0},

{  -1,   0,   8, -26,  43, 189,  66, -29,   6,   1,  -1,   0},

{   0,  -1,   8, -24,  32, 187,  77, -29,   5,   2,  -1,   0},

{   0,  -1,   8, -22,  22, 183,  90, -29,   3,   3,  -1,   0},

{   0,  -1,   8, -19,  13, 178, 101, -28,   2,   3,  -1,   0},

{   0,  -1,   8, -17,   5, 172, 113, -26,   0,   4,  -2,   0},

{   0,  -2,   8, -14,  -3, 165, 125, -23,  -3,   5,  -2,   0},

{   0,  -2,   7, -11,  -9, 156, 136, -20,  -5,   6,  -2,   0},

{   0,  -2,   7,  -8, -15, 146, 146, -15,  -8,   7,  -2,   0},

{   0,  -2,   6,  -5, -20, 136, 156,  -9, -11,   7,  -2,   0},

{   0,  -2,   5,  -3, -23, 125, 165,  -3, -14,   8,  -2,   0},

{   0,  -2,   4,   0, -26, 113, 172,   5, -17,   8,  -1,   0},

{   0,  -1,   3,   2, -28, 101, 178,  13, -19,   8,  -1,   0},

{   0,  -1,   3,   3, -29,  90, 183,  22, -22,   8,  -1,   0},

{   0,  -1,   2,   5, -29,  77, 187,  32, -24,   8,  -1,   0},

{   0,  -1,   1,   6, -29,  66, 189,  43, -26,   8,   0,  -1},

chroma:

{   -8,   41,  190,   41,   -8,    0},

{   -7,   36,  189,   46,   -8,    0},

{   -7,   31,  190,   51,   -9,    0},

{   -6,   27,  187,   57,   -9,    0},

{   -5,   22,  186,   63,  -10,    0},

{   -5,   18,  184,   69,  -10,    0},

{   -4,   15,  181,   74,  -10,    0},

{   -4,   11,  179,   81,  -11,    0},

{   -3,    8,  175,   87,  -11,    0},

{   -3,    5,  172,   93,  -11,    0},

{   -3,    3,  167,  100,  -11,    0},

{   -2,    0,  163,  106,  -11,    0},

{   -2,   -2,  158,  112,  -10,    0},

{   -2,   -4,  153,  119,  -10,    0},

{   -1,   -5,  147,  124,   -9,    0},

{   -1,   -7,  143,  131,   -9,   -1},

{   -1,   -8,  137,  137,   -8,   -1},

{   -1,   -9,  131,  143,   -7,   -1},

{    0,   -9,  124,  147,   -5,   -1},

{    0,  -10,  119,  153,   -4,   -2},

{    0,  -10,  112,  158,   -2,   -2},

{    0,  -11,  106,  163,    0,   -2},

{    0,  -11,  100,  167,    3,   -3},

{    0,  -11,   93,  172,    5,   -3},

{    0,  -11,   87,  175,    8,   -3},

{    0,  -11,   81,  179,   11,   -4},

{    0,  -10,   74,  181,   15,   -4},

{    0,  -10,   69,  184,   18,   -5},

{    0,  -10,   63,  186,   22,   -5},

{    0,   -9,   57,  187,   27,   -6},

{    0,   -9,   51,  190,   31,   -7},

{    0,   -8,   46,  189,   36,   -7},

affine:

{  -1,   7, -28,  55, 190,  55, -28,   7,   0,  -1},

{  -1,   8, -26,  43, 189,  66, -29,   6,   1,  -1},

{  -1,   8, -24,  32, 187,  77, -29,   5,   2,  -1},

{  -1,   8, -22,  22, 183,  90, -29,   3,   3,  -1},

{  -1,   8, -19,  13, 178, 101, -28,   2,   3,  -1},

{  -1,   8, -17,   5, 172, 113, -26,   0,   4,  -2},

{  -2,   8, -14,  -3, 165, 125, -23,  -3,   5,  -2},

{  -2,   7, -11,  -9, 156, 136, -20,  -5,   6,  -2},

{  -2,   7,  -8, -15, 146, 146, -15,  -8,   7,  -2},

{  -2,   6,  -5, -20, 136, 156,  -9, -11,   7,  -2},

{  -2,   5,  -3, -23, 125, 165,  -3, -14,   8,  -2},

{  -2,   4,   0, -26, 113, 172,   5, -17,   8,  -1},

{  -1,   3,   2, -28, 101, 178,  13, -19,   8,  -1},

{  -1,   3,   3, -29,  90, 183,  22, -22,   8,  -1},

{  -1,   2,   5, -29,  77, 187,  32, -24,   8,  -1},

{  -1,   1,   6, -29,  66, 189,  43, -26,   8,  -1},

# Results

The proposed filters and thresholds have been implemented in ECM. Due to bugs in the RPR code in ECM-9.0, the version from July 12 (commit id d45900ab3ad3590399658a936c0ac13ac046797d where the RPR bugs had been fixed) was used instead. The test configuration was in alignment with the configuration described in [7].

## PSNR1 Results EE2-3.7a (1.25x LDB)

|  |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.35% | -0.27% | -0.16% | 99.1% | 99.4% |
| Class C | -1.15% | -0.72% | -1.03% | 101.9% | 103.3% |
| Class E | -0.96% | -1.17% | -0.55% | 99.1% | 98.9% |
| Overall | -0.77% | -0.64% | -0.55% | 100.0% | 100.6% |
| Class D | -2.58% | -2.09% | -1.88% | 100.0% | 102.0% |
| Class F | -11.32% | -7.66% | -8.65% | 98.7% | 99.7% |

Table 1. EE2-3.7a (1.25x LDB) PSNR1

## PSNR2 Results EE2-3.7a (1.25x LDB)

|  |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.38% | -0.17% | -0.05% | 99.1% | 99.4% |
| Class C | -0.97% | -0.31% | -0.81% | 101.9% | 103.3% |
| Class E | -1.11% | -0.93% | -0.43% | 99.1% | 98.9% |
| Overall | -0.76% | -0.40% | -0.40% | 100.0% | 100.6% |
| Class D | -2.37% | -2.04% | -1.82% | 100.0% | 102.0% |
| Class F | -7.34% | -6.95% | -7.85% | 98.7% | 99.7% |

Table 2. EE2-3.7a (1.25x LDB) PSNR2

## PSNR1 Results EE2-3.7b (1.33x LDB)

|  |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.92% | -0.58% | -0.64% | 97.2% | 98.0% |
| Class C | -3.49% | -2.49% | -2.46% | 96.7% | 102.4% |
| Class E | -2.16% | -0.66% | -0.50% | 96.6% | 101.6% |
| Overall | -2.09% | -1.24% | -1.21% | 96.9% | 100.3% |
| Class D | -3.39% | -2.14% | -2.21% | 90.0% | 94.4% |
| Class F | -12.23% | -8.59% | -8.80% | 93.7% | 99.8% |

Table 3. EE2-3.7b (1.33x LDB) PSNR1

## PSNR2 Results EE2-3.7b (1.33x LDB)

|  |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.92% | -0.63% | -0.67% | 97.2% | 98.0% |
| Class C | -3.09% | -2.58% | -2.52% | 96.7% | 102.4% |
| Class E | -2.44% | -0.92% | -0.61% | 96.6% | 101.6% |
| Overall | -2.02% | -1.35% | -1.27% | 96.9% | 100.3% |
| Class D | -2.58% | -2.29% | -2.35% | 90.0% | 94.4% |
| Class F | -8.08% | -8.00% | -8.21% | 93.7% | 99.8% |

Table 4. EE2-3.7b (1.33x LDB) PSNR2

# References

- J. Samuelsson, S. Desphande, “AHG12: RPR filters for scale factors below 1.5x”, JVET-AD0169, April 2023.

- “HTTP Live Streaming (HLS) Authoring Specification for Apple Devices”, https://developer.apple.com/documentation/http_live_streaming/http_live_streaming_hls_authoring_specification_for_apple_devices

- “Per-Title Encode Optimization”, https://netflixtechblog.com/per-title-encode-optimization-7e99442b62a2

- R. Yu, K. Andersson, “AHG10/12: Suggestion for new CTC for RPR in VTM and ECM”, JVET-AC0096, January 2023.

- S. Iwamura, P. de Lagrange, M. Wien, “Draft verification test plan for VVC multilayer coding”, JVET-AC2021, January 2023.

- P. de Lagrange, “AHG4: experiments in preparation of dual-layer VVC visual tests”, JVET-AC0145, January 2023.

- J. Luo, V. Seregin, “JVET functionality confirmation test conditions for reference picture resampling”, JVET-Q2015, Jan. 2020.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Acknowledgement
These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.