| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, 11–19 July 2023 | Document: JVET-AE0159-v3 |
| --- | --- |

| Title: | EE2-2.5: Filtered Intra Block Copy (FIBC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | EE2 |  |  |
| Author(s) or Contact(s): | 1Hong-Jheng Jhu, Xiaoyu Xiu, Che-Wei Kuo, Wei Chen, Ning Yan, Changyue Ma, Xianglin Wang (Kwai) 2Bappaditya Ray, Muhammed Coban, Vadim Seregin, Marta Karczewicz (Qualcomm) | Email: | jhuhong-jheng@kwai.com bray@qti.qualcomm.com |
| Source: | Kwai, Qualcomm |  |  |

# Abstract

This contribution reports the results of EE2-2.5c method (a combination of filtered IBC test – EE2-2.5a and EE2-2.5b) in the context of Exploration Experiments 2 (JVET-AD2024).

The test results on top of ECM-9.0 are as follows:

AI(Y/U/V):

Natural content: 0.01*%/0.02%/0.03%, EncT 100%, DecT 99%

Class F: -0.46%/-0.29%/-0.44%, EncT 101%, DecT 100%

Class TGM: -0.22%/-0.25%/-0.18%, EncT 101%, DecT 100%

RA(Y/U/V):

Natural content: 0.00*%/-0.07%/0.08%, EncT 100%, DecT 100%

Class F: -0.32%/-0.31%/-0.42%, EncT 101%, DecT 99%

Class TGM: -0.08%/-0.10%/-0.08%, EncT 101%, DecT 99%

For test-2.5a, results are as follows:

AI(Y/U/V):

Natural content: 0.00*%/0.03%/0.05%, EncT 101%, DecT 99%

Class F: -0.38%/-0.52%/-0.41%, EncT 102%, DecT 101%

Class TGM: -0.09%/-0.11%/-0.06%, EncT 101%, DecT 100%

RA(Y/U/V):

Natural content: 0.00*%/-0.04%/-0.01%, EncT 100%, DecT 100%

Class F: -0.25%/0.02%/-0.16%, EncT 102%, DecT 100%

Class TGM: -0.04%/-0.03%/-0.10%, EncT 101%, DecT 100%

For test-2.5b, results are as follows:

AI(Y/U/V):

Natural content: -0.04*%/-0.03%/-0.01%, EncT 102%, DecT 99%

Class F: -0.27%/-0.31%/-0.27%, EncT 107%, DecT 99%

Class TGM: -0.18%/-0.20%/-0.14%, EncT 108%, DecT 102%

RA(Y/U/V):

Natural content: 0.00*%/-0.02%/0.08%, EncT 101%, DecT 100%

Class F: -0.15%/-0.05%/-0.47%, EncT 104%, DecT 100%

Class TGM: -0.04%/-0.05%/-0.11%, EncT 103%, DecT 100%

# Introduction

In 30th JVET meeting, the filtering on top of IBC was investigated in JVET-AC0217 [1] and JVET-AC0223 [2]. In these proposals filter is derived from minimizing MSE between current and reference template (template corresponding to reference block), and then this filter is applied on top of IBC predicted block. This contribution reports a combination of these two proposals.

# Proposed method:

Test-2.5a:

In Test 2.5a, the proposed 7-tap filter consists of a 5-tap plus sign shape spatial component, a bias term and a nonlinear term (same to CCCM model). The input to the spatial 5-tap component of the filter consists of a center (C) sample in the reference block which is at corresponding locations with the sample in the current block to be predicted and its above/north (N), below/south (S), left/west (W) and right/east (E) neighbors as illustrated below.

Figure 1: Spatial part of the filter

The nonlinear term P is represented as power of two of the center sample C and scaled to the sample value range of the content:

P = ( C*C + midVal ) >> bitDepth

The bias term B represents a scalar offset between the input and output and is set to middle luma value (512 for 10-bit content).

Output of the filter is calculated as follows:

predLumaVal = c0C + c1N + c2S + c3E + c4W + c5P + c6B

The filter coefficients ci are calculated by minimising the MSE between the reference template and current template, very similar to the CCCM process.

One extra indication flag is introduced for the FIBC which is signaled conditioned on the IBC-LIC flag. Specifically, when the IBC-LIC flag is true, the flag is signaled and used to indicate whether the FIBC is applied to the current block or not. For IBC merge modes, this filtering mode is inherited when merge mode list is constructed, so there is no extra signaling.

Test -2.5b:

Test – 2.5b is mostly similar to Test-2.5a, except for the two normative differences,

- In test-2.5b, there is no nonlinear term, i.e., only 5 tap spatial filtering and bias term.

- The indication flag is signaled before IBC-LIC flag.

Additionally, the encoder design for test-2.5b is different than test-2.5a.

Test -2.5c:

For test-2.5c, the filtering is the same as test-2.5a (7 tap filtering, including nonlinear term), and the signaling is the same as Test-2.5b. The different aspects of encoder design is chosen from both 2.5a and 2.5b, considering best tradeoff.

## Performance on top of ECM-9.0:

In this test, the anchor is ECM-9.0 and the proposed method, i.e., Test -2.5c is tested on top of this anchor. The results are as follows. For ClassA2 of AI configuration and For ClassA1 and ClassA2 of RA configuration, some rate points have not finished, those are copied from the anchor, and marked in italics.

Test -2.5c:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM9 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.10% | -0.07% | 100% | 99% |
| Class A2 | 0.01% | -0.03% | 0.05% | 100% | 98% |
| Class B | 0.01% | 0.00% | 0.07% | 101% | 100% |
| Class C | -0.01% | 0.00% | -0.01% | 101% | 100% |
| Class E | 0.03% | 0.03% | 0.10% | 100% | 99% |
| Overall | 0.01% | 0.02% | 0.03% | 100% | 99% |
| Class D | 0.04% | -0.03% | -0.17% | 100% | 97% |
| Class F | -0.46% | -0.29% | -0.44% | 101% | 100% |
| Class TGM | -0.22% | -0.25% | -0.18% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM9 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.20% | 0.00% | 100% | 100% |
| Class A2 | -0.02% | -0.09% | 0.00% | 100% | 100% |
| Class B | 0.01% | -0.02% | 0.13% | 99% | 100% |
| Class C | 0.00% | -0.04% | 0.12% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.07% | 0.08% | 100% | 100% |
| Class D | -0.03% | -0.24% | 0.16% | 99% | 100% |
| Class F | -0.32% | -0.31% | -0.42% | 101% | 99% |
| Class TGM | -0.08% | -0.10% | -0.08% | 101% | 99% |

Additionally, the individual test results of Test-2.5a and Test-2.5b are provided below.

Test -2.5a:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM9 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.08% | -0.01% | 100.1% | 100.1% |
| Class A2 | 0.01% | -0.04% | 0.07% | 100.8% | 99.3% |
| Class B | 0.00% | 0.05% | 0.08% | 100.9% | 99.7% |
| Class C | 0.01% | 0.01% | 0.01% | 100.5% | 100.6% |
| Class E | 0.00% | 0.04% | 0.12% | 100.2% | 99.7% |
| Overall | 0.00% | 0.03% | 0.05% | 100.6% | 99.9% |
| Class D | 0.02% | -0.02% | 0.11% | 100.4% | 99.5% |
| Class F | -0.38% | -0.52% | -0.41% | 102.3% | 100.6% |
| Class TGM | -0.09% | -0.11% | -0.06% | 101.4% | 99.7% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM9 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.15% | 0.01% | 100.2% | 100.1% |
| Class A2 | -0.03% | -0.07% | -0.07% | 100.2% | 100.0% |
| Class B | 0.00% | 0.06% | 0.03% | 100.2% | 99.3% |
| Class C | 0.03% | -0.28% | -0.02% | 100.2% | 99.2% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.04% | -0.01% | 100.2% | 99.6% |
| Class D | 0.02% | -0.17% | 0.17% | 100.4% | 99.3% |
| Class F | -0.25% | 0.02% | -0.16% | 101.8% | 99.6% |
| Class TGM | -0.04% | -0.03% | -0.10% | 100.7% | 100.2% |

Test-2.5b:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM9 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | -0.01% | 102.0% | 100.2% |
| Class A2 | -0.03% | -0.01% | 0.00% | 101.5% | 99.4% |
| Class B | -0.07% | 0.00% | -0.06% | 102.4% | 98.4% |
| Class C | -0.05% | -0.08% | -0.09% | 103.2% | 99.9% |
| Class E | -0.04% | -0.13% | 0.15% | 101.7% | 99.7% |
| Overall | -0.04% | -0.03% | -0.01% | 102.2% | 99.4% |
| Class D | 0.02% | -0.08% | -0.03% | 100.2% | 99.6% |
| Class F | -0.27% | -0.31% | -0.27% | 107.3% | 99.1% |
| Class TGM | -0.18% | -0.20% | -0.14% | 107.6% | 101.5% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM9 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.12% | 0.06% | 101.3% | 100.2% |
| Class A2 | -0.03% | 0.02% | -0.01% | 100.4% | 99.0% |
| Class B | -0.02% | 0.06% | 0.00% | 100.6% | 99.7% |
| Class C | 0.02% | -0.09% | 0.15% | 100.2% | 99.6% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | 0.05% | 100.6% | 99.6% |
| Class D | 0.02% | -0.06% | -0.04% | 100.0% | 100.2% |
| Class F | -0.15% | -0.05% | -0.47% | 103.8% | 99.9% |
| Class TGM | -0.04% | -0.05% | -0.11% | 103.4% | 100.2% |

# References

- H.-J. Jhu, et al., “Non-EE2: Filtered Intra Block Copy (FIBC)”, JVET-AD0217, Apr 2023.

- B. Ray et al. “Non-EE2: Filtering for IBC predicted block”, JVET-AD0223, Apr 2023.

- ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

# Patent rights declaration(s)

Kwai may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).