| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting: Geneva, CH, 11–19 July 2023 | Document: JVET-AE0161-v3 |
| --- | --- |

| Title: | AHG11: Input and output rotation of model for NNVC in-loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Renjie Chang Liqiang Wang Xiaozhong Xu Shan Liu | Email: | renjiechang@tencent.com liqiangwang@tencent.com xiaozhongxu@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this contribution, the rotation operations are applied to the inputs and outputs of NNVC HOP network to further improve the coding performance. In the rotation process, all the input samples including image information and side information are rotated and then fed into network to perform the inference. After that, the output of network is rotated back to restore the original order of filtered samples. Based on NNVC-5.1 the test results are shown in order of RA and LDB configurations as follows.

Compared with NNVC-5.0 (only HOP):

RA: -0.14%	-0.55%	  		-0.65%			EncT: 140%	DecT: 122%

Introduction

In NNVC-5.0, neural-network based HOP filter [1] is developed as an additional in-loop filter to improve the coding performance. The network architecture of HOP filter is shown in Fig. 1. The input of HOP network contains reconstruction, prediction, boundary strength, sliceQP, baseQP and IPB. The output of HOP network contains luma and chroma reconstruction which are denoted by Rec Y and Rec UV respectively.

Fig. 1: Network architecture of HOP filter

Proposed method

Rotation operation

In the proposed method, all the input samples may be rotated 90° to the left or right and then fed into the network. If rotation is performed on the inputs, the output of network would be rotated back to restore the original order of the filtered samples. The operations of rotating left and right are shown as follows.

Fig. 2: Rotation to the left

Fig. 3: Rotation to the right

Specifically, a slice-level flag is signaled to indicate the rotation type including no rotation, rotation to the left and rotation to the right. All the CTUs in a slice are applied the same rotation operation before and after network inference.

Besides, to reduce the encoding time, only partial CTUs are used in the RDO process of selecting the best rotation type.

Inference stage

The inference information is identical to NNVC-5.0 HOP filter.

| - Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
| Mandatory | HW environment: |  |  |
|  | GPU Type | CPU only |  |
|  | Framework: | SADL |  |
|  | Number of GPUs per Task | 0 |  |
|  | Number of Parameters (Each Model) | 1.45 M/model |  |
|  | Total Number of Parameters (All Models) | 1.45 M/model |  |
|  | Parameter Precision (Bits) | 16 |  |
|  | Memory Parameter (MB) | 2.8 MB/model |  |
|  | Multiply Accumulate (kMAC/pixel) | 477 |  |
|  | Calculation Method | block basis |  |
| Optional |  |  |  |
|  | Total Conv. Layers |  |  |
|  | Total FC Layers | 0 |  |
|  | Total Memory (MB) |  |  |
|  | Batch size: | 1 |  |
|  | Patch size | 144x144 |  |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage (Total) |  |  |
|  | Peak Memory Usage (per Model) |  |  |
|  | Border handling |  |  |
|  | Other information: |  |  |

Results

All the experiments are performed based on the common test conditions in [2]. The proposed method is implemented on top of NNVC-5.0 HOP filter. The results are shown as follows.

Compared with NNVC-5.0 (NNIntra + HOP)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0_NNIntra+HOP |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class C | -0.13% | -0.78% | -0.95% |  |  |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! |  |  |
| Class D | -0.17% | -1.37% | -1.33% |  |  |
| Class F | #VALUE! | #VALUE! | #VALUE! |  |  |

Compared with NNVC-5.0 (only HOP)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-5.0 (only HOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.18% | -0.27% | -0.74% | 144% | 123% |
| Class A2 | -0.18% | -0.77% | -0.57% | 145% | 122% |
| Class B | -0.08% | -0.37% | -0.55% | 141% | 122% |
| Class C | -0.14% | -0.82% | -0.74% | 134% | 121% |
| Class E |  |  |  |  |  |
| Overall | -0.14% | -0.55% | -0.65% | 140% | 122% |
| Class D | -0.21% | -1.10% | -1.42% | 133% | 115% |
| Class F | -0.09% | -0.58% | -0.49% | 156% | 125% |

Conclusions

In this contribution, the rotation operations are proposed to be applied to both input and output of HOP network to further improve the coding performance. Experimental results demonstrate the effectiveness of the proposed method.

References

- E. Alshina, F. Galpin. BoG report on NN-filter design unification. JVET-AD0380, 30th Meeting, Antalya, TR, 21–28 April 2023.

- E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AD2016, 30th Meeting, Antalya, TR, 21–28 April 2023.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).