| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0166-v2 |
| --- | --- |

| Title: | AHG12: IntraTMP and IBC search area harmonization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | D. Ruiz Coll, B. Chen, P. Andrivon. M. Blestel | Tel: Email: | duizcoll@ofinno.com, bchen@ofinno.com, pandrivon@ofinno.com mblestel@ofinno.com |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This contribution explores the search area harmonization of the IBC and IntraTMP tools. IBC and IntraTMP use the reconstructed samples of the current picture to select the prediction block for the current block. The IBC search area is fixed to a specific number of CTUs according to the current block locations, while the IntraTMP search area is based on the block size, which exceeds the IBC search region for some block sizes. In order to harmonize the decoded buffer for both tools, this contribution investigates two aspects, the constraint of the current IntraTMP search regions and the extent of such regions to the IBC search region.

On top of ECM-9.0, simulation results are reported as follows:

Test-1: IntraTMP search sub-regions constraint to IBC search area.

AI {-0.00% Y, -0.00% U, -0.00% V, 99.3% EncT, 99.1% DecT};

Test-2: Adjustment of the IntraTMP sub-regions to the IBC search area.

AI: Class F : {-0.44%, -0.57%, -0.38%, 102.4%, 109.5%},

AI: Class TGM : {-0.68%, -0.68 %, -0.69%, 102.5%, 114.5%};

# Introduction

Regardless of whether the CTU size is 128 or 256, the IBC reference region was extended to 256 samples above the current CTU, including all the left neighbor CTUs of the same row of the current CTU and two CTU rows for CTU size of 128 or one CTUs row for CTU size of 256. In both cases, the top row of CTUs is constrained to one or two CTUs to the left of the current CTU, and the top left corner of the BC reference region is located at 256 samples to the left of the current CTU and 256 samples above the current CTU, as depicted in FIG 1.

In JVET-V0130 [1], the IntraTMP search area was set proportional to the block dimensions (CbWidth, CbHeight) using a factor of 5 (SearchRangew= 5 * CbWidth,  SearchRange_h = 5 * CbHeight), using different sub-regions (Ri). As can be observed in FIG. 2, for the block size 64x64, the top sub-region (R1), exceeds the left and top boundaries of the IBC search area, demanding additional memory compared to the IBC reconstructed buffer.

Figure 2. IntraTMP search area for a block size of 64x64 (CTU size 128)

# Proposal

This contribution explores two aspects to harmonize the IntraTMP and IBC search areas, allowing the same memory size for the reconstructed samples to be used for both tools. This feature is particularly desirable from the hardware point of view, considering that the reconstructed samples use high-cost on-chip memory. The first test constrains the size of the IntraTMP subregions to ensure they are enclosed in the IBC boundaries of the search region. Consequently, for a block size of 64x64, the top and left boundaries of the subregion R1 are constrained to the IBC search region boundaries.

The first aspect also proposed a fix for the boundaries between the R1 and R3, which are overlapped, increasing the number of template-matching computations for all block sizes. The fix increases in one sample of the top boundary of R3 (iVerMin).

if( regionId == 0 ) //above outside LCU { iVerMax = (iCurrY - iBlkHeight - offsetLCUY) << iMvShift; } else if( regionId == 1 ) //left outside LCU { iVerMin = std::max( (tempType != LEFT_TEMPLATE) ? iTemplateSize << iMvShift : 0, (iCurrY - iBlkHeight - offsetLCUY) << iMvShift ); }

The second aspect explores the adaptation of the current IntraTMP sub-regions to the particular shape of the IBC search region, extending the searching area beyond the current IntraTMP. FIG. 3 shows the new Intra TMP subregions arrangement.

For screen content coding, it is proposed to use the sampling rate of 3 (SR1) for the predefined TMP search region in ECM-9.0 with the maximum limits of 5 times the current block dimensions and a second adaptive sampling rate which is a power of 8 (SR2).

Figure 3. TMP and IBC search region harmonization using two different sampling rates.

# Experimental Results

The proposed method is tested on top of ECM-9.0 software [2] under JVET ECM common test conditions [3] in AI and RA configurations. The following results were obtained:

Test-1: (IntraTMP search sub-regions constraint to IBC search area)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | -0.05% | 99.5% | 99.1% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | 0.09% | 0.02% | 99.6% | 99.2% |
| Class C | 0.00% | -0.04% | -0.07% | 99.6% | 98.7% |
| Class E | -0.01% | 0.04% | 0.08% | 99.7% | 99.4% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | -0.01% | 0.03% | 99.2% | 98.8% |
| Class F | -0.01% | 0.00% | 0.04% | 98.9% | 98.4% |

Test-2: (Adjustment of the IntraTMP sub-regions to the IBC search area)

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.44% | -0.57% | -0.38% | 102.4% | 109.5% |
| Class TGM | -0.68% | -0.68% | -0.69% | 102.5% | 114.5% |

- References

- K. Naser, T. Poirier, F. Le Léannec, G. Martin-Cocher, “EE2: Intra Template Matching”, JVET-V0130, April 2021.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-9.0

- M. Karczewicz and Y. Ye, "Common Test Conditions and evaluation procedures for enhanced compression tool testing," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-Y2017, 25th meeting, by teleconference, Jan. 2022.

# Patent rights declaration(s)

Ofinno, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).