| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0167-v2 |
| --- | --- |

| Title: | AHG12: DBV improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luhang Xu, Yue Yu, Haoping Yu, Dong Wang (OPPO) | Tel: Email: | xuluhang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

When dual-tree block partition is enabled for an intra slice in ECM9, DBV is one of the chroma coding modes if one of the five corresponding luma positions is coded with either intraTMP or IBC. If the current luma block is coded with IBC or IntraTMP and DBV is not used, this contribution proposes to use one of the 4 prediction modes, i.e., DC, Planar, Horizontal, and Vertical, without the DM flag to code the corresponding chroma block. On top of ECM-9.0, the simulation results are reported as follows:

AI: 		-0.02% (Y), -0.09% (U), -0.08% (V), xx% (EncT), xx% (DecT)

RA: 		-0.01% (Y), -0.13% (U), -0.01% (V), xx% (EncT), xx% (DecT)

Introduction

When dual-tree block partition is enabled for an intra slice in ECM9, DBV is one of the chroma coding modes if one of the five corresponding luma positions is coded with either intraTMP or IBC. If the current chroma block is coded with DBV, the BV of the corresponding luma block is used to directly form a chroma prediction from the reconstructed chroma area. If the current chroma block is not coded with DBV and the corresponding luma block is coded with IBC or IntraTMP, a DM flag will be coded. If the DM flag is equal to 1 and the corresponding luma block is coded with IBC or IntraTMP, the DC mode or the Planar mode will be used to generate a prediction for the current chroma block, respectively. If the DM flag is equal to 0, the current chroma block is coded with other chroma mode.

Proposed method

This contribution proposes to use only one of the 5 prediction modes (DIMD, DC, Planar, Horizontal, and Vertical) to generate a prediction, without the DM flag, for the chroma block when DBV is not used and the corresponding luma block is coded with IBC or IntraTMP.

Simulation results

The proposed method is implemented on top of the ECM-9.0 software [1], and tested based on the common test condition [2]. The test results are summarized in Table 1 and Table 2.

Table 1. The proposed method over ECM-9.0 (AI)

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.20% | -0.30% |  |  |
| Class A2 | -0.04% | -0.12% | -0.10% |  |  |
| Class B | -0.02% | -0.11% | -0.14% |  |  |
| Class C | -0.02% | -0.04% | 0.11% |  |  |
| Class E | -0.01% | 0.00% | 0.00% |  |  |
| Overall | -0.02% | -0.09% | -0.08% |  |  |
| Class D | 0.02% | 0.04% | -0.03% |  |  |
| Class F | -0.03% | -0.14% | -0.11% |  |  |
| Class TGM | -0.04% | -0.04% | 0.01% |  |  |

Table 2. The proposed method over ECM-9.0 (RA)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.28% | -0.05% |  |  |
| Class A2 | -0.02% | -0.09% | -0.08% |  |  |
| Class B | -0.01% | -0.07% | 0.01% |  |  |
| Class C | -0.02% | -0.13% | 0.03% |  |  |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.13% | -0.01% |  |  |
| Class D | 0.05% | -0.21% | 0.08% |  |  |
| Class F | -0.02% | -0.17% | -0.17% |  |  |
| Class TGM | -0.01% | -0.04% | -0.05% |  |  |

Conclusions

This contribution proposes to use only DIMD, DC, or Planar, or Horizonal, or Vertical mode without the DM flag to code the chroma block when the corresponding luma block is IBC or IntraTMP and DBV is not used. The proposed method brings 0.02%/0.09%/%0.08 coding gain for AI without any complexity increase over ECM-9.0. It is suggested to study the proposed method in EE.

References

- ECM-9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-9.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AD2017, April, 2023.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).