| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0175 |
| --- | --- |

| Title: | [AHG9] Editorial improvements of nnpfc_mode_idc |  |  |
| --- | --- | --- | --- |
| Status: | Input contribution to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Youngkwon Lim 665 Clyde Avenue Mountain View, CA 94043 USA | Tel: Email: | young.L@samsung.com |
| Source: | Samsung Research America |  |  |

_____________________________

# Abstract

Current design of NNPFC SEI message in Draft v4 (JVET-AD2006, April 2023) includes the flag, nnpfc_mode_idc to indicate whether the message includes base NNPF or updates to the existing NNPF. However, its definition is confusing as its definition not only talk about the modes of updates but also the format of data. This contribution proposes updates of the definition of nnpfc_mode_idc to avoid any confusion.

# Inconsistency in definition of nnpfc_mode_idc and nnpfc_tag_uri

Current design of NNPFC SEI message in Draft v4 (JVET-AD2006, April 2023) includes the flag, nnpfc_mode_idc to indicate whether the message includes base NNPF or updates to the existing NNPF. However, its definition is confusing as its definition not only talk about the modes of updates but also the format of data which supposed to be signaled by another flag, nnpfc_tag_uri.

The definition of nnpfc_mode_idc says that if its value equals to 0 then the SEI message contains an ISO/IEC 15938-17 bitstream but it is silent about the format of data when its value equals to 1. It is not quite clear that the update cannot be in ISO/IEC 15938-17 bitstream format or not with the current definition. In addition, location of data in the SEI message is split apart in the beginning of the messages when the value of mode equals to 0 and at the end of the message when the value of mode equals to 1.

# Proposed updates of nnpfc_mode_idc

- Option 1
To clarify that the base NNPF can only be in the ISO/IEC 15938-17 bitstream format and the update can be any formats.

- Option 2
To clarify the nnpfc_mode_idc only indicates whether it is the base NNPF or updates relative to the base NNPF and the format is exclusively indicate the format of the data regardless of the value of nnpfc_mode_idc.

# Proposed specification text update

- Option 1

nnpfc_mode_idc equal to 0 indicates that this SEI message contains a base NNPF (when nnpfc_base_flag is equal to 1) or is an update relative to the base NNPF with the same nnpfc_id value (when nnpfc_base_flag is equal to 0).

When nnpfc_mode_idc is equal to 0, the data is in the format of ISO/IEC 15938-17 bitstream format. When nnpfc_mode_idc is equal to 1, the format of data is indicated through nnpfc_tag_uri.

- Option 2

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| nnpfc_base_flag | u(1) |  |
| nnpfc_mode_idc | ue(v) |
| nnpfc_reserved_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| if(nnpfc_tag_uri != "tag:iso.org,2023:15938-17") |  |
| nnpfc_uri | st(v) |
| else |  |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| nnpfc_property_present_flag | u(1) |
| **** no updates **** |  |
| } |  |

nnpfc_mode_idc equal to 0 indicates that this SEI message contains a base NNPF (when nnpfc_base_flag is equal to 1) or is an update relative to the base NNPF with the same nnpfc_id value (when nnpfc_base_flag is equal to 0). The format of data is indicated through nnpfc_tag_uri regardless of the value of this flag.

# Patent rights declaration(s)

Samsung Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).