| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0185-v2 |
| --- | --- |

| Title: | AHG12: IntraTMP with Neighboring Candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser Fabrice Le Léannec Antoine Robert Franck Galpin | Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In ECM-9.0, IntraTMP search is performed within predefined search area to obtain the best candidate block vectors. This contribution proposes to augment the TM search with the block vectors of neighboring PU’s that are coded with IBC or IntraTMP. The purpose is to improve the IntraTMP by further including probable candidates during the TM search. On top of ECM-9.0, the following results are obtained:

AI -0.09%	0.18%	0.17%	100%	100%

# Introduction

In IntraTMP mode, a TM search process within 4 pre-defined regions is performed to obtain up to 19 candidate block vectors. Compared to IBC, IntraTMP TM search does not take into consideration the neighboring PU’s block vector information. Specifically, in IBC merge mode, a merge index is coded to indicate which neighboring PU block vector is selected. Additionally, the IntraTMP block vectors are used as IBC merge candidate, but not the other way around. This is a limitation of IntraTMP that this contribution is trying to tackle.

# Proposal

This contribution proposes to make use of the neighboring PU’s information while performing the TM search for IntraTMP. Specifically, while performing IntraTMP search, the block vectors of the neighboring PU’s, which are coded with IBC or IntraTMP, are added to the search process. The same TM cost to rank the new candidates among the already computed one is used.

# Experimental results

The proposal is implemented on ECM-9.0. The following results are obtained in using the ECM common testing conditions:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.23% | 0.39% | 100% | 99% |
| Class A2 | -0.08% | -0.18% | 0.02% | 100% | 100% |
| Class B | -0.11% | 0.59% | 0.25% | 99% | 100% |
| Class C | -0.01% | 0.02% | 0.32% | 100% | 100% |
| Class E | -0.20% | 0.01% | -0.23% | 101% | 100% |
| Overall | -0.09% | 0.18% | 0.17% | 100% | 100% |
| Class D | 0.12% | -0.71% | 0.38% | 99% | 99% |
| Class F | -0.12% | -0.92% | 0.78% | 99% | 99% |
| Class TGM | -0.21% | -0.30% | -0.12% | 100% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.08% | 0.08% | -0.01% | 97% | 96% |
| Class C | 0.04% | -0.14% | 0.07% | 95% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | -0.33% | 0.07% | 95% | 103% |
| Class F | -0.13% | -0.07% | -0.02% | 99% | 104% |
| Class TGM | -0.19% | -0.30% | -0.34% | 96% | 101% |

During the review of this document, the reason for the loss in chroma was question. After reviewing the implementation, it is noted that there is some non-useful interaction with DBV mode. Fixing this, the following results are obtained (some points from anchor are copied for class A1 and A2):

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.12% | -0.04% | #NUM! | #NUM! |
| Class A2 | -0.09% | -0.11% | -0.13% | #NUM! | #NUM! |
| Class B | -0.16% | -0.15% | -0.16% | #NUM! | #NUM! |
| Class C | -0.03% | -0.06% | -0.05% | #NUM! | #NUM! |
| Class E | -0.09% | -0.18% | 0.06% | #NUM! | #NUM! |
| Overall | -0.08% | -0.12% | -0.08% | #NUM! | #NUM! |
| Class D | 0.01% | -0.09% | 0.03% | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).