| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0214-v2 |
| --- | --- |

| Title: | AHG12: on Intra CIIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser Philippe Bordes Franck Galpin Kevin Reuze Antoine Robert | Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

IntraCIIP is a method proposed in JVET-AC0201 and studied in last EE under the test of EE2-1.18 (JVET-AD0177). This tool was not tested on top of the adoption of several IntraTMP improvement of EE2-1.20j (JVET-AD0086). It is reported that IntraCIIP provides further coding gain compared to last EE as it benefits from the overall improvement of IntraTMP. The following result are obtained on top of ECM-9.0

AI: -0.08%	-0.02%	0.07%	100%	99%

# Introduction

IntraCIIP is a coding mode that combines the prediction of IntraTMP and regular intra prediction. The two predictions are blended using ECM’s CIIP blending method. The intra prediction part is generated via TIMD mode.

# Proposal

It is proposed to add IntraCIIP as a sub-mode of IntraTMP. That is, a flag is signaled after IntraTMP flag to indicate the usage of IntraTMP.

# Experimental results

The proposal is implemented on ECM-9.0. The following results are obtained in using the ECM common testing conditions:

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  |  | DecT |
| Class A1 | -0.12% | -0.09% |  | 0.18% |  | 100% |  |  | 99% |
| Class A2 | -0.04% | 0.15% |  | -0.01% |  | 101% |  |  | 100% |
| Class B | -0.10% | 0.00% |  | -0.24% |  | 99% |  |  | 99% |
| Class C | -0.03% | -0.38% |  | -0.18% |  | 100% |  |  | 100% |
| Class E | -0.13% | 0.32% |  | 0.86% |  | 100% |  |  | 100% |
| Overall | -0.08% | -0.02% |  | 0.07% |  | 100% |  |  | 99% |
| Class D | 0.10% | -1.39% |  | 1.45% |  | 99% |  |  | 100% |
| Class F | -0.13% | -0.94% |  | -0.07% |  | 99% |  |  | 99% |
| Class TGM | -0.17% | -0.44% |  | -0.58% |  | 100% |  |  | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
|  | Over ECM-9.0 |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT | DecT |  |
| Class A1 | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! | #NUM! |  |
| Class A2 | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! | #NUM! |  |
| Class B | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! | #NUM! |  |
| Class C | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! | #NUM! |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! | #NUM! |  |
| Class D | 0.03% |  | -0.20% |  | 0.13% |  | 99.7% | 103.6% |  |
| Class F | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! | #NUM! |  |
| Class TGM | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! | #NUM! |  |

It should be noted that in the previous EE, the performance on top of ECM-8 was:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.05% | 98% | 101% |
| Class A2 | -0.05% | -0.04% | -0.02% | 104% | 106% |
| Class B | -0.06% | -0.07% | 0.01% | 99% | 100% |
| Class C | -0.05% | -0.06% | -0.12% | 102% | 102% |
| Class E | -0.10% | -0.08% | -0.02% | 101% | 100% |
| Overall | -0.05% | -0.05% | -0.02% | 101% | 102% |
| Class D | -0.01% | -0.21% | 0.01% | 98% | 97% |
| Class F | 0.02% | -0.05% | -0.03% | 98% | 98% |
| Class TGM | -0.01% | 0.03% | 0.03% | 100% | 99% |

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).