| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0227-v1 |
| --- | --- |

| Title: | AHG4: experiments in preparation of scalable quality ladder visual testing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | P. de Lagrange F. Urban | Email: | philippe.delagrange@interdigital.com |  |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This document reports about some experiments in preparation of VVC multilayer verification tests planned in JVET-AD2021, especially the 2nd test category involving a quality ladder, and more specifically the definition of the anchor. Encodings at candidate resolutions and overlapping bit rates were performed, PSNR and VMAF metrics of upscaled decoded pictures are reported, along with visual checks to estimate expected scores and to compare resolutions at comparable bitrate. The intent is to provide data to help define rate points and resolutions of the above-mentioned anchor.

# Introduction

Verification tests of VVC multilayer coding are planned, as described in JVET-AD2021. The first two test categories involve visual assessment of dual-layer coding performance compared to single-layer coding. More specifically, the second category tests the potential benefits of dual-layer coding in terms of edge storage savings for streaming applications.

In JVET-AC0145, experiments were performed using a single test sequence (Procession), testing a candidate quality ladder derived from existing HEVC recommendations extrapolated for VVC, and testing some dual-layer combinations to help determine if the test category was relevant in the first place.

The experiments reported here use all 7 candidate test sequences listed in JVET-AC2021 and are intended to help refine the bitrate and resolution of the anchor.

# Experiments

The test sequences were encoded using VTM-17.0 (same as in JVET-AC0145) with default random-access configuration, at the following resolutions and QP ranges:

- 3840x2160: QP 26 to 48

- 2560x1440: QP 26 to 45

- 1920x1080: QP 26 to 42

- 1280x720: QP 23 to 39

- 960x540: QP 20 to 36

- 640x360: QP 17 to 33

Results, including objective metrics, rate-distortion plots, and visual checks, are reported in a spreadsheet and a presentation attached to this contribution.

In a nutshell, 640x360 resolution does not appear to be of interest because objective metrics are always inferior and visual checks indicate excessive loss of detail. 960x540 might be of interest in some corner cases. For higher resolutions, visual check sometimes gives an advantage to lower resolution when serious blocking or bad motion artefacts are attenuated, but the loss of detail is quickly a problem (in the case the target device is capable of decoding and displaying the higher resolution, of course).

# Conclusion

The results of this contribution may hopefully help define rate points and resolutions of the anchor for the quality ladder category of the VVC multilayer verification tests planned in JVET-AD2021, and may also more generally help recommend a quality ladder for VVC.