| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0272-v2 |
| --- | --- |

| Title: | BoG report on SEI messages for VSEI v3 and VVC v3 |  |  |
| --- | --- | --- | --- |
| Status: | BoG report to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan Dolby Laboratories, USA | Email: | gary.sullivan@dolby.com |
| Source: | BoG coordinator |  |  |

_____________________________

# Abstract

This document is a break-out-group report on SEI messages for VSEI v3 and VVC v3. The BoG met from 15:15 to 18:30 on Tuesday 11 July 2023 and from 08:30 to 12:30 and 14:30 to 18:30 on Wednesday 12 July 2023 in Salle 3 of the Geneva CICG facility. About 15-20 people participated in person and another 10-15 participated remotely.

The BoG reviewed the relevant proposals for NNPF SEI messages based primarily on the summary of NNPF-related SEI messages provided in JVET-AE0187-v3, which formed the original basis of the structure of this report.

No specific plans were recommended for topics that were agreed not to be within the scope of VSEI v3 and VVC v3 (except to encourage progress toward future support for VSEI v3 SEI messages for NNPF and filtering phase indication in HEVC and AVC).

Actions recommended by the BoG are highlighted in this report. Five items were marked to revisit for further discussion; these five were suggested to be relatively minor and may not require a further BoG session (if the BoG scope is not expanded to consider additional topics).

Notes of the JVET plenary review of the first version of the BoG report were included in the -v2 version of the report (with revision marking). In the -v2 version, one item remained marked to revisit for text review.

# Discussion topics categorization

The proposals on NNPF-related SEI messages were classified by JVET-AE0187-v3 into the following 14 categories based on contributions in section 6 of the draft JVET meeting notes:

1)	On missing value ranges for ue(v)-coded syntax elements

2)	On order, value inference, conditional presence, etc., of syntax elements and semantics

3)	On the persistence scope of NNPF SEI messages

4)	On presence of multiple NNPFC and NNPFA SEI messages and on NNPF activation

5)	On NNPF input pictures and repeated NNPF inference

6)	On NNPF generated pictures and NNPF output pictures

7)	On NNPF purpose(s) determined by the application

8)	On extensibility

9)	On base NNPF and NNPF updates

10)	Editorial changes

11)	On new NNPF purposes/functionalities

12)	On NNPF cascades, alternatives, etc.

13)	On region-based NNPF operations

14)	On NNPF SEI messages and phase indication SEI messages for AVC and HEVC

The following two additional contributions on other SEI topics potentially relevant to VSEI v3 and VVC v3 were also discussed in the BoG:

15)	JVET-AE0054 AHG2/AHG9: Editorial improvements of annotated regions SEI message [T. Chujoh, T. Aono, T. Ikai (Sharp)]

16)	JVET-AE0155 Editor commentary on the post-filter hint SEI message semantics [G. J. Sullivan, Y.-K. Wang (Editors)]

# Topic summaries and BoG discussions

Below is the list of design questions / proposals and their discussion and BoG-recommended dispositions.

- On missing value ranges for ue(v)-coded syntax elements

This item was discussed in the BoG at the beginning of its discussions on 11 July

  - Add missing value ranges for resampling syntax elements nnpfc_pic_width_num_minus1, nnpfc_pic_width_denom_minus1, nnpfc_pic_height_num_minus1, and nnpfc_pic_height_denom_minus1 to be in the range of 0 to 65535, inclusive. (JVET-AE0048 item 1)

BoG recommendation (correction to define missing value range): Decoders shall allow nnpfc_pic_width_num_minus1, nnpfc_pic_width_denom_minus1, nnpfc_pic_height_num_minus1, and nnpfc_pic_height_denom_minus1 to be in the range of 0 to 65535, inclusive.

  - Add missing value ranges for nnpfc_inp_format_idc and nnpfc_out_format_idc. (JVET-AE0048 item 2, JVET-AE0106)

Note: This is only for what values decoders would be required to allow; encoders only have a couple of values currently defined.

BoG recommendation (correction to define missing value range): 0 to 255, inclusive.

    - To be in the range of 0 to 255, inclusive. (JVET-AE0048 item 2)

    - To be in the range of 0 to 7, inclusive. (JVET-AE0106)

- On order, value inference, conditional presence, etc., of syntax elements and semantics

  - Arrange the definitions of DeriveInputTensors(), the syntax elements related to overlap and padding type, and the definition of the InpSampleVal function so that forward references are avoided by either of the following (JVET-AE0048 item 3):

BoG recommendation (Ed. cleanup): Proposed approach ii, since that approach is only editorial.

    - Move all the overlap and padding type syntax elements earlier in the syntax, between the syntax elements defining the input format and the syntax elements defining the output format, i.e., just before nnpfc_out_format_idc. Similarly, move the related semantics and function derivations respectively.

    - Move the derivations of strengthControlScaledVal (Formula 87), DeriveInputTensor (Formula 88), and StoreOutputTensor (91) to a later position that immediately follows the derivation of the InpSampleVal function.

  - Clarify that indicated or inferred values of vui_colour_primaries, vui_transfer_characteristics, vui_matrix_coeffs, and vui_full_range_flag apply when nnpfc_separate_colour_description_present_flag is equal to 0. Here, the possibility of inferring these syntax elements is made explicit when compared to JVET-AD2006. (JVET-AE0048 item 4)

BoG recommendation (Ed. clarification of existing intent): Adopt.

  - Signal nnpfc_chroma_loc_info_present_flag only when nnpfc_out_order_idc is not equal to 0. Infer nnpfc_chroma_loc_info_present_flag to be equal to 0 when not present. (JVET-AE0060 item 1)

BoG recommendation (BF): Adopt.

  - Move the syntax present when nnpfc_mode_idc is equal to 1 to be immediately after the syntax present when nnpfc_mode_idc is equal to 0, such that information about the network topology and parameters of the NNPF is after the NNPF properties regardless of the value of nnpfc_mode_idc. This also allows for sharing of the byte alignment syntax for nnpfc_mode_idc equal to 0 and nnpfc_mode_idc equal to 1, including merging of two syntax elements nnpfc_reserved_zero_bit_a and nnpfc_reserved_zero_bit_b into one syntax element nnpfc_reserved_zero_bit. (JVET-AE0126 item 1)

It was commented that if the tag is not understood, with the current scheme, other syntax can be skipped.

It was also commented that having the URI appear early in the syntax could be helpful for prefixing.

No action was recommended on this subtopic by the BoG.

  - Make either of the following changes: (JVET-AE0175)

    - Clarify that the base NNPF can only be in the ISO/IEC 15938-17 bitstream format and the update can be any formats.

    - Clarify that the nnpfc_mode_idc only indicates whether it is the base NNPF or updates relative to the base NNPF and the format exclusively indicates the format of the data regardless of the value of nnpfc_mode_idc.

The current sentence: “nnpfc_mode_idc equal to 0 indicates that this SEI message contains an ISO/IEC 15938-17 bitstream that specifies a base NNPF (when nnpfc_base_flag is equal to 1) or is an update relative to the base NNPF with the same nnpfc_id value (when nnpfc_base_flag is equal to 0).”

Suggestion 0: “nnpfc_mode_idc equal to 0 indicates that this SEI message contains an ISO/IEC 15938-17 bitstream. This ISO/IEC 15938-17 bitstream specifies a base NNPF (when nnpfc_base_flag is equal to 1) or is an update relative to the base NNPF with the same nnpfc_id value (when nnpfc_base_flag is equal to 0).”

Suggestion 1: “nnpfc_mode_idc equal to 0 indicates that the stuff is in the SEI message. nnpfc_mode_idc equal to 1 indicates the stuff is in a location identified by a URI. When nnpfc_mode_idc is equal to 0, the stuff is in the format specified by ISO/IEC 15938-17.”

BoG recommendation (Ed. clarification): Clarify using the approach in suggestion 1.

  - Change nnpfc_reserved_zero_bit(_a/_b) to nnpfc_alignment_zero_bit(_a/_b). It is asserted that there is no need to reserve the value 1 for this byte alignment bit, as the VVC and VSEI specs almost always just simply specify it as "shall be equal to 0". Only one of all cases in the latest VVC and VSEI specs is specified as a reserved bit, that is ptl_reserved_zero_bit. Even that one is not really necessary; however, that one is a bit special, as the bits in the PTL syntax structure are way more precious than other places, because the PTL syntax structure are often directly copied into top-level systems signalling/negotiation of capabilities. (JVET-AE0126 item 2)

BoG recommendation (cleanup/consistency/simplicity): Adopt.

  - Change the condition for the constraint on either nnpfcOutputPicWidth is not equal to CroppedWidth or nnpfcOutputPicHeight is not equal to CroppedHeight from "when nnpfc_pic_width_num_minus1, nnpfc_pic_width_denom_minus1, nnpfc_pic_height_num_minus1, and nnpfc_pic_height_denom_minus1 are present" to "when resolutionResamplingFlag is equal to 1". From the meaning point of view, it should be better to use resolutionResamplingFlag equal to 1 as the condition instead of the presence of the syntax elements, as the essence herein is really to disallow not changing either the width or height when the NNPF purpose includes resolution resampling. (JVET-AE0126 item 5)

BoG recommendation (Ed. simplification of expression): Adopt.

  - Move nnpfa_target_base_flag to be immediately after nnpfa_persistence_flag, such that the persistence flag, when present, immediately follows the cancel flag, thus these two flags specifying the persistence scope are together, same as all other SEI message syntax containing a cancel flag and a persistence flag. (JVET-AE0126 item 7)

BoG recommendation (cleanup/consistency): Adopt (subject to disposition of item 1 of JVET-AE0189; this was confirmed in review of the first version of the BoG report by the JVET plenary).

  - Update the constraint on when an NNPFC SEI message shall be a repetition of the first NNPFC SEI message in the CLVS to be as follows, such that it is clearly specified that the current CLVS is the CLVS containing the NNPFC SEI message nnpfcB, and that the first NNPFC SEI message nnpfcA is within the current CLVS: (JVET-AE0128 item 2)

When an NNPFC SEI message nnpfcB is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, which contains nnpfcB, and the value of nnpfc_base_flag is equal to 1, the NNPFC SEI message shall be a repetition of the first NNPFC SEI message nnpfcA with the same nnpfc_id value, in decoding order, within the current CLVS, i.e., the payload content of nnpfcB shall be the same as that of nnpfcA.

BoG recommendation (Ed. clarification and simplification of existing intent): State that “All NNPFC SEI messages in a CLVS that have a particular nnpfc_id value and nnpfc_base_flag equal to 1 shall have identical SEI payload content.” (or editorial equivalent)

  - To allow colourization on non-4:0:0 video, which was claimed to be the intent agreed at the previous meeting, remove "ChromaFormatIdc or" from the following constaint: (JVET-AE0141)

When ChromaFormatIdc or chromaUpsamplingFlag is not equal to 0, colourizationFlag shall be equal to 0.

BoG recommendation (Ed. correction to express existing intent): Remove ChromaFormatIdc from the constraint for when colourizationFlag shall be equal to 0.

- On persistence scopes of the NNPF SEI messages

Discussed in the BoG at 15:40 on 11 July

  - Cancel the persistence of an NNPFA SEI message with any subsequent NNPFA SEI message with the same nnpfa_target_id, Here, when compared to JVET-AD2006, the requirement on nnpfa_cancel_flag equal to 1 is removed for cancelling the persistence. (JVET-AE0048 item 5)

BoG recommendation (Ed. correction to express existing intent): Adopt. The editors are also requested to check/clarify the existing intent that if the current picture contains an SEI message (any SEI message that uses the typical persistence scope scheme) with a cancel flag equal to 1, the current picture is not within the scope of the previous SEI message (regardless of whether the SEI message is a suffix or prefix SEI message).

  - Update the constraint related to nnpfcTargetPictures and nnpfaTargetPictures by also considering the value of nnpfa_target_base_flag, such that the constraint is as follows: (JVET-AE0189 item 3)

Let the nnpfcTargetPictures be the set of pictures to which the NNPFC SEI message corresponding to the target NNPF pertains. Let nnpfaTargetPictures be the set of pictures for which the target NNPF is activated by the current NNPFA SEI message. It is a requirement of bitstream conformance that any picture included in nnpfaTargetPictures shall also be included in nnpfcTargetPictures.

BoG recommendation (Ed. correction to express existing intent): Adopt as expressed above (or editorial equivalent).

- On presence of NNPFC and NNPFA SEI messages and on NNPF activation

  - Allow multiple NNPFC and NNPFA SEI messages with different content in the same picture unit by excluding NNPFC and NNPFA SEI messages from the existing constraint in JVET-AD2005, subclause D.3.1. If item 3)a (i.e., JVET-AE0048 item 5) is agreed the NNPFA SEI message is constrained in a manner that the payload shall be identical for the same nnpfa_target_id value. Otherwise, an NNPFA SEI message cancelling the previous NNPFA SEI message followed by an NNPFA SEI with the same nnpfa_target_id for activating an NNPF are allowed for the same picture unit. (JVET-AE0049 item 1).

BoG recommendation (Ed. correction to express existing intent): Adopt.

BoG suggested editor action item: Editors are requested to make sure the blanket constraint in VVC on constraining SEI payload content to be the same within a PU does not impose a problem for SEI messages that contain ID values. No such blanket constraint appears to exist in HEVC, and this was intentional for some SEI messages such as tone mapping information and colour remapping information SEI messages.

  - If the design intent is that both a base NNPF and an update of the NNPF can be activated at the same time for the same picture (which is not the design intent), act on 4)b.i; otherwise (the intent is that only one of a base NNPF and an update of the NNPF can be activated for a picture), act on 4)b.ii. (JVET-AE0189 item 1)

    - Move nnpfa_target_base_flag to be not conditioned on the value of nnpfa_cancel_flag, and specify that, when nnpfa_cancel_flag is equal to 1, the NNPFA SEI message cancels the persistence of the target NNPF that was previously activated with the same value of nnpfa_target_id and nnpfa_target_base_flag.

    - Add a constraint such that there shall not be two or more NNPFA SEI message active (i.e, with nnpfa_cancel_flag equal to 0) for the same picture regardless their value of nnpfa_target_base_flag.

The BoG agreed that no action is needed on this, since the issue is resolved by the action on 4)a.

BoG suggested editor action item: Editors are requested to make sure it is clear that activating an “update” can (and basically always would) require using information from a “base” filter in order to interpret the update.

  - If it is intended to be disallowed for an NNPFA SEI message with nnpfa_target_base_flag equal to 0 to activate a base NNPF (and yes, that is the intent), add a constraint as in 4)c.i. Otherwise, add a NOTE as in 4)c.ii: (JVET-AE0189 item 2)

    - When nnpfa_target_base_flag in an NNPFA SEI message is equal to 0, there shall be at least one NNPFC SEI message with nnpfc_id equal to nnpfa_target_id and nnpfc_base_flag equal to 0 that precedes the NNPFA SEI message in decoding order.

    - NOTE –When nnpfa_target_base_flag is equal to 0, the target NNPF can be a base NNPF in the case where there is no NNPFC SEI message with nnpfc_id equal to nnpfa_target_id and nnpfc_base_flag is equal to 0 that precedes the NNPFA SEI message in decoding order.

BoG recommendation (Ed. correction to express existing intent): Adopt 4)c.i.

End of discussion of 11 July at 18:30 in BoG.

- On NNPF input pictues and repeated NNPF inference

Beginning of discussion of 12 July at 08:30 in BoG

  - Constrain the repeated inference to generate interpolated pictures up to the end of the bitstream only to NNPFs that perform only picture rate upsampling. (JVET-AE0049 item 2)

BoG recommendation (Ed. correction/simplification to express existing intent): Adopt.

  - Change the index range in input picture assignment for repeated inference at the end of a bitstream: it is proposed to remove "− j" from "When numInputPics is greater than 1, the following applies for each value of i in the range of j + 1 to numInputPics − j − 1, inclusive, in increasing order of i:" (JVET-AE0049 item 3, JVET-AE0142 item 1)

BoG recommendation (Ed. correction to express existing intent): Adopt.

  - As an asserted bug fix, in the following sentence in the NNPFC interface text, replace "inputPic" with "resampledPic": (JVET-AE0142 item 2)

If sourceWidth is equal to CroppedWidth and sourceHeight is equal to CroppedHeight, inputPic is set to be the same as sourcePic.

BoG recommendation (Ed. correction to express existing intent): Adopt.

  - As an asserted bug fix, in the NNPFC interface text, condition the following constraint on "when pictureRateUpsamplingFlag is equal to 1", because currently this constraint can also be applied for cases where pictureRateUpsamplingFlag is equal to 0: (JVET-AE0142 item 3)

It is a requirement of bitstream conformance that num_interpolated_pics[ i − 1 ] shall not be greater than 0.

It was asked if this should instead be for when the nnpf_purpose is equal to 8, but this was not advisable because of cases other than the end of the bitstream.

BoG recommendation (Ed. correction to express existing intent): Adopt (and num_interpolated_pics[ i − 1 ] shoud be nnpf_interpolated_pics[ i − 1 ]).

  - Add a syntax element nnpfa_no_prev_clvs_flag to the NNPFA SEI message. When equal to 1, nnpfa_no_prev_clvs_flag specifies that input pictures that would originate from a previous CLVS in output order are treated as if they were not present in the bitstream. When nnpfa_no_prev_clvs_flag is equal to 0, the current operation in JVET-AD2005 and JVET-AD2006 is kept unchanged. The value of nnpfa_no_prev_clvs_flag can be changed from 0 to 1, when the current CLVS is spliced from another bitstream next to the previous CLVS in output order and this NNPFA SEI message would cause one or more input pictures to be selected from the previous CLVS in output order. (JVET-AE0050 item 1)

The inferred values of luma and chroma equal to 0 associated with nnpfc_absent_input_pic_zero_flag equal to 1 were discussed, with a participant questioning the sensibility of these values. Another participant said a filter could recognize these as special values and do something different in response to them. In the JVET plenary review of the first version of the BoG report, this was further discussed, with no change recommended.

The next subitem is closely related.

  - Add a new flag named nnpfa_broken_link_flag in the NNPFA SEI message to specify whether the result of applying the target NNPF may result in broken-link / mismatch (i.e., filtered picture in decoder that is different from is corresponding filtered picture at encoder). When the bitstream is spliced at a CLVS that resulted in one or more input pictures of the target NNPF becomes unavailable, the value of nnpfa_broken_link_flag is set to be equal to 1. The nnpfa_broken_link_flag can be used by decoder to decide whether or not to activate / apply the NNPF. (JVET-AE0070 item 1)

The basic difference between the two approaches is that in approach “e”, the syntax indicates what to do, while in approach “f”, the syntax would just provide a warning without an indication of how the decoder should react to that warning.

BoG recommendation (cleanup of corner-case behaviour): Adopt approach 5)e.

  - Add nnpfa_no_foll_clvs_flag to the NNPFA SEI message, gated by nnpfa_persistence_flag being equal to 1. nnpfa_no_foll_clvs_flag equal to 1 specifies that when this NNPFA SEI message persists for the last PU of a CLVS in output order, the NNPFA SEI message is treated as if it persisted for the last PU of the current layer within the bitstream. In this case, the picture rate upsampling post-filter may be invoked for multiple sets of input pictures until pictures are interpolated between the last pair of input pictures within the CLVS in output order. When nnpfa_no_foll_clvs_flag is equal to 0, the current operation in JVET-AD2005 and JVET-AD2006 is kept unchanged. (JVET-AE0050 item 2)

The next subitem is related (but not the same).

BoG recommendation (cleanup of corner-case behaviour): Adopt.

  - At the end of a bitstream, add repeated inference of NNPF for creating pictures corresponding to input pictures (regardless of the purpose indicated by nnpfc_purpose except for picture rate upsampling) when there are multiple input pictures to which the NNPF applies: When an NNPF is active until the end of the bitstream and the current picture for which the NNPF is activated is the last picture in output order in the bitstream, the NNPF is applied up to but excluding a set of input pictures that would cause creation of any NNPF-generated picture after the last picture of the bitstream in output order. (JVET-AE0134 item 2)

It was commented that it would be more correct to use the nnpfa_output_flag rather than the nnpfc_input_pic_output_flag in this modification.

BoG recommendation (cleanup of corner-case behaviour): Adopt (subject to offline checking of the suggested modification, later confirmed in JVET plenary review of the first version of the BoG report to use nnpfa_output_flag).

- On NNPF generated pictures and NNPF output pictures

  - Enable persistent activation of an NNPF that outputs multiple filtered input pictures or interpolates pictures between multiple pairs of input pictures as follows: (JVET-AE0051)

    - In the NNPFA SEI semantics of the VSEI text: Derivation of the pictures for which the NNPF is activated when nnpfa_persistence_flag is equal to 1 at the smallest interval such that the same picture is not output by the NNPF process more than once in subsequent NNPF activations and interpolated pictures are not generated between the same pair of input pictures in subsequent NNPF activations.

    - In the NNPF SEI use of the VVC text: The repeated inference to generate interpolated pictures up to the end of the bitstream is performed at the smallest interval that does not cause interpolated pictures to be generated between the same pair of input pictures in subsequent NNPF inferences.

It was commented that the text changes for this seem a bit too complicated and it might really be more of a proposed extra functionality, and thus its consideration could be deferred to v4 or beyond.

The BoG thus recommended no action on this (at least for v3).

Another participant questioned the usefulness of NNPFA persistence in general, saying it could be simpler to just send an NNPFA for every picture (since they are small). It was noted that on the previous day it had been pointed out that persistence doesn’t really operate the same way for NNPF as it does for other SEI messages. In the interest of stability of the planned approach, this line of thinking was not further discussed.

  - Add the following NOTE to clarify an implied, hidden constraint on picture-rate-upsampling NNPFs with persistent activation: (JVET-AE0135 item 1)

NOTE – When pictureRateUpsamplingFlag is equal to 1 for an NNPF and the NNPFA SEI message that activated this NNPF has nnpfa_persistence_flag equal to 1, nnpfc_interpolated_pics[ i ] is greater than 0 only for a single value of i in the range of 0 to numInputPics − 1, inclusive.

It was asked whether the text should also express a similar observation for other cases, for example, when nnpfc_purpose indicates resolution upsampling only, when there are multiple input pictures and multiple output pictures. This suggestion was supported.

BoG recommendation (Ed.): Adopt (editorial only, with expansion to consider all possible cases that have a similar implication).

  - Act on one of the following three suggestions for picture-rate-upsampling NNPFs, to avoid the seemingly vagueness and easy violation of the constraint on no more than one NNPF output picture for any particular output time instance: (JVET-AE0135 item 2)

    - Clearly specify that the interpolated pictures are uniformly located between each pair of consecutive input pictures.

    - Explicitly indicate an output time instance for each interpolated picture, e.g., through signalling a POC delta between the POC values of the interpolated picture and the input picture closest and succeeding in output order.

It was commented that POC is not intended as timing information, although it could be used to establish relative order.

    - Disallow generating NNPF output pictures between any particular pair of consecutive input pictures more than once.

It was suggested that unless some strong need to do otherwise is identified, approach 6)c.iii seems like the practical action to take.

BoG recommendation (BF / clarification): Approach 6)c.iii, pending revisit for text review (remaining open at the time of the JVET plenary review of the first version of the BoG report).

  - Add the following constraint to ensure that there is at least one NNPF output picture when an NNPF is activated and applied: (JVET-AE0069)

When nnpfc_purpose & 0x08 is equal to 0 for the associated NNPFC [SEI message] and nnpfa_num_output_entries is equal to NumInpPicsInOutputTensor, nnpfa_output_flag[ i ] shall be equal to 1 for at least one value of i in the range of 0 to nnpfa_num_output_entries − 1, inclusive.

BoG recommendation (BF / clarification of existing intent): Adopt (with some editorial refinement).

  - Add the following constraint as part of the semantics of nnpfa_output_flag[ i ], to ensure that input pictures that are not present in the bitstream do not have a corresponding NNPF output picture: (JVET-AE0070 item 2)

When the target NNPF takes multiple input pictures and one or more of the input pictures are not present in the bitstream (i.e., unavailable input pictures) and the unavailable input pictures have associated output pictures (i.e., the value of their associated nnpfc_input_pic_output_flag[ i ] in NNPFC SEI message is equal to 1), for each of those unavailable input pictures that has associated output picture, the nnpfa_output_flag[ i ] associated with such input picture shall be present and be equal to 0.

It was commented that item f is related. See further notes for that item.

  - Add the following constraint as part of the process for derivation of the NNPF input pictures in the interface text, to ensure that input pictures that are not present in the bitstream do not have a corresponding NNPF-generated picture (and consequently do not have a corresponding NNPF output picture): (JVET-AE0134 item 1)

It is a requirement of bitstream conformance that, when inputPresentFlag[ i ] is equal to 0, nnpfc_input_pic_output_flag[ i ] shall be equal to 0.

It was commented that the desire is to prohibit output of unavailable pictures that are before the first picture in the bitstream. The control of the pictures that are output is in the NNPFA SEI message.

It was suggested that since a similar issue is addressed in the VVC interface text, it would be best to express this constraint there as well.

BoG recommendation (BF / clarification of existing intent): Adopt approach 6)e, expressed in the VVC interface text. This was agreed in the JVET plenary review of the first version of the BoG report to use the nnpfa_output_flag rather than the nnpfc_input_pic_output_flag in this modification.

- On NNPF purpose(s) determined by the application

  - Signal a new syntax element nnpfc_application_purpose_tag_uri when nnpfc_purpose is equal to 0 (i.e. when the NNPF may be used as determined by the application) to provide more details about the application-specific purpose. (JVET-AE0061 item 1)

As proposed, the tag URI would be mandatory to be present when nnpfc_purpose is equal to 0.

It was commented that nnpfc_purpose equal to 0 is for an application-specific purpose that may not benefit from the extra tag signalling; others said the tag could be useful for some external organization to document/indicate a particular purpose.

One participant indicated that allowing indication of an application-specific purpose should be sufficient, without needing to make additional provisions for support of information about application-specific purposes – e.g., additional information could be signalled in user data.

It was commented that this may be new functionality that could be deferred for later consideration beyond v3. There is a metadata extension capability that could indicate this.

It was suggested to make the tag presence optional. There was some support for that.

No action was recommended on this subtopic by the BoG for v3; this could be considered for a future version.

  - Signal one bit in the nnpfc_purpose syntax element to specify some additional purpose of the NNPF as determined by the application, for use in combination with other pre-defined NNPF usages. (JVET-AE0061 item 2)

It was commented that this could cause confusion with nnpfc_purpose equal to 0. The proponent pointed out that the proposal has a constraint to prevent setting only this bit equal to 1 without also indicating some defined purpose.

The proponent said another possibility is to allow this and remove the nnpfc_purpose equal to 0 (without the constraint that some other purpose needs to be indicated). Others indicated that text/design stability at this stage should be preferred over considering some significant change of how an application-specific purpose would be indicated.

Examples suggested by the proponent were dehazing (not a current defined purpose) together with some existing defined purpose such as chroma upsampling.

Participants other than the proponent did not see a clear need for this simultaneous indication of application-specific purpose along with other currently defined purposes. Such supplemental indications could potentially be accomplished in other ways (e.g. user data or system data).

It was commented that having this flag equal to 1 might cause decoders to become confused about whether to use the filter if they don’t understand the application-specific purpose, which could basically make the indication the same as only indicating an application-specific purpose.

No action was recommended on this subtopic by the BoG, at least for v3.

- On extensibilities

  - Add u(8)-coded nnpfc_purpose_len just before nnpfc_purpose, and change the coding of nnpfc_purpose from u(16) to (v). (JVET-AE0068)

One participant commented that by the time we use up all of the existing purpose bits, it may be unlikely we would want to keep using the same SEI message.

Another commented that there are other ways to deal with this, such as reserving an “escape” value; v3 decoders are already required to ignore the message if they don’t understand the indicated purpose.

Subitem b is related.

No action was recommended on subtopics a and b by the BoG.

  - Change the coding of nnpfc_purpose from u(16) to u(24) or ue(v). Additionally, this contribution demonstrates a mechanism on how to add additional NNPFC purposes in a future v4 version of VSEI such that, according to the proponents, a v4 decoder would decode v3 NNPFC SEI messages to indicate “unknown” for those additional purposes. The mechanism is claimed to work without any text changes to the v3 text, so no other change but the change to u(24) or ue(v) is being proposed. (JVET-AE0113)

Subitem a is related. See notes for that.

  - Specify the metadata extension mechanism, enabled by the syntax elements nnpfc_metadata_extension_num_bits and nnpfc_reserved_metadata_extension, to be applied only when nnpfc_mode_idc is equal to 0, by making the following changes: (JVET-AE0128 item 1)

    - The signalling of the syntax element nnpfc_metadata_extension_num_bits is skipped when nnpfc_mode_idc is not equal to 0.

    - It is specified that, when nnpfc_metadata_extension_num_bits is not present, its value is inferred to be equal to 0.

The proponent noted that the existing general SEI message extension mechanism is available for use when nnpfc_mode_idc is not equal to 0.

Another participant said it may be nicer to be able to use the same mechanism for multiple modes.

The proposed modification is not really necessary; there is no clear bug.

No action was recommended on this subtopic by the BoG.

The BoG stopped for a lunch break here at 12:30 on 12 July.

- On base NNPF and NNPF updates

The BoG resumed is discussions here at 14:30 on 12 July.

  - As an asserted bug fix, specify that all NNPFC SEI messages with a particular value of nnpfc_id within a CLVS shall have the same value of nnpfc_purpose. (JVET-AE0142 item 4)

BoG recommendation (BF / sensibility correction of existing intent): Adopt. This was further discussed in the JVET plenary review of the BoG report. The persistence of an NNPFC is for a CLVS and the indicated purpose is part of the NNPFC, so this constraint is to require alignment of the two. We already have a constraint that an NNPFC update that has property parameters signalled shall have the same indicated purpose as the base NNPF. It was suggested that “purpose” might really mean “capability”.

  - Make the following changes: (JVET-AE0173, updated)

    - Move nnpfa_target_base_flag out of the syntax condition "if( !nnpfa_cancel_flag )".

We can activate a base even after sending an update. The intended interpretation of the current syntax was discussed.

BoG recommendation (clarification of existing intent): Clarify, as necessary, that an NNPFA message can activate a base with a particular ID when an update is active, which will switch the filter from the updated filter to the base filter.

    - NNPFA message with nnpfa_cancel_flag equal to 0 and nnpfa_target_base_flag equal to 0 shall only be sent to the NNPF which is not active. The NNPFA messages with nnpfa_cancel_flag equal to 0 and nnpfa_target_base_flag equal to 1 shall be ignored if the NNPF whose nnpfc_id equal to nnpfa_target_id of the NNPFA message is not active when the message is received by the receiver.

It was commented that it should be OK to receive a cancel indication without having previously received relevant data with the same ID.

Other SEI messages use the word “any” to cover this possibility. The same word is used for this new one.

No action was recommended on this subtopic by the BoG.

    - (from updated contribution) Adding a flag nnpfa_cumulative_update_flag such that if nnpfa_cumulative_update_flag is equal to 1, the target NNPF is the NNPF specified by combination of all NNPFC SEI messages with nnpfc_id equal to nnpfa_target_id that precedes the first VCL NAL unit of the current picture in decoding order and after the NNPFC SEI message contains the base NNPF which is not a repetition.

It was commented that this proposes adding an additional mode of operation and we do not have evidence of benefit of the proposed cumulative update behaviour, and also that cumulative update could cause a loss resilience problem.

No action was recommended on this subtopic by the BoG, at least for v3.

    - Target of NNPFA message with nnpfa_cancel_flag is decided by the value of nnpfa_target_base_flag. If nnpfa_target_base_flag is equal to 0 then the target NNPF is completely deactivated and is no longer used until it is activated by another NNPFA SEI message with the same nnpfa_target_id as the current SEI message and nnpfa_cancel_flag equal to 0. If nnpfa_target_base_flag is equal to 1 then the updates applied to the base NNPF is deactivated, i.e. the base NNPF is applied to the pictures following the current NNPFA SEI message in decoding order.

This aspect had been clarified in the context of discussion of the first aspect.

No action was recommended on this subtopic by the BoG.

- Editorial changes.

BoG recommendation (Ed.): Delegate these minor refinements to the editors for consideration.

  - Clarify the description of the InpIdx[ idx ] variable to by adding "in inverse output order" as follows: (JVET-AE0048 item 5)

InpIdx[ idx ] specifying the input picture index of the idx-th picture in inverse output order that is present in the output tensor of the NNPF and has a corresponding input picture

  - Rename the syntax element nnpfc_input_pic_output_flag[ i ] to be nnpfc_input_pic_filtering_flag[ i ] and change the semantics accordingly, as this flag indicates whether for the i-th input picture the NNPF generates a corresponding filtered picture stored in the output tensor, not whether the NNPF outputs a corresponding picture, which is controlled by the nnpfa_output_flag[ i ] instead. This also includes changing of the name of the variable numOutputPics to numPicsInOutputTensor. (JVET-AE0126 item 3)

  - Change the local variables chromaUpsamplingFlag, resolutionResamplingFlag, pictureRateUpsamplingFlag, bitDepthUpsamplingFlag, and colourizationFlag to be global variables ChromaUpsamplingFlag, ResolutionResamplingFlag, PictureRateUpsamplingFlag, BitDepthUpsamplingFlag, and ColourizationFlag, because most, if not all, of them are used in more than one (sub)clauses of the text. (JVET-AE0126 item 4)

  - Change all instances of "Values of se_a greater than y shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use" to "The value of se_a shall be in the range of 0 to y, inclusive", as the latter is a wording consistent with the usual way of expressing the value range for a ue(v)-coded syntax element in AVC/HEVC/VVC/VSEI specifications. (JVET-AE0126 item 6)

  - Other miscellaneous proposed editorial changes not covered above (found in JVET-AE0126 attachment, JVET-AE0127 attachment, and ticket #1605)

- On new NNPF purposes/functionalities (the BoG suggested these are not for v3 consideration, as further study seemed necessary and late addition of new features seems undesirable)

  - Add decolourization (converting colour video to monochrome) as a new nnpfc_purpose. (JVET-AE0060 item 2)

It was suggested that “recolourization” may be different from “colourization”.

  - Add deinterlacing as a new NNPF purpose, by the following: (JVET-AE0062)

    - Signal a flag that indicates the number of output pictures for each interlaced input picture to the NNPF.

    - Signal if the input needs to be flipped vertically so that a neural network trained on top field first can work on bottom field first content and vice-versa.

It was commented that an NNPF might not only deinterlace content that is known to be interlaced, but might involve detecting whether the content is interlaced.

- On NNPF cascades, alternatives, etc. (the BoG and the contributions themselves suggested these are not for v3 consideration, as further study seemed necessary and late addition of new features seems undesirable)

  - Add support of NNPF cascades and alternatives as follows: (JVET-AE0052)

    - Add a new SEI message named the NNPF group characteristics (NNPFGC) SEI message, which can be used to define either a cascade of NNPFs or indicate a set of alternative NNPFs or alternative NNPF cascades.

    - Add a new SEI message named the NNPF group activation (NNPFGA) SEI message, which is used for activating an NNPF cascade.

    - Update the general post-processing filtering process using NNPFs to take into account the proposed NNPFGC and NNPFGA SEI messages.

  - Add a new SEI message NNPF group SEI message to indicate multiple NNPFs as a group/list and related operations for a picture. The related operations for the indicated multiple NNPFs consist of four types: cascade, alternate, coherence and option. (JVET-AE0063)

- On region-based NNPF operations (the BoG and the contributions themselves suggested these are not for v3 consideration, as further study seemed necessary and late addition of new features seems undesirable)

  - Add a new SEI message named the NNPF regions (NNPFR) SEI message. The NNPFR SEI message synchronizes the annotated regions SEI message with the NNPF SEI messages, identified by nnpfr_target_id, for post-processing filtering of regions in pictures. The regions in the pictures indicated by the annotated regions SEI message are activated by the NNPFA SEI message and post-filtered by the NNPFC SEI messages. (JVET-AE0053)

- On NNPF SEI messages and phase indication SEI messages for AVC and HEVC (the BoG suggested no immediate incorporation into these for Consent at this meeting, but encouraged further development, with possibly issuing some draft text(s) at this meeting and ISO/IEC timeline planning)

  - Proposes adding the support of the NNPFC SEI message, the NNPFA SEI message, and the phase indication SEI message to AVC and HEVC. (JVET-AE0101)

Non-NNPF topics reviewed in the BoG follow as items 15 and 16 below.

- JVET-AE0054 AHG2/AHG9: Editorial improvements of annotated regions SEI message

JVET-AE0054 AHG2/AHG9: Editorial improvements of annotated regions SEI message [T. Chujoh, T. Aono, T. Ikai (Sharp)]

This contribution was discussed in the BoG at 16:30 on Wednesday 12 July.

The annotated regions SEI message carries parameters that identify annotated regions using bounding boxes representing the size and location of identified objects and has been introduced in VSEI, HEVC and AVC. After reviewing the current their texts, several editorial issues have been found. They should be improved when revising VSEI, HEVC and AVC. This contribution proposes revised texts for each VSEI, HEVC and AVC.

The following problems exist in the texts of the annotated regions SEI message.

- The “ar_” prefix of several syntax elements is lacking. BoG recommendation (Ed.): Agreed.

- The offset variables for the conformance window are used in the semantics, but they are not needed because the input picture of the SEI message has been cropped.

The BoG disagreed with the interpretation and instead suggested only to clarify the existing specification as follows.

BoG recommendation (Ed.): Add a NOTE saying (approximately) “NOTE – Although some variables used in this subclause are related to picture cropping, the coordinates used in these semantics are relative to the entire decoded picture prior to cropping.”

- The name of the chroma format indicator differs between VSEI and HEVC/AVC.

ChromaArrayType does not exist in VVC. In HEVC and AVC, ChromaArrayType is assigned as follows:

–	If separate_colour_plane_flag is equal to 0, ChromaArrayType is set equal to chroma_format_idc.

–	Otherwise (separate_colour_plane_flag is equal to 1), ChromaArrayType is set equal to 0.

BoG recommendation (Ed. BF): Change draft VSEI to use ChromaFormatIdc instead of ChromaArrayType. If HEVC and/or AVC are changed to refer to VSEI instead of having an in-line definition of the annotated regions SEI message, we need to define that variable in the interface text when using the SEI message.

- In the case of VVC, the offset variables of the conformance window are defined on PPS, but in the case of HEVC/AVC, they are defined on SPS.

In VVC, no action is needed since the interface description does not refer to where the cropping window is specified.

BoG recommendation (Ed. BF): In VSEI, remove the phrase “specified by the active SPS” (since that is not correct for VVC).

- It is reported that many unknown characters are inserted in the test of Word versions of HEVC and AVC. It was pointed out that these are no-width optional line breaks that are being used intentionally, so no action was considered needed on this by the BoG.

- JVET-AE0155 Editor commentary on the post-filter hint SEI message semantics [G. J. Sullivan, Y.-K. Wang (Editors)]

JVET-AE0155 Editor commentary on the post-filter hint SEI message semantics [G. J. Sullivan, Y.-K. Wang (Editors)]

The authors provided this contribution to address issues encountered in the specification of the post-filter hint SEI message during the finalization editing of WG 5 N 179 (FDIS ISO/IEC 23008-2 5th edition) and JVET-AD2006 (Additional SEI messages for VSEI (Draft 4)) after consideration of item 4 of JVET-AD0078 (AHG9: On comments for the draft of VSEI [T. Chujoh, Y. Yasugi, T. Ikai (Sharp)]).

- The semantics do not say that the described post-filtering should be applied.

In discussion, it was noted that some other similar messages (e.g. film grain and colour remapping) are also not saying they should be applied (but tone mapping uses “should” and NNPF uses “may”).

- It does not seem clear whether the “16-bit precision” description applies only to the cross-correlation matrix case or also to the filter coefficient cases.

It was commented that the first version of this SEI message was in the 2009 edition [actually, see the H.264 2007-04 Amendment 2 pre-publication and H.264 2007-11 edition] of AVC and the original proposal was from Thomas Wedi and Steffen Wittmann of Panasonic. A couple of changes were introduced in the 2010 edition [H.264 2010-03] which may have introduced problems [it corrected a missing value range, although apparently more generously than necessary, and tried to clarify the scaling of the cross-correlation matrix data but accidentally basically scaled down by 216 rather than scaling up by that factor]. In the JVET plenary review of the first version of the BoG, it was commented that a JVT contribution JVT-U035 contained a software showcase that might help clarify the intent. Other contributions before that were JVT-S030 and JVT-T039.

- A specified 32-bit range may be excessive for a syntax element that has 16-bit precision (see remarks on previous aspect for the history on this).

- Much of the text is written from the encoder perspective. We generally do not specify things from the encoder perspective. The decoder-side processing is not clearly described or is discussed only vaguely. For example, no equations are given for the decoder’s filtering process.

- It does not seem clear whether the decoder should divide the filter tap values (or equivalently, the result of the filtering process) by the sum of the filter tap values or if the encoder is responsible for the filter normalization.

- There are issues relating to how to interpret the filter depending on whether the filter length is even (in which case the output samples may presumably be located spatially between input samples) or odd (in which case the location of an output sample may presumably have a location corresponding to the centre tap).

- The handling of the boundaries of the picture and cropping window are not described and seems unclear.

- It seems unclear why should it matter whether a horizontal filter or a vertical filter is applied first if both are to be applied. Perhaps integer rounding is expected to be applied between the two filtering operations, but this is not stated.

- The discussion of Wiener post-filtering is a bit vague (e.g., what exactly is a Wiener filter, and how would a decoder derive one from the provided data?).

- A decoder could use a subset of a provided cross-correlation matrix to derive a Wiener post-filter with a smaller region of support than the provided statistics would enable. It does not seem clear whether this is intended to be an acceptable or possible practice.

The contribution was written to point out the issues and does not contain an exact recommendation about what to do about them.

BoG recommendation (action considering text specification quality): Do not include this SEI message in VVC v3 and VSEI v3 due to its apparent lack of clarity.

Further study was recommended for future clarification/action.

It was discussed whether the film grain and colour remapping and NNPF SEI messages should say they “should” be used.

Another potential phrasing was “is suggested” rather than “should”.

The BoG initially recommended, as a clarification of existing intent, to, recommend that “should” be used for the film grain and NNPF SEI messages; but not to say this for colour remapping (since that has an ID value that could indicate whether it is appropriate or not appropriate for a particular decoding system to apply, e.g. depending on the target display). In the JVET plenary review of the first version of the BoG report, there was discussion of the meaning of “should” and the potential implications of using that word (and of changing a draft to introduce it). Discussed alternatives included “is suggested” and “is intended”. Some participants suggested that application standards are the appropriate place to provide guidance on usage and are customized to the application environment. Film grain synthesis was suggested to be an especially important functionality to encourage.

Tone mapping was another message to consider about phrasing. It is not in VSEI, but in AVC and HEVC it uses “should”.

JVET decision: Use “should” for film grain synthesis and retain prior drafted language for other SEI messages.

Further study toward some profile / conformance expression to include film grain synthesis support was encouraged.

The BoG discussions ended with this topic at 1830 on 12 July.

# List of NNPF-related considered contributions (26)

The list of NNPF-related contributions that were considered in this BoG, per section 6 of the draft JVET meeting notes, is as follows:

JVET-AE0048 AHG9: Miscellaneous VSEI changes on neural-network post-filter SEI messages [M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia)]

JVET-AE0049 AHG9: Miscellaneous VVC changes on neural-network post-filter SEI messages [M. M. Hannuksela, F. Cricri, M. Santamaria (Nokia)]

JVET-AE0050 AHG9: On NNPF input picture selection [M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AE0051 AHG9: On persistent NNPF activation [M. M. Hannuksela, F. Cricri (Nokia)]

JVET-AE0052 AHG9: NNPF cascades and alternatives [M. M. Hannuksela, F. Cricri (Nokia)] [late]

JVET-AE0053 AHG8/AHG9: Neural-network post-filter regions SEI message [T. Chujoh, Y. Yasugi, T. Ikai (Sharp)]

JVET-AE0060 [AHG9] Comments on NNPFC [S. Deshpande (Sharp)]

JVET-AE0061 [AHG9] On NNPFC Application Purpose [S. Deshpande (Sharp)]

JVET-AE0062 [AHG9] On NNPF for Deinterlacing [A. Sidiya, S. Deshpande (Sharp)]

JVET-AE0063 [AHG9] On operations for multiple NNPFs [L. Chen, O. Chubach, Y.-W. Huang, S. Lei (MediaTek)]

JVET-AE0068 AHG9: On extensibility of purpose syntax element in NNPFC SEI message [Hendry, J. Nam, S. Kim, J. Lim (LGE)]

JVET-AE0069 AHG9: On the signalling of output pictures in NNPFA SEI message [Hendry, J. Nam, S. Kim, J. Lim (LGE)]

JVET-AE0070 AHG9: On input pictures that are not present in the bitstream for NNPF SEI messages [Hendry, J. Nam, S. Kim, J. Lim (LGE)]

JVET-AE0101 [AHG2][AHG9] Neural network post filter and phase indication SEI messages for AVC and HEVC [T. Ikai, T. Chujoh (Sharp), Y.-K. Wang, J. Xu, W. Jia (Bytedance)]

JVET-AE0106 AHG9: On missing value ranges for some syntax elements in the NNPFC SEI message [C. Lin, Y.-K. Wang, J. Xu, W. Jia, J. Li, Y. Li, K. Zhang, L. Zhang (Bytedance)] [late]

JVET-AE0113 AHG9: Extendibility and code word length of nnpfc_purpose [R. Sjöberg, M. Pettersson (Ericsson)]

JVET-AE0126 AHG9: NNPF cleanup and editorial changes for VSEI [Y.-K. Wang, W. Jia, J. Xu, C. Lin (Bytedance)]

JVET-AE0127 AHG9: NNPF editorial changes for VVC [Y.-K. Wang, W. Jia, J. Xu (Bytedance)]

JVET-AE0128 AHG9: On NNPFC extensibility and base filter signalling [Y.-K. Wang (Bytedance)]

JVET-AE0134 AHG9: Align the design of NNPF with multiple input pictures to NNPF including picture rate upsampling [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AE0135 AHG9: On NNPF picture rate upsampling constraints [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AE0141 AHG9: Fix a bug in NNPFC SEI message for colourization [J. Xu, Y.-K. Wang (Bytedance)]

JVET-AE0142 AHG9: On derivation of NNPF input pictures and the value of nnpfc_purpose [W. Jia, Y.-K. Wang, J. Xu, L. Zhang (Bytedance)]

JVET-AE0173 [AHG9] Clarification and improvements of signaling of NNPF update [Y. Lim (Samsung)]

JVET-AE0175 [AHG9] Editorial improvements of nnpfc_mode_idc [Y. Lim (Samsung)]

JVET-AE0189 [AHG9] AHG9: On the design of nnpfa_target_base_flag in NNPFA SEI message [Hendry (LGE)] [late]

# List of other considered contributions (2)

See section 4.2 of the draft JVET meeting notes.

JVET-AE0054 AHG2/AHG9: Editorial improvements of annotated regions SEI message [T. Chujoh, T. Aono, T. Ikai (Sharp)]

JVET-AE0155 Editor commentary on the post-filter hint SEI message semantics [G. J. Sullivan, Y.-K. Wang (Editors)]