| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AD0288-v2 |
| --- | --- |

| Title: | Results of expert viewing for the spatial scalability category of the VVC multilayer VT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | M. Wien K. Andersson P. de Lagrange | Email: | wien@lfb.rwth-aachen.de kenneth.r.andersson@ericsson.com philippe.delagrange@interdigital.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

This document reports the results of an on-site expert viewing performed at the 31st JVET meeting in Geneva, CH.

The evaluation targets at the spatial scalability category of the test plan document JVET-AD2021, comparing the compression performance of dual layer (DL) coding with a downscaled base layer at scaling ratio 2 to single layer coding at the low resolution with upscaling to the target resolution using the VVC interpolation filters (UP) as well as single layer coding at the target resolution (SL).

A number of 29 experts participated in the expert viewing. Based on this larger database, the results reveal small confidence intervals and can be considered quite reliable.

With respect to the goals of this experiment, the following observations are made:

- DL shows a slight benefit over UP for DrivingPOV3 (overlapping confidence intervals), very minor for MountainBay2.

- No benefit or slight loss for DL compared to UP for Procession.

- More loss for DL compared to UP for CorsaireJoy, Marathon2, and, partially significant, for TallBuilding2.

- Relation of SL, DL, UP somewhat according to expectations on MountainBay2 (SL gaining at high rates, while DL, UP gaining at low rates.

- ISSUE observed for TallBuildings2: UP better than SL and DL for lower two rate points. Possible reason: less pumping artifacts for UP streams which are coded at a lower QP. Better detail of SL or DL does not compensate for this issue

- ISSUE observed for CorsaireJoy: UP rated better, although more detail is present in DL version. Inspection indicates that DL version also shows partially more visible artifacts.

The observed issues appear to be related to suboptimum bit allocation in the SL and DL configurations. It is recommended to take these results into account when setting up the set of four rate points for a formal evaluation.

# Introduction

This document reports the results of an on-site expert viewing performed at the 31st JVET meeting in Geneva, CH. The evaluation targets at the spatial scalability category of the test plan document JVET-AD2021 [1], comparing the compression performance of dual layer (DL) coding with a downscaled base layer at scaling ratio 2 to single layer coding at the low resolution with upscaling to the target resolution using the VVC interpolation filters (UP) as well as single layer coding at the target resolution (SL).

# Test setup

## Logistics

The setup at the meeting site included a PC with a Decklink video board for HDMI connection and SSD drives capable of stable playout of the raw YUV data at the required frame rate.

| Test Site | On-site |
| --- | --- |
| Display, size, connection (resolution setting) | Sony Bravia 65”, HDMI (3840×2160), 10bit input |
| Viewing distance | 2 viewers sitting at 1.5H, 3 views standing at 1.6H |
| Viewing angle | ±75°, 90° (at screen center) |
| Total number of viewers | 29 (5 female, 24 male) |

Participants confirmed visual acuity and normal colour vision.

The setup was chosen with two viewers sitting and three viewers standing right behind them at about the same distance was chosen to maximize the number of collected scores per session. The tests were performed with 9 groups of 3 to 5 participants with the seats and the center standing position covered first. The viewers entered their scores into the test database after the viewing session.

## Test sequences and quantizer settings

For evaluation, a subset of test sequences and quantizer settings listed in JVET-AD2021 were tested. These included the evaluation of the DL and UP configurations at the highest and the second-highest rate points for all tested sequences and an additional comparison to SL at the lowest rate point for the sequences not yet subjectively evaluated in the expert viewing tests in Antalya, see JVET-AD0399 [2]. For the upscaled base resolution sequences, bitstreams with approximately matching bitrates were generated as detailed below. A subset of the available streams was selected to adapt the duration of the viewing activity to the available time slot. The rate points considered for the verification test are represented by the corresponding QP values in the table below. The QP values printed in bold italics in this table have been selected and evaluated in the expert viewing.

Table 1 – Selected test sequences and QP settings for the viewing.

|  | Single layer | Upscaled SL | Dual layer 2x |  |
| --- | --- | --- | --- | --- |
| Sequence | QP SL | QP UP | QP BL | QP EL |
| CorsaireJoy | 33 | 27 | 27 | 34 |
| CorsaireJoy | 40 | 34 | 34 | 41 |
| DrivingPOV3 | 34 | 28 | 28 | 35 |
| DrivingPOV3 | 42 | 36 | 36 | 43 |
| Marathon2 | 32 | 26 | 26 | 33 |
| Marathon2 | 41 | 34 | 35 | 42 |
| MountainBay2 | 30 | 24 | 24 | 31 |
| MountainBay2 | 38 | 32 | 32 | 39 |
| MountainBay2 | 42 | 37 | 36 | 43 |
| Procession | 35 | 29 | 29 | 36 |
| Procession | 44 | 38 | 38 | 45 |
| TallBuildings2 | 34 | 26 | 28 | 35 |
| TallBuildings2 | 42 | 35 | 36 | 43 |
| TallBuildings2 | 46 | 39 | 40 | 47 |

### Configuration for upscaled base-resolution encoding

For the generation of upscaled base-resolutions encodings GOP-based RPR was used on the 4K sequences with a setting that forces encoding in 1920×1080 respectively 2540×1440 with indication of target resolution in SPS. After decoding 4K resolution is achieved by upscaling with VVC interpolation filters.

Following configuration settings are used for both base-resolutions encodings:

DMVREncMvSelect = 1, GOPBasedRPR = 1, UpscaledOutput = 2, GOPBasedRPRQPTh = 22,    QpOffsetRPR = 0,  QpOffsetRPR2 = 0, QpOffsetRPR3 = 0, QpOffsetChromaRPR = 0, QpOffsetChromaRPR2 = 0, QpOffsetChromaRPR3 = 0, UpscaleFilterForDisplay = 0

Specifically, for 1920x1080:

PsnrThresholdRPR = -100.0      # force 2x downscaling before encoding

Specifically, for 2540x1440:

PsnrThresholdRPR = 100.0

PsnrThresholdRPR2 = -100.0    # force 1.5x downscaling before encoding

### Rate distortion plots for the selected test points

For information, the Y PSNR for the test points evaluated in the expert viewing are provided in the figure below. Test points not assessed visually are omitted to maintain clarity in the presentation. The complete rate distortion data are provided in the accompanying csv-files.

CorsaireJoy	DrivingPOV3	Marathon2

MountainBay2	Procession	TallBuildings2

Figure 1: Rate-distortion plots for the test points evaluated in the expert viewing tests.

## Test method and test design

The assessment was performed with the reconstructed UHD output. A single DCR test session with 40 BTCs was presented to the viewers. The duration of the test session was about 20 min.

### Degradation category rating

#### Session setup

The test sequences were evaluated using an 11-grade scale shown in Figure 2 below.

Figure 2: Meaning of the used 11 grade numerical scale

The test session included 40 basic test cells (BTCs) showing the uncompressed video sequence and one processed video sequence. Each BTC was structured as followed (PVS denotes the processed video sequence under evaluation):

“Original” (1sec) – [uncompressed sequence] (10sec) – “A” (1sec) – [PVS] (10sec) – “Vote <N>” (5sec)

The session included a stabilization phase and three trapping BTCs where uncompressed sequences were to be scored.

#### Training

For the training, the training method reported in JVET-AD0399 was applied. The method is briefly summarized as follows:

- Explain procedure and scoring scale. 
- Run test session (6 BTCs, covering examples of the impairment range) 
- Let viewers note their scores. 
- Let them read out their scores. If variation is larger than 3 on the grading scale, re-run BTC 
   - Viewers shall reconsider their scoring for that sequence (not enforced to change their mind)
   - No indications where to look allowed, viewers have to make it up by themselves.

# Subjective results and analysis

## Data processing

As a first step, the complete set of votes experts was screened for the trapping sequences. One viewer was excluded on this basis. The Pearson correlation coefficient of the viewers scores relative to the MOS was assessed. The scores of two more viewers were removed based on a PCC threshold of 0.75%. As a last step, the z-scores of the viewer scores were compared to a threshold of 2. The z-score is computed as

zij= vij-MOSiσi, 

where vij is the score of viewer j for PVS i, MOSi is the mean opinion score for that PVS for the viewers, and σi is the standard deviation associated with MOSi. Out of 1040 scores, 41 (3.9%) were removed from the dataset on this basis.

## Subjective results

The measured MOS values of the reconstructed video on the 11-grade scale are plotted over the bit rate of the corresponding bitstream. The ±95% confidence intervals for the MOS values are indicated.

Figure 3: MOS-over-rate plots for the DCR experiment

UP = upscaled single layer coded at base resolution, DL = Dual layer, SL = Single layer at full resolution

# Discussion

The tests were performed with 29 participants which resembles an exceptionally good participation rate by JVET experts. Their effort is gratefully acknowledged. Based on this larger database, the results reveal small confidence intervals and can be considered quite reliable.

With respect to the goals of this experiment, the following observations are made:

- DL shows a slight benefit over UP for DrivingPOV3 (overlapping confidence intervals), very minor for MountainBay2.

- No benefit or slight loss for DL compared to UP for Procession.

- More loss for DL compared to UP for CorsaireJoy, Marathon2, and, partially significant, for TallBuilding2.

- Relation of SL, DL, UP somewhat according to expectations on MountainBay2 (SL gaining at high rates, while DL, UP gaining at low rates.

- ISSUE observed for TallBuildings2: UP better than SL and DL for lower two rate points. Possible reason: Less pumping artifacts for UP streams which are coded at a lower QP. Better detail of SL or DL does not compensate for this issue. Remarkably, the relation of PSNR-over-rate curves diverges significantly from the relation of the corresponding MOS-over-rate curves.

- ISSUE observed for CorsaireJoy: UP rated better, although more detail is present in DL version. Inspection indicates that DL version also shows partially more visible artifacts.

The observed issues appear to be related to suboptimum bit allocation in the SL and DL configurations.

It is recommended to take these results into account by updating the encoder configuration to be used in the verification tests.

# References

- S. Iwamura, P. de Lagrange, M. Wien, “Verification test plan for VVC multilayer coding,” Doc. JVET-AD2022, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 5, 30th meeting, online, Apr. 2023.

- M. Wien, “Results of expert viewing for the spatial scalability category of the VVC multilayer VT,” Doc. JVET-AD0399, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 5, 30th meeting, online, Apr. 2023.

- Recommendation ITU-T P.910 (2008), Subjective video quality assessment methods for multimedia applications.

- Recommendation ITU-R BT.500-14 (2019), Methodologies for the subjective assessment of the quality of television images.