| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0049-v1 |
| --- | --- |

| Title: | AHG9: Modality Information SEI for AVC and HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal, Information |  |  |
| Author(s) or Contact(s): | Jingying Gao Han Boon Teo Chong Soon Lim Kiyofumi Abe Virginie Drugeon | Tel: Email: | +65-6550-5406 jingying.gao@sg.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

In the 33rd JVET meeting, the modality information SEI message was adopted into the SEI messages for VSEI version 4 (Draft 1), JVET-AG2034. The modality information SEI message provides information about the source of optical radiation used to generate pictures. As infrared/ultraviolet imaging has many applications, such information would be beneficial to the legacy codec. Therefore, this contribution proposes to add the modality information SEI message to AVC and HEVC with the corresponding text.

# Introduction

Today's advanced image sensors, which operate across a variety of spectral ranges, are being integrated into cameras. These advanced image sensors are widely used in applications including autonomous vehicles, UAVs, industrial automation, and more. As illustrated in Figure 1, the picture can be captured from various ranges of spectrum, such as ultraviolet light, visible light, and infrared light. The source of light defines the picture modality type, and pictures of different modality types serve different purposes.

Figure 1: An example of the electromagnetic spectrum.

In the 33rd JVET meeting, signaling the modality information in SEI proposed in JVET-AG0322 [1] was adopted into the SEI messages for VSEI version 4 (Draft 1), with the document number of JVET-AG2034 [2]. The modality information SEI message provides information about the source of optical radiation used to generate pictures, for example, it specifies the type of modality such as visible picture, infrared picture, and ultraviolet picture, and the method of signaling the spectral band corresponding to the type of modality. As infrared/ultraviolet imaging has many applications, signaling the source properties at a high level would be beneficial to the legacy codec. In addition, the current text of the modality information SEI message is well-established to be included as it is in HEVC and AVC. Therefore, this contribution proposed to add the modality information SEI message to HEVC and AVC with the corresponding text.

- Proposed text for HEVC

## 2.4  Additional references

– ISO/IEC 23001-11 (in force), Information Technology – MPEG Systems technologies – Part 11: Energyefficient media consumption (green metadata).

– ISO 20473:2007, Optics and photonics – Spectral bands.

## D.2 SEI payload syntax

### D.2.1 General SEI message syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| else if( payloadType = = 210 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| nn_post_filter_characteristics( payloadSize ) |  |
| else if( payloadType = = 211 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| nn_post_filter_activation( payloadSize ) |  |
| else if( payloadType = = 212 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| phase_indication( payloadSize ) |  |
| … |  |
| else if( payloadType = = 218) |  |
| modality_info( payloadSize ) |  |
| … |  |
| } |  |

### D.2.xx Modality information SEI message syntax

| modality_info( payloadSize ) { | Descriptor |
| --- | --- |
| modality_info_cancel_flag | u(1) |
| if(!modality_info_cancel_flag) { |  |
| modality_info_persistence_flag | u(1) |
| modality_type | u(5) |
| spectrum_range_present_flag | u(1) |
| if( spectrum_range_present_flag ) { |  |
| min_wavelength_mantissa | u(11) |
| min_wavelength_exponent_plus15 | u(5) |
| max_wavelength_mantissa | u(11) |
| max_wavelength_exponent_plus15 | u(5) |
| } |  |
| modality_type_extension_bits | ue(v) |
| if(modality_type_extension_bits > 0 ) |  |
| reserved_modality_type_extension | u(v) |
| } |  |
| } |  |

## D.3 SEI payload semantics

### D3.1 General SEI payload semantics

The list SingleLayerSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 134 to 152, inclusive, 154 to 159, inclusive, 200 to 202, inclusive, 205, 210 to 212, inclusive, and 218.

The list VclAssociatedSeiList is set to consist of the payloadType values 2, 3, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 131, 132, 134 to 152, inclusive, 154 to 159, inclusive, 200 to 202, inclusive, 205, 210 to 212, inclusive, and 218.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 2, 6, 9, 15, 16, 17, 19, 22, 23, 45, 47, 56, 128, 129, 131, 132, 133, 135 to 152, inclusive, 154 to 159, inclusive, 200 to 202, inclusive, 205, 210 to 212, inclusive, and 218.

| Table D.1 – Persistence scope of SEI messages (informative) |  |
| --- | --- |
| SEI message | Persistence scope |
| User data registered by Rec. ITU-T T.35 | Unspecified |
| … | … |
| Modality information | Specified by the syntax of the SEI message |

### D.3.xx Modality information SEI message semantics

The modality information SEI message provides information about the source of optical radiation (such as visible light, infrared, or ultraviolet) used for generating the associated pictures and the wavelength of the spectrum band. As pictures of different modality types serve different purposes, the information conveyed in this SEI message can be used by a receiver to determine the purpose of the associated pictures.

NOTE 1– The interpretations of modality_type and the wavelength of the spectrum band associated with modality_type are specified by reference to the division of optical radiation specified in ISO 20473:2007.

modality_info_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous modality information SEI message in output order. modality_info_cancel_flag equal to 0 indicates that modality information follows.

modality_info_persistence_flag specifies the persistence of the modality information SEI message for the current layer.

modality_info_persistence_flag equal to 0 specifies that the modality information SEI message applies to the current decoded picture only.

Let picA be the current picture. modality_info_persistence_flag equal to 1 specifies that the modality information SEI message persists for the current layer in output order until any of the following conditions are true:

– A new CLVS of the current layer begins.

– The bitstream ends.

–A picture picB in the current layer in an access unit containing a modality information SEI message that is applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

modality_type indicates the type of modality of the associated pictures as specified in Table 1. When not present, the value of modality_type is inferred to be equal to 0, denoting that the modality type of the picture is unknown or unspecified or determined by other means not specified in this Specification. When modality_type is equal to 2 or 3, decoders shall ignore vui_colour_primaries, vui_transfer_characteristics, vui_matrix_coeffs, and vui_full_range_flag indicated in the VUI parameters

Table 1– Mapping of modality_type to the type of picture modalities

| modality_type | type of picture modality |
| --- | --- |
| 0 | Unspecified |
| 1 | Visible Picture |
| 2 | Infrared Picture |
| 3 | Ultraviolet Picture |
| 4..31 | Reserved for future use |

NOTE 2– When a reserved value of modality_type is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by modality_type being equal to that value or any one of a set of values including that value.

spectrum_range_present_flag equal to 1 specifies that the spectrum band of the optical radiation wavelength represented by the associated pictures is present in the modality information SEI message. spectrum_range_present_flag equal to 0 specifies that the spectrum band of the optical radiation wavelength represented by the associated pictures is not present in the modality information SEI message.

min_wavelength_mantissa specifies the mantissa part of the minimum wavelength indicating the spectral band of optical radiation represented by the associated pictures. When min_wavelength_mantissa is equal to 0 or is not present, the minimum wavelength indicating the spectral band of optical radiation represented by the associated pictures is unknown or unspecified or determined by other means not specified in this Specification

min_wavelength_exponent_plus15 minus 15 specifies the exponent part of the minimum wavelength indicating the spectral band of optical radiation represented by the associated pictures. When min_wavelength_mantissa is not present or is equal to 0, decoders shall ignore the value of min_wavelength_exponent_plus15.

The value of the minimum wavelength indicating the spectral band of optical radiation represented by the associated pictures is derived as follows:

MinWavelength = min_wavelength_mantissa * 10 min_wavelength_exponent_plus15-15

max_wavelength_mantissa specifies the mantissa part of the maximum wavelength indicating the spectral band of optical radiation represented by the associated pictures. When max_wavelength_mantissa is equal to 0 or is not present, the maximum wavelength indicating the spectral band of optical radiation represented by the associated pictures is unknown or unspecified or determined by other means not specified in this Specification.

max_wavelength_exponent_plus15 minus 15 specifies the exponent part of the maximum wavelength indicating the spectral band of optical radiation represented by the associated pictures. When max_wavelength_mantissa is not present or is equal to 0, decoders shall ignore the value of max_wavelength_exponent_plus15.

The value of the maximum wavelength indicating the spectral band of optical radiation represented by the associated pictures is derived as follows:

MaxWavelength = max_wavelength_mantissa * 10 max_wavelength_exponent_plus15-15

MinWavelength and MaxWavelength are in units of meters as specified in ISO/IEC 80000.

modality_type_extension_bits equal to 0 specifies that reserved_modality_type_extension is not present. modality_type_extension_bits greater than 0 specifies the length, in bits, of reserved_modality_type_extension.

The value of modality_type_extension_bits shall be in the range of 0 to 2 048, inclusive. Values in the range of 1 to 2 048, inclusive, for modality_type_extension_bits are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this document. Decoders conforming to this version of this document shall allow any value of modality_type_extension_bits in the range of 0 to 2 048, inclusive, to appear in the syntax.

reserved_modality_type_extension shall not be present in bitstreams conforming to this version of this document. However, decoders conforming to this version of this document shall allow the presence of reserved_modality_type_extension in the syntax, but ignore the value. When present, the length, in bits, of reserved_modality_type_extension is equal to modality_type_extension_bits.

- Proposed text for AVC

### 2  Normative references

– ISO 12232:2006, Photography – Digital still cameras – Determination of exposure index, ISO speed ratings, standard output sensitivity, and recommended exposure index.

– ISO 20473:2007, Optics and photonics – Spectral bands.

### D.1 SEI payload syntax

### D.1.1 General SEI message syntax

| sei_payload( payloadType, payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| … |  |  |
| else if( payloadType = = 210 ) |  |  |
| nn_post_filter_characteristics( payloadSize ) | 5 |  |
| else if( payloadType = = 211 ) |  |  |
| nn_post_filter_activation( payloadSize ) | 5 |  |
| else if( payloadType = = 212 ) |  |  |
| phase_indication( payloadSize ) | 5 |  |
| … |  |  |
| else if( payloadType = = 218 ) |  |  |
| modality_info( payloadSize ) | 5 |  |
| else |  |  |
| reserved_sei_message( payloadSize ) | 5 |  |
| if( !byte_aligned( ) ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 5 | f(1) |
| while( !byte_aligned( ) ) |  |  |
| bit_equal_to_zero /* equal to 0 */ | 5 | f(1) |
| } |  |  |
| } |  |  |

### D.1.xx Modality information SEI message syntax

| modality_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| modality_info_cancel_flag | 5 | u(1) |
| if(!modality_info_cancel_flag) { |  |  |
| modality_info_persistence_flag | 5 | u(1) |
| modality_type | 5 | u(5) |
| spectrum_range_present_flag | 5 | u(1) |
| if( spectrum_range_present_flag ) { |  |  |
| min_wavelength_mantissa | 5 | u(11) |
| min_wavelength_exponent_plus15 | 5 | u(5) |
| max_wavelength_mantissa | 5 | u(11) |
| max_wavelength_exponent_plus15 | 5 | u(5) |
| } |  |  |
| modality_type_extension_bits | 5 | ue(v) |
| if(modality_type_extension_bits > 0 ) |  |  |
| reserved_modality_type_extension | 5 | u(v) |
| } |  |  |
| } |  |  |

## D.2 SEI payload semantics

### D.2.xx Modality information SEI message semantics

The modality information SEI message provides information about the source of optical radiation (such as visible light, infrared, or ultraviolet) used for generating the associated pictures and the wavelength of the spectrum band. As pictures of different modality types serve different purposes, the information conveyed in this SEI message can be used by a receiver to determine the purpose of the associated pictures.

NOTE 1– The interpretations of modality_type and the wavelength of the spectrum band associated with modality_type are specified by reference to the division of optical radiation specified in ISO 20473:2007.

modality_info_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous modality information SEI message in output order. modality_info_cancel_flag equal to 0 indicates that modality information follows.

modality_info_persistence_flag specifies the persistence of the modality information SEI message.

modality_info_persistence_flag equal to 0 specifies that the modality information SEI message applies to the current decoded picture only.

Let picA be the current picture. modality_info_persistence_flag equal to 1 specifies that the modality information SEI message persists in output order until one or more of the following conditions are true:

– A new coded video sequence begins.

– The bitstream ends.

– A picture picB in an access unit containing a modality information SEI message is output having PicOrderCnt( picB ) greater than PicOrderCnt( picA ).

modality_type indicates the type of modality of the associated pictures as specified in Table 1. When not present, the value of modality_type is inferred to be equal to 0, denoting that the modality type of the picture is unknown or unspecified or determined by other means not specified in this Specification. When modality_type is equal to 2 or 3, decoders shall ignore vui_colour_primaries, vui_transfer_characteristics, vui_matrix_coeffs, and vui_full_range_flag indicated in the VUI parameters

Table 1– Mapping of modality_type to the type of picture modalities

| modality_type | type of picture modality |
| --- | --- |
| 0 | Unspecified |
| 1 | Visible Picture |
| 2 | Infrared Picture |
| 3 | Ultraviolet Picture |
| 4..31 | Reserved for future use |

NOTE 2– When a reserved value of modality_type is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by modality_type being equal to that value or any one of a set of values including that value.

spectrum_range_present_flag equal to 1 specifies that the spectrum band of the optical radiation wavelength represented by the associated pictures is present in the modality information SEI message. spectrum_range_present_flag equal to 0 specifies that the spectrum band of the optical radiation wavelength represented by the associated pictures is not present in the modality information SEI message.

min_wavelength_mantissa specifies the mantissa part of the minimum wavelength indicating the spectral band of optical radiation represented by the associated pictures. When min_wavelength_mantissa is equal to 0 or is not present, the minimum wavelength indicating the spectral band of optical radiation represented by the associated pictures is unknown or unspecified or determined by other means not specified in this Specification

min_wavelength_exponent_plus15 minus 15 specifies the exponent part of the minimum wavelength indicating the spectral band of optical radiation represented by the associated pictures. When min_wavelength_mantissa is not present or is equal to 0, decoders shall ignore the value of min_wavelength_exponent_plus15.

The value of the minimum wavelength indicating the spectral band of optical radiation represented by the associated pictures is derived as follows:

MinWavelength = min_wavelength_mantissa * 10 min_wavelength_exponent_plus15-15

max_wavelength_mantissa specifies the mantissa part of the maximum wavelength indicating the spectral band of optical radiation represented by the associated pictures. When max_wavelength_mantissa is equal to 0 or is not present, the maximum wavelength indicating the spectral band of optical radiation represented by the associated pictures is unknown or unspecified or determined by other means not specified in this Specification.

max_wavelength_exponent_plus15 minus 15 specifies the exponent part of the maximum wavelength indicating the spectral band of optical radiation represented by the associated pictures. When max_wavelength_mantissa is not present or is equal to 0, decoders shall ignore the value of max_wavelength_exponent_plus15.

The value of the maximum wavelength indicating the spectral band of optical radiation represented by the associated pictures is derived as follows:

MaxWavelength = max_wavelength_mantissa * 10 max_wavelength_exponent_plus15-15

MinWavelength and MaxWavelength are in units of meters as specified in ISO/IEC 80000.

modality_type_extension_bits equal to 0 specifies that reserved_modality_type_extension is not present. modality_type_extension_bits greater than 0 specifies the length, in bits, of reserved_modality_type_extension.

The value of modality_type_extension_bits shall be in the range of 0 to 2 048, inclusive. Values in the range of 1 to 2 048, inclusive, for modality_type_extension_bits are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this document. Decoders conforming to this version of this document shall allow any value of modality_type_extension_bits in the range of 0 to 2 048, inclusive, to appear in the syntax.

reserved_modality_type_extension shall not be present in bitstreams conforming to this version of this document. However, decoders conforming to this version of this document shall allow the presence of reserved_modality_type_extension in the syntax, but ignore the value. When present, the length, in bits, of reserved_modality_type_extension is equal to modality_type_extension_bits.

## G.13 Supplemental enhancement information

The specifications in Annex ‎D together with the extensions and modifications specified in this clause apply.

### G.13.2 SEI payload semantics

The semantics of the SEI messages with payloadType in the range of 0 to 23, inclusive, or equal to 45, 47, 137, 142, 144, 147, 148, 149, 150, 151, 154, 155, 156, 200, 201, 202, 205, 210, 211, 212 or 218, which are specified in clause ‎D.2, are extended as follows:

–	If payloadType is equal to 3, 8, 19, 20, or 22, the following applies:

–	If the SEI message is not included in a scalable nesting SEI message, it applies to the layer representations of the current access unit that have dependency_id equal to 0 and quality_id equal to 0.

The semantics as specified in clause ‎D.2 apply to the bitstream that would be obtained by invoking the bitstream extraction process as specified in clause ‎G.8.8.1 with dIdTarget equal to 0 and qIdTarget equal to 0. All syntax elements and derived variables that are referred to in the semantics in clause ‎D.2 are syntax elements and variables for layer representations with dependency_id equal to 0 and quality_id equal to 0. All SEI messages that are referred to in clause ‎D.2 are SEI messages that apply to layer representations with dependency_id equal to 0 and quality_id equal to 0.

–	Otherwise (the SEI message is included in a scalable nesting SEI message), the SEI message applies to all layer representations of the current access unit for which DQId is equal to any value of ( ( sei_dependency_id[ i ] << 4 ) + sei_quality_id[ i ] ) with i in the range of 0 to num_layer_representations_minus1, inclusive.

For each value of i in the range of 0 to num_layer_representations_minus1, inclusive, the semantics as specified in clause ‎D.2 apply to the bitstream that would be obtained by invoking the bitstream extraction process as specified in clause ‎G.8.8.1 with dIdTarget equal to sei_dependency_id[ i ] and qIdTarget equal to sei_quality_id[ i ]. All syntax elements and derived variables that are referred to in the semantics in clause ‎D.2 are syntax elements and variables for layer representations with dependency_id equal to sei_dependency_id[ i ] and quality_id equal to sei_quality_id[ i ]. All SEI messages that are referred to in clause ‎D.2 are SEI messages that apply to layer representations with dependency_id equal to sei_dependency_id[ i ] and quality_id equal to sei_quality_id[ i ].

–	Otherwise, if payloadType is equal to 2, 6, 7, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 21, 23, 45, 47, 137, 142, 144, 147, 148, 149, 150, 151, 154, 155, 156, 200, 201, 202, 205, 210, 211, 212 or 218, the following applies:

–	If the SEI message is not included in a scalable nesting SEI message, it applies to the dependency representations of the current access unit that have dependency_id equal to 0.

The semantics as specified in clause ‎D.2 apply to the bitstream that would be obtained by invoking the bitstream extraction process as specified in clause ‎G.8.8.1 with dIdTarget equal to 0. All syntax elements and derived variables that are referred to in the semantics in clause ‎D.2 are syntax elements and variables for dependency representations with dependency_id equal to 0. All SEI messages that are referred to in clause ‎D.2 are SEI messages that apply to dependency representations with dependency_id equal to 0.

–	Otherwise (the SEI message is included in a scalable nesting SEI message), the scalable nesting SEI message containing the SEI message shall have all_layer_representations_in_au_flag equal to 1 or, when all_layer_representations_in_au_flag is equal to 0, all values of sei_quality_id[ i ] present in the scalable nesting SEI message shall be equal to 0. The SEI message that is included in the scalable nesting SEI message applies to all dependency representations of the current access unit for which dependency_id is equal to any value of sei_dependency_id[ i ] with i in the range of 0 to num_layer_representations_minus1, inclusive.

For each value of i in the range of 0 to num_layer_representations_minus1, inclusive, the semantics as specified in clause ‎D.2 apply to the bitstream that would be obtained by invoking the bitstream extraction process as specified in clause ‎G.8.8.1 with dIdTarget equal to sei_dependency_id[ i ]. All syntax elements and derived variables that are referred to in the semantics in clause ‎D.2 are syntax elements and variables for dependency representations with dependency_id equal to sei_dependency_id[ i ]. All SEI messages that are referred to in clause ‎D.2 are SEI messages that apply to dependency representations with dependency_id equal to sei_dependency_id[ i ].

When payloadType is equal to 10 for the SEI message that is included in a scalable nesting SEI message, the semantics for sub_seq_layer_num of the sub-sequence information SEI message is modified as follows:

sub_seq_layer_num specifies the sub-sequence layer number of the current picture. When the current picture resides in a sub-sequence for which the first picture in decoding order is an IDR picture, the value of sub_seq_layer_num shall be equal to 0. For a non-paired reference field, the value of sub_seq_layer_num shall be equal to 0. sub_seq_layer_num shall be in the range of 0 to 255, inclusive.

–	Otherwise, if payloadType is equal to 0 or 1, the following applies:

–	If the SEI message is not included in a scalable nesting SEI message, the following applies. When the SEI message and all other SEI messages with payloadType equal to 0 or 1 not included in a scalable nesting SEI message are used as the buffering period and picture timing SEI messages for checking the bitstream conformance according to Annex ‎C and the decoding process specified in clauses ‎2 to ‎9 is used, the bitstream shall be conforming to this Recommendation | International Standard.

The value of seq_parameter_set_id in a buffering period SEI message not included in a scalable nesting SEI message shall be equal to the value of seq_parameter_set_id in the picture parameter set that is referenced by the layer representation with DQId equal to 0 of the primary coded picture in the same access unit.

–	Otherwise (the SEI message is included in a scalable nesting SEI message), the following applies. When the SEI message and all other SEI messages with payloadType equal to 0 or 1 included in a scalable nesting SEI message with identical values of sei_temporal_id, sei_dependency_id[ i ], and sei_quality_id[ i ] are used as the buffering period and picture timing SEI messages for checking the bitstream conformance according to Annex ‎C, the bitstream that would be obtained by invoking the bitstream extraction process as specified in clause ‎G.8.8.1 with tIdTarget equal to sei_temporal_id, dIdTarget equal to sei_dependency_id[ i ], and qIdTarget equal to sei_quality_id[ i ] shall be conforming to this Recommendation | International Standard.

In the semantics of clauses ‎D.2.1 and ‎D.2.3, the syntax elements num_units_in_tick, time_scale, fixed_frame_rate_flag, nal_hrd_parameters_present_flag, vcl_hrd_parameters_present_flag, low_delay_hrd_flag, and pic_struct_present_flag and the derived variables NalHrdBpPresentFlag, VclHrdBpPresentFlag, and CpbDpbDelaysPresentFlag are substituted with the syntax elements vui_ext_num_units_in_tick[ i ], vui_ext_time_scale[ i ], vui_ext_fixed_frame_rate_flag[ i ], vui_ext_nal_hrd_parameters_present_flag[ i ], vui_ext_vcl_hrd_parameters_present_flag[ i ], vui_ext_low_delay_hrd_flag[ i ], and vui_ext_pic_struct_present_flag[ i ] and the derived variables VuiExtNalHrdBpPresentFlag[ i ], VuiExtVclHrdBpPresentFlag[ i ], and VuiExtCpbDpbDelaysPresentFlag[ i ].

The value of seq_parameter_set_id in a buffering period SEI message included in a scalable nesting SEI message with the values of sei_dependency_id[ i ] and sei_quality_id[ i ] shall be equal to the value of seq_parameter_set_id in the picture parameter set that is referenced by the layer representation with DQId equal to (( sei_dependency_id[ i ] << 4 ) + sei_quality_id[ i ]) of the primary coded picture in the same access unit.

–	Otherwise (payloadType is equal to 4 or 5), the corresponding SEI message semantics are not extended.

When an SEI message having a particular value of payloadType equal to 137 or 144, contained in a scalable nesting SEI message, and applying to a particular combination of dependency_id, quality_id, and temporal_id is present in an access unit, the SEI message with the particular value of payloadType applying to the particular combination of dependency_id, quality_id, and temporal_id shall be present a scalable nesting SEI message in the IDR access unit that is the first access unit of the coded video sequence.

All SEI messages having a particular value of payloadType equal to 137 or 144, contained in scalable nesting SEI messages, and applying to a particular combination of dependency_id, quality_id, and temporal_id present in a coded video sequence shall have the same content.

For the semantics of SEI messages with payloadType in the range of 0 to 23, inclusive, or equal to 45, 47, 137, 142, 144, 147, 148, 149, 150, 151, 154, 155, 156, 200, 201, 202, 205, 210, 211, 212 or 218, which are specified in clause ‎D.2, SVC sequence parameter set is substituted for sequence parameter set; the parameters of the picture parameter set RBSP and SVC sequence parameter set RBSP that are in effect are specified in clause ‎G.7.4.1.2.1.

Coded video sequences conforming to one or more of the profiles specified in Annex ‎G shall not include SEI NAL units that contain SEI messages with payloadType in the range of 36 to 44, inclusive, or equal to 46, which are specified in clause ‎H.13, or with payloadType in the range of 48 to 53, inclusive, which are specified in clause ‎I.13.

When an SEI NAL unit contains an SEI message with payloadType in the range of 24 to 35, inclusive, which are specified in clause ‎G.13, it shall not contain any SEI message that has payloadType less than 24 or equal to 45, 47, 137, 142, 144, 147, 148, 149, 150, 151, 154, 155, 156, 200, 201, 202, 205, 210, 211, 212 or 218 that is not included in a scalable nesting SEI message, and the first SEI message in the SEI NAL unit shall have payloadType in the range of 24 to 35, inclusive.

When an SEI NAL unit contains an SEI message with payloadType equal to 24, 28, or 29, it shall not contain any SEI message with payloadType not equal to 24, 28, or 29.

When a scalable nesting SEI message (payloadType is equal to 30) is present in an SEI NAL unit, it shall be the only SEI message in the SEI NAL unit.

The semantics for SEI messages with payloadType in the range of 24 to 35, inclusive, are specified in the following.

# Reference

[1] J. Gao, H. B. Teo, C. S. Lim, K. Abe, V. Drugeon, “AHG9: On Picture Modality Information for SEI”, JVET-AG0322, 33rd Meeting, Online, 17–26 January 2024.

[2] S. McCarthy, J. Chen, S. Deshpande, M. M. Hannuksela, Hendry, G. J. Sullivan, and Y.-K. Wang, “SEI messages for VSEI version 4 (Draft 1) ”, JVET-AF2034, 33rd Meeting, Online, 17–26 January 2024.

# Patent rights declaration(s)

Panasonic may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).