| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 14–24 April 2024 | Document: JVET-AH0051-v1 |
| --- | --- |

| Title: | EE1-5: Study of the NN architecture at Very Low Operational Point |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm) Junru Li, Yue Li, Chaoyi Lin, Kai Zhang, Li Zhang (Bytedance) Zhihuang Xie, Yue Yu, Haoping Yu, Dong Wang (OPPO) | Email: | dmytror@qti.qualcomm.com yli30@qti.qualcomm.com lijunru@bytedance.com yue.li@bytedance.com xiezhihuang@oppo.com |
| Source: | Qualcomm, Bytedance, OPPO |  |  |

_____________________________

# Abstract

In this test, a study on NN architecture for Very Low Operational Point, targeting complexity of 5kMAC/pixel level is conducted. A combination of separable convolutions in fusion/transition block (JVET-AG0155) and filter configurations of JVET-AG0057 and JVET-AF0206 was independently trained and tested by the proponents.

Inference was conducted in int16 implementation with NNVC8.0. Resulting SADL model has 16k parameters and complexity of 5.1kMAC/pixel.

Proposed EE1-5 with nnIntra OFF reportedly provides the following BD-rate changes vs VTM anchor:

RA: { -3.7%, -4.8%, -4.0%}, AI: {-3.7%, -4.7%, -4.5%}.

Proposed EE1-5 with nnIntra ON reportedly provides the following BD-rate changes vs VTM anchor:

RA: { -5.3%, -5.5%, -5.0%}, AI: {-7.1%, -7.8%,-7.7%}.

It is proposed to consider establishing VLOP for NNVC and adopting EE1-5 filter into NNVC.

# Introduction

A Unified Filter Architecture for the LOP and its training was proposed in JVET-AF0043 and adopted to NNVC as LOP2, see Figure 1. LOP2 filter has complexity of 17kMAC/pixel implemented with a single model that consist of 50K parameters.

In test EE1-1.0, the LOP2 training strategy was refined and selected as anchor to the LOP tests for this meeting cycle. The hyper parameters of LOP2 training selected in EE1-1.0 are shown below:

- batch size 64

- rate 0.0008

- ‘milestones’ 41, 51, 55.

Figure 1 LOP2 network architecture.

Furthermore, several inputs to investigate applicability of LOP Unified Filter Architecture for Very Low Operational Points (5kMAC) have been made in JVET-AG0155, and JVET-AG0057 and JVET-AF0206.

In this EE1-5 test, a combination of separable convolutions in fusion/transition block (JVET-AG0155) and filter configurations of JVET-AG0057 and JVET-AF0206 is jointly trained by proponents.

# Proposed architecture

A candidate VLOP filter is shown in Figure 2. The proposed filter is based on LOP architecture and introduces separable convolutions in fusion/transition block (JVET-AG0155) and filter configurations of JVET-AG0057 and JVET-AF0206. The proposed architecture has complexity of 5.1 kMAC/pixel and 16k parameters.

Figure 2 EE1-5 VLOP filter.

# Training process

The design was independently trained by proponents using Stage 3 LOP2 dataset following EE1-1.0 training procedure. Summary of training information is shown in Table 1.

Table 1 Training information.

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Nvidia GeForce RTX4090 / A100 |
|  | Framework: | Pytorch v1.12.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | Stage3: 60 epochs |
|  | Batch size: | Stage3: 64 |
|  | Loss function: | L1, L2 |
|  | Training time (for 1 model): | ~7 to 10 days per stage |
|  | Training data information: | LOP2 Stage 3 dataset: DIV2K, BVI-DVC, TVD |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | Stage3: 8e-4 |
|  | Learning rate update strategy | Stage3: 41, 51, 55. mse-epoch:58 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | flipped, rotated |
|  | Mini-batch selection process: | random cropped |
|  | Training data update strategy: |  |
|  | Other information: |  |

# Inference process

Inference testing was conducted using NNVC8.0 software by Qualcomm, Bytedance and OPPO independently. The inference information is shown in Table 2, summary of simulation are presented in Tables 3, and 4. Complete simulation results are provided along with this document.

Table 2 Inference information

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | SADL float, SADL int16 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters | 16000 |
|  | Total Parameter Number |  |
|  | Parameter Precision (Bits) | Floating (32 bits) / integer (16 bits) |
|  | Memory Parameter (MB) |  |
|  | MAC/Pixel (Kilo) | 5.1 |
| Optional |  |  |
|  | Total Conv. Layers | 59 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size | 128x128 / 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Other information: |  |

Summary of comparative results for proposed method (int16, nnIntra OFF) comparing to VTM Anchor is shown in Table 3.

Table 3 Inference testing results, BD-rate change comparing to VTM Anchor.

|  | EE1-5 vs VTM Anchor, Int16 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RA |  |  | AI |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -4.05% | -3.79% | -3.60% | -3.39% | -4.35% | -4.21% |
| Class A2 | -3.95% | -4.40% | -2.71% | -3.54% | -4.89% | -3.44% |
| Class B | -3.43% | -5.47% | -4.95% | -3.42% | -4.92% | -5.24% |
| Class C | -3.55% | -5.13% | -4.18% | -3.39% | -5.11% | -5.06% |
| Class E |  |  |  | -4.80% | -3.84% | -4.11% |
| Overall | -3.69% | -4.83% | -4.03% | -3.66% | -4.68% | -4.54% |
| Class D | -4.00% | -5.44% | -5.10% | -3.67% | -5.01% | -5.50% |
| Class F | -1.82% | -3.54% | -2.83% | -2.27% | -4.45% | -4.24% |

Summary of comparative results for proposed method (int16, nnIntra ON) comparing to VTM Anchor is shown in Table 4.

Table 4 Inference testing results with nnIntra ON, BD-rate change comparing to VTM anchor.

|  | EE1-5 vs VTM Anchor, Int16 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | RA |  |  | AI |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | -6.13% | -4.82% | -5.32% | -7.49% | -8.00% | -8.23% |
| Class A2 | -5.26% | -4.53% | -3.53% | -6.25% | -7.29% | -6.16% |
| Class B | -5.05% | -5.96% | -5.57% | -6.48% | -7.64% | -8.22% |
| Class C | -5.05% | -6.22% | -5.31% | -6.37% | -7.86% | -7.91% |
| Class E |  |  |  | -9.27% | -8.43% | -7.81% |
| Overall | -5.31% | -5.51% | -5.04% | -7.05% | -7.82% | -7.74% |
| Class D | -5.27% | -6.32% | -6.11% | -6.42% | -7.61% | -8.31% |
| Class F | -2.85% | -4.21% | -3.48% | -4.39% | -6.21% | -5.97% |

# Conclusion:

EE1-5 tested a combination of separable convolutions in fusion/transition block (JVET-AG0155) and filter configurations of JVET-AG0057 and JVET-AF0206. The proposed filter was independently trained and tested by proponents.  Inference was conducted in int16 implementation with NNVC8.0. Resulting SADL model has 16k parameters and complexity of 5.1kMAC/pixel.

Proposed EE1-5 with nnIntra OFF reportedly provides the following BD-rate changes vs VTM anchor:

RA: { -3.7%, -4.8%, -4.0%}, AI: {-3.7%, -4.7%, -4.5%}.

Proposed EE1-5 with nnIntra ON reportedly provides the following BD-rate changes vs VTM anchor:

RA: { -5.3%, -5.5%, -5.0%}, AI: {-7.0%, -7.8%,-7.7%}.

It is proposed to consider establishing VLOP for NNVC and adopting EE1-5 filter into NNVC.

# References

- D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm), J. N. Shingala, A. Shyam (Ittiam), T. Shao, P. Yin (Dolby), “AHG11: Status of the joint EE1-0 (LOP.2) unified filter training”, JVET-AF0043, October 2023.

- D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm), “EE1-Related: On LOP2 training process”, JVET-AG0156, January 2024.

- Z. Xie, Y. Yu, H. Yu, D. Wang(OPPO), “[AHG11] Study on lower-complexity NNLF”, JVET-AG0155, January 2024.

- D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm), “EE1-Related: On Low Complexity Operational Point for In-Loop Filtering”, JVET-AG0155, January 2024.

- J. Li, Y. Li, C. Lin, K. Zhang, L. Zhang (Bytedance), “AHG11: Complexity reduction of NN-based loop-filters”, JVET-AF0206, October 2023.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).