| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document:JVET-AH0063-v3 |
| --- | --- |

| Title: | AHG12: On Shifting Quantization Center |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Yu, Haoping Yu, Luhang Xu, Jonathan Gan, Fan Wang, Zhihuang Xie, Dong Wang | Tel: Email: | yue.yu@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

A method of shifting quantization centers was adopted in the ECM software where the shifting amount is inversely proportional to the quantization indices of DQ. This contribution first proposes to also apply the quantization center shifting in the transform skip mode. In addition, this contribution further proposes for both RRC and TSRC the amount of quantization center shifting is inversely proportional to the quantization level of individual quantizer instead of the quantization indices of DQ. On top of ECM-12.0, the simulation results are reported as follows.

AI:   	-0.03% (Y), -0.04% (U), -0.07% (V), 100.6% EncT, 100.6% DecT for natural content

-0.06% (Y), -0.00% (U), -0.05% (V), 100.8% EncT, 100.5% DecT for class F

-0.09% (Y), -0.02% (U), -0.20% (V), 100.7% EncT, 100.2% DecT for class TGM

# Introduction

Dependent quantization (DQ) [1] is used to quantize the transformed coefficients and the two quantizers of DQ are shown in Figure 1.

Fig. 1 Two quantizers of DQ

The quantization level k from either Q0 or Q1 is coded into the bitstream using regular residual coding. While DQ can be considered a quantizer with a quantization step size ∆, both Q0 and Q1 can be considered as an individual scalar quantizer with a quantization step size 2∆. The quantization index, idxDQ of DQ can be calculated as follows.

idxDQ={2*k-sgnk,    Q1 2*k                  ,    Q0                                                (1)

where k is the quantization level of Q0 or Q1, sgn. denotes the signum function

sgnk=(k==0 ?0:k<0 ? -1:1)                                 (2)

A method of shifting quantization centers [2] was adopted into ECM software where the shifting amount is inversely proportional to the quantization indices of DQ as follows.

- x=1024-TidxDQ*Q-1idxDQ+TidxDQ*Q-1idxDQ'≫10    if  T>idxDQ>0       (3)

where x is the dequantized coefficient,  T is the size of the lookup table, Q-1idxDQ is the dequantized value of the idxDQ-th of DQ,  idxDQ'is the auxiliary quantization index that can be calculated using Eq. (4).

- idxDQ'=idxDQ+(idxDQ>0 ?1:-1)                                                 (4)

A lookup table of 64 T, defined in (5), is used in ECM-12.0.

T=[0,63, 31, 21, 15, 12, 10, 9, 7, 7, 6, 5, 5, 4, 4, 4, 3, 3, 3, 3, 3, 3, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 1,…,1]       (5)

# Proposed method

This contribution first proposes to also apply the quantization center shifting in the transform skip mode. The same table with ECM-12.0 [3] is used.

Since the quantization level k from Q0 or Q1 instead of the quantization index of DQ is actually coded into a bitstream and the value of quantization level k can be used to better reflect the rate, this contribution proposes that the quantization level from Q0 and Q1 instead of quantization indices of DQ is used to derive the shifting amount.

Furthermore, the same quantization level from quantizer Q0 and Q1 will generate a similar rate while the dequantization value of Q1 is smaller than that of Q0 for the same quantization level k. Therefore, this contribution further proposes that Q1 and Q0 may use different sizes of tables. In the real implementation, a lookup table of 32 T0and a lookup table of 48 T1 are used for Q0 and Q1, respectively.

- x=1024-Tik*Qi-1(k)+TQik*Qi-1(k')≫10    if  Ti>k>0                (6)

where i indicates the Q0 or Q1, x is the dequantized coefficient, Ti is the size of the lookup table, Qi-1(k) is the dequantized value of the quantization level k from quantizer Qi, k'is the auxiliary quantization index that can be calculated using Eq. (7).

- k'=k+(k>0?1:-1)                                                          (7)

T0[32]=0, 31, 15, 10, 7, 6, 5, 4, 3, 3, 3, 2, 2, 2, 2, 2, 1,1,1,1,1,1,1,1,1,1, 1, 1, 1, 1, 1, 1                (8)

T148=0, 47, 23,15, 11,9, 7, 6, 5, 5,4,4,3, 3, 3, 3, 2, 2, 2, 2, 2,2,2,2, 1,1,1, 1, 1, 1,1……,1,1,1,1,1 (9)

# Simulation results

The proposed method is implemented on top of the ECM-12.0 and tested based on the common test conditions [4]. The test results are summarized in the following Tables. Table 1 shows the results of also applying the same method of shifting quantization centers proposed in [2] in the transform skip mode.

Table 1. Shifting quantization center for transform skip mode

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.01% | -0.01% | 0.04% | 101.0% | 102.0% | 100.3% | 100.0% |
| Class A2 | -0.01% | -0.03% | -0.01% | 100.5% | 101.1% | 100.0% | 100.0% |
| Class B | -0.01% | 0.03% | 0.03% | 100.7% | 100.5% | 99.3% | 100.0% |
| Class C | 0.00% | 0.01% | -0.02% | 101.7% | 101.8% | 100.3% | 100.0% |
| Class E | 0.01% | -0.10% | -0.07% | 100.9% | 101.3% | 100.0% | 100.0% |
| Overall | 0.00% | -0.01% | 0.00% | 101.0% | 101.3% | 99.9% | 100.0% |
| Class D | -0.03% | 0.19% | 0.04% | 101.5% | 105.5% | 100.0% | 100.0% |
| Class F | -0.05% | 0.11% | -0.04% | 101.2% | 102.5% | 100.0% | 100.0% |
| Class TGM | -0.14% | -0.04% | -0.19% | 100.7% | 102.2% | 100.0% | 100.1% |

Table 2 shows the results of applying the modified method of shifting quantization centers in both RRC and TSRC.

Table 2. Proposed method applied in both TSRC and RRC

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.11% | -0.10% | 100.1% | 100.1% | 99.4% | 100.0% |
| Class A2 | -0.03% | 0.02% | -0.07% | 100.2% | 100.2% | 99.7% | 100.1% |
| Class B | -0.04% | -0.04% | -0.01% | 100.6% | 100.0% | 100.0% | 100.0% |
| Class C | -0.01% | -0.07% | -0.03% | 100.5% | 101.1% | 100.4% | 100.0% |
| Class E | -0.02% | 0.01% | -0.18% | 101.4% | 101.7% | 100.0% | 100.0% |
| Overall | -0.03% | -0.04% | -0.07% | 100.6% | 100.6% | 99.9% | 100.0% |
| Class D | -0.01% | 0.07% | -0.20% | 101.4% | 98.6% | 100.2% | 100.1% |
| Class F | -0.06% | 0.00% | -0.05% | 100.8% | 100.5% | 100.0% | 100.1% |
| Class TGM | -0.09% | -0.02% | -0.20% | 100.7% | 100.2% | 100.0% | 100.0% |

Note: The encoding and decoding time are not accurate.

# Conclusions

This contribution proposes to apply quantization center shifting in the transform skip mode. This contribution further proposes the shifting amount to be inversely proportional to the quantization level of individual quantizers instead of the quantization indices of DQ. Simulation results show a noticeable coding gain for all the classes of test content while no increase in complexity. It is suggested to study the proposed method in the next round of EE.

# References

- H. Schwarz, T. Nguyen, D. Marpe, T. Wiegand, “CE7: Transform coefficient coding and dependent quantization (Tests 7.1.2, 7.2.1)”, JVET-K0071, July, 2018.

- M. Balcilar, K. Naser, F. Galpin, F. Le Léannec, “EE2-4.1: Shifting quantization center”, JVET-AE-125, July, 2023.

- ECM-12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-12.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, April, 2023.

# Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).