| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0069 |
| --- | --- |

| Title: | EE2-3.1: Chained motion vector prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Haruhisa Kato Kei Kawamura (KDDI) 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | yo-kidani@kddi.com |
| Source: | KDDI Corporation (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of Test 3.1 (chained motion vector prediction) in the context of Exploration Experiments 2 (JVET-AG2024).

In Test 3.1, chained motion vector prediction (CMVP) is introduced as a new inter merge candidate list construction. CMVP candidates can be derived as the accumulation of the recursively traced motion vectors (MVs) and block vectors (BVs) based on the pre-derived MVs.

Two tests (Test 3.1a and Test 3.1b) are conducted to study the trade-off between coding gains and additional motion storage. On top of the ECM-12.0, simulation results {Y, U, V, EncT, DecT} of Test 3.1a and Test 3.1b for overall test sequences in CTC are reported below:

Test 3.1a, i.e., chained motion vector prediction:

RA: {-0.15%, -0.25%, -0.29%, 99.8%, 100.5%};

LDB: {-0.36%, -0.73%, -0.51%, 99.7%, 101.8%};

LDP: {-0.28%, -0.49%, -0.06%, 100.1%, 102.4%};

Test 3.1b, i.e., Test 3.1a + temporal buffer constraint limiting the motion trace of CMVP to only come from the collocated pictures:

RA: {-0.14%, -0.20%, -0.27%, 100.0%, 100.6%};

LDB: {-0.23%, -0.51%, -0.31%, 99.8%, 101.3%};

LDP: {-0.23%, -0.57%, -0.25%, 100.2%, 102.0%}.

Introduction

In this test, chained motion vector prediction (CMVP) is introduced as a new inter merge candidate list construction [1]. As shown in Figure 1, CMVP candidates can be derived as the accumulation of the recursively traced motion vectors (MVs) and block vectors (BVs) based on the pre-derived MVs.

Figure 1 An example of derivation of CMVP candidate.

When deriving MVk(m), all five-positions shown in Figure 2 including the center, top-left, top-right, bottom-left, and bottom-right of the current block, are checked to find traced MVs or BVs.

Figure 2 Operation of referencing source and destination of tracing MVs in CMVP.

CMVP candidates are derived for each merge index, each list (i.e., L0 and L1), and each trace depth, and they are inserted after HMVP candidates for the regular merge and TM merge. The number of traced depths to derive CMVPs is restricted to one (i.e., m=1), the same design as auto-relocated block vector prediction (AR-BVP) [2]. The traceable reference pictures are only within the reference picture list.

Test description

Two tests are conducted to study the trade-off between coding gains and additional motion storage (i.e., temporal buffer) as follows:

Test 3.1a: chained motion vector prediction (CMVP);

Test 3.1b: Test 3.1a + temporal buffer constraint limiting the motion trace of CMVP to only come from the collocated pictures.

Experimental results

The proposed methods were implemented on top of ECM-12.0 [3] and tested according to the common test condition of RA, LB, and LP configuration as specified in the JVET-AF2017 [4] and JVET-AG2024 [5].

The experimental results of Test 3.1a and Test 3.1b are summarized below. More detailed results can be found in the attached spreadsheets.

Table 1 Test 3.1a results over ECM-12.0.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.10% | -0.23% | -0.31% | 100.0% | 100.4% | 99.6% | 100.0% |
| Class A2 | -0.12% | -0.17% | -0.30% | 99.7% | 100.8% | 100.1% | 99.9% |
| Class B | -0.19% | -0.33% | -0.31% | 99.7% | 100.4% | 100.0% | 99.9% |
| Class C | -0.15% | -0.22% | -0.25% | 99.8% | 100.5% | 99.6% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.15% | -0.25% | -0.29% | 99.8% | 100.5% | 99.8% | 99.9% |
| Class D | -0.12% | 0.05% | -0.02% | 99.7% | 101.3% | 100.2% | 100.0% |
| Class F | -0.10% | -0.12% | -0.36% | 100.1% | 101.7% | 100.2% | 100.6% |
| Class TGM | -0.26% | -0.31% | -0.32% | 99.8% | 101.8% | 99.8% | 100.2% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.29% | -0.43% | -0.24% | 99.7% | 101.3% | 100.0% | 99.5% |
| Class C | -0.44% | -0.48% | -0.49% | 99.6% | 102.1% | 100.0% | 100.0% |
| Class E | -0.37% | -1.57% | -1.00% | 99.9% | 102.2% | 100.2% | 100.1% |
| Overall | -0.36% | -0.73% | -0.51% | 99.7% | 101.8% | 100.0% | 99.8% |
| Class D | -0.58% | 0.19% | -0.79% | 99.3% | 102.2% | 99.9% | 100.1% |
| Class F | -0.36% | -1.05% | -1.45% | 100.0% | 101.8% | 97.6% | 99.8% |
| Class TGM | -0.64% | -0.71% | -0.76% | 99.9% | 102.7% | 100.6% | 101.4% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.19% | -0.24% | 0.11% | 100.2% | 101.4% | 100.0% | 99.9% |
| Class C | -0.35% | -0.52% | -0.09% | 99.9% | 103.2% | 100.1% | 100.2% |
| Class E | -0.35% | -0.88% | -0.31% | 100.0% | 103.1% | 100.0% | 100.2% |
| Overall | -0.28% | -0.49% | -0.06% | 100.1% | 102.4% | 100.0% | 100.0% |
| Class D | -0.37% | -1.28% | 0.47% | 99.5% | 101.4% | 100.4% | 100.0% |
| Class F | -0.50% | -0.25% | 0.18% | 100.0% | 102.9% | 100.3% | 100.1% |
| Class TGM | -0.08% | -0.18% | -0.22% | 100.0% | 105.3% | 99.6% | 100.2% |

Table 2 Test 3.1b results over ECM-12.0.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.11% | -0.16% | -0.25% | 100.2% | 100.5% | 99.8% | 100.2% |
| Class A2 | -0.12% | -0.19% | -0.26% | 99.8% | 100.7% | 99.8% | 99.9% |
| Class B | -0.17% | -0.23% | -0.34% | 100.0% | 100.5% | 100.0% | 100.0% |
| Class C | -0.13% | -0.20% | -0.22% | 99.9% | 100.7% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.14% | -0.20% | -0.27% | 100.0% | 100.6% | 99.9% | 100.0% |
| Class D | -0.12% | -0.01% | 0.13% | 100.0% | 101.3% | 100.0% | 100.0% |
| Class F | -0.09% | -0.07% | -0.25% | 100.0% | 101.6% | 100.2% | 100.1% |
| Class TGM | -0.18% | -0.19% | -0.21% | 100.1% | 101.4% | 99.9% | 99.7% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.21% | -0.27% | 0.13% | 100.1% | 100.8% | 99.9% | 99.9% |
| Class C | -0.27% | -0.64% | -0.33% | 99.7% | 101.4% | 100.0% | 100.5% |
| Class E | -0.19% | -0.72% | -1.04% | 99.3% | 101.9% | 100.7% | 100.2% |
| Overall | -0.23% | -0.51% | -0.31% | 99.8% | 101.3% | 100.1% | 100.2% |
| Class D | -0.43% | 0.19% | -0.30% | 99.6% | 101.5% | 100.4% | 100.1% |
| Class F | -0.19% | -0.79% | -0.74% | 100.1% | 101.4% | 100.3% | 99.5% |
| Class TGM | -0.38% | -0.36% | -0.33% | 100.3% | 103.1% | 100.0% | 100.4% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.23% | -0.41% | -0.05% | 100.3% | 101.2% | 100.0% | 99.9% |
| Class C | -0.31% | -0.57% | -0.41% | 99.8% | 102.6% | 100.0% | 100.1% |
| Class E | -0.14% | -0.83% | -0.36% | 100.6% | 102.7% | 100.0% | 100.1% |
| Overall | -0.23% | -0.57% | -0.25% | 100.2% | 102.0% | 100.0% | 100.0% |
| Class D | -0.44% | -1.50% | -0.70% | 99.5% | 101.8% | 100.4% | 100.0% |
| Class F | -0.40% | -0.70% | -0.64% | 100.1% | 101.9% | 100.2% | 100.1% |
| Class TGM | -0.07% | -0.17% | -0.14% | 99.8% | 103.3% | 99.6% | 100.4% |

Conclusion

This contribution proposes a chained motion vector prediction. The proposed method brings some coding gains with increasing minor decoder runtime. It is recommended to adopt the proposed method into the next ECM.

Reference

- Y. Kidani, H. Kato, and K. Kawamura, “Non-EE2: Chained motion vector prediction,” JVET-AG0073, January 2024.

- N. Zhang, K. Zhang, and L. Zhang, “EE2-1.8: Auto-relocated block vector prediction,” JVET-AG0091, teleconference, January 2024.

- ECM-12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-12.0

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, October. 2023.

- V. Seregin, et al., “Exploration experiment on enhanced compression beyond VVC capability (EE2)”, JVET-AG2024, January 2024.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).