| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0078-v1 |
| --- | --- |

| Title: | AhG10: Distortion Propagation Factor for VVC Low-Delay Configuration |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongwei Guo, Ce Zhu, Junjie Chen, Lei Luo (UESTC) Yongkai Huo, Yutian Liu (Transsion) | Email: | hwguo@uestc.edu.cn eczhu@uestc.edu.cn 202221010107@std.uestc.edu.cn luolei1010@uestc.edu.cn yongkai.huo@transsion.com yutian.liu@transsion.com |
| Source: | University of Electronic Science and Technology of China, Transsion Holdings |  |  |

_____________________________

- Abstract

This contribution presents an updated version of the method proposed in contributions JVET-AF0089 and JVET-AG0055.

The proposed method involves CTU-level Lagrange multiplier and QP adaptation for VVC low-delay configuration by estimating the distortion propagation factor (DPF) through pre-encoding. As an update, this contribution introduces the distortion propagation length as a configurable parameter, and the propagation length of key frames is configured separately from non-key frames. If the propagation length is set to 0, the pre-encoding of the frame is skipped to reduce the time complexity.

The proposed method was implemented on VTM-23.0. Compared to the original VTM-23.0, the recorded overall PSNR-Y, U, V BD-rate and {EncT, DecT} variations in low-delay B and low-delay P are reported below, respectively.

Over VTM-23.0, all frames pre-encoded:

LDB: -4.09%, -4.13%, -3.40%  {104%, 100%}

LDP: -4.09%, -3.87%, -2.75%  {104%, 100%}

Over VTM-23.0, only key frames pre-encoded:

LDB: -3.78%, -3.09%, -2.21%  {101%, 100%}

LDP: -3.72%, -2.96%, -1.83%  {102%, 100%}

It is recommended to adopt this proposed scheme into VTM under low-delay configuration.

Introduction

This contribution presents an updated version of the method proposed in contributions JVET-AF0089 [3] and JVET-AG0055 [4].

VVC employs a hierarchical QP setting at frame-level within a GOP. Compared to a flat QP setting scheme, the hierarchical QP setting achieves better performance by enhancing the coding quality of the frequently referenced key frames. Similarly, we propose an optional scheme for CTU-level Lagrange multiplier and QP adaptation, aiming at enriching the coding quality of frequently referenced regions within a frame.

The proposed method involves CTU-level Lagrange multiplier and QP adaptation for VVC low-delay configuration. Firstly, we estimate a distortion propagation factor (DPF) for each 32×32 block in the frame through pre-encoding. Then, a weight coefficient of each CTU is calculated based on the obtained DPFs. Finally, the adaptive Lagrange multiplier and QP for each CTU are determined based on the weight coefficient and frame-level Lagrange multiplier.

Proposal

Pre-encoding

The frame to be encoded is first pre-encoded with a 32×32 block integer pixel inter-prediction. The purpose of pre-encoding is to roughly assess the coding distortion of a block and its motion compensation predicted (MCP) error, which will be used to calculate the DPF of a block. Note that the QP adopted for pre-encoding is set as the highest temporal level QP, and the pre-encoding does not perform in-loop filter or sample adaptive offset.

The pre-encoding process can be implemented in the function "EncSlice::precompressSlice( )". Firstly, a pre-coding configuration is performed, including setting the QP and fixing split mode and depth. Then, "EncSlice::compressSlice( )" is called to pre-encode the frame to obtain the coding distortion of a block and its MCP error. Finally, the DPF calculation and CTU-level Lagrange multiplier and QP adaptation are performed, and the pre-encoding process is terminated to return to the full encoding process.

Distortion Propagation Factor

The distortion propagation factor (DPF) of each coding block is calculated by:

ϕi=k=1LiDiEik                                    (1)

where Di and Ei are the coding distortion of a block and its MCP error, both of which are quantified using Mean Squared Error (MSE), and Li is the configurable distortion propagation length.

In our configuration, the distortion propagation length of key frames (POC is a multiple of 8) is set to 8, while that of non-key frames is set to 0 or 1. When the distortion propagation length is set to 0, the frame does not undergo CTU-level Lagrange multiplier and QP adaptation, so the pre-encoding of the frame can be skipped.

Lagrange multiplier and QP adaptation

Based on the DPFs obtained, the weight coefficient of each CTU is calculated by:

ωio=11+ϕi                                         (2)

ωaverage=1Mframei=1Mframeωio                               (3)

ωi=ωioωaverage                                       (4)

ψj=1Mji=1Mjωi                                    (5)

where ϕi is the DPF of a block, Mframe is the number of blocks in a frame, and Mj is the number of blocks in the jth CTU.

Finally, the Lagrange multiplier and QP of a CTU are determined by:

λj=ψj⋅λg                                         (6)

QPj=4.3281×logλj+2.1829                       (7)

where λj, ψj and QPj are the Lagrange multiplier, weight coefficient and QP of jth CTU in the frame to be encoded, and λg is the frame-level Lagrange multiplier.

Comparison with BIM

Block importance mapping (BIM) was proposed in contributions JVET-V0057 [5] and JVET-Y0077 [6]. It selects individual QP values for each CTU based on its importance to future pictures. Here is a comparison between our method and BIM.

Similarity:

Function: Both methods aim for CTU-level QP adaptation.

Weights determination: Both are based on motion compensation to calculate the weights of different CTUs.

Difference:

Estimation avenue: BIM first determines the delta QP by calculating the motion compensation prediction error and then adjusts the Lagrange multiplier accordingly. Our method, on the other hand, starts with a defined calculation method for the Lagrange multiplier and then adjusts QP accordingly, offering finer granularity in adjusting the Lagrange multiplier.

Motion compensation algorithm: BIM derives its motion compensation through a Motion Compensation Temporal Filter (MCTF), while our method uses pre-encoding.

Modified Frames: Due to BIM’s reliance on MCTF, QP is only modified for frames where the temporal filter is active, i.e., when the Picture Order Count (POC) is divisible by eight, including I-frames. In contrast, our method’s pre-encoding approach allows modification for every frame except I-frames.

In summary, while both methods share a similar purpose and computational basis, they differ significantly in their specific schemes and algorithms, leading to different implementation effects. Therefore, it is appropriate to regard these two methods as independent QP adaptation approaches.

Results

Performance: all frames pre-encoded

The distortion propagation length of key frames is referred to as DPFKeyLength, while that of non-key frames is referred to as DPFNonkeyLength. In this configuration, DPFKeyLength is set to 8, while DPFNonkeyLength is set to 1. All frames are pre-encoded.

Based on VTM-23.0 [1], coding performance experiment was conducted following the common test conditions defined in [2]. For low-delay B configuration, experimental results are shown in Table 1. The proposed method can obtain -4.09%, -4.13%, and -3.40% BD-rate on average for Y, Cb, and Cr components, respectively.

Table 1. Performance over VTM-23.0, LDB, DPFKeyLength=8, DPFNonkeyLength=1

|  |  | Low delay B Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-23.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -3.72% | -2.63% | -2.97% | 104% | 100% |
| Class C | -5.21% | -7.14% | -4.92% | 104% | 99% |
| Class E | -3.22% | -2.59% | -2.08% | 106% | 100% |
| Overall | -4.09% | -4.13% | -3.40% | 104% | 100% |
| Class D | -1.37% | -1.33% | -1.01% | 101% | 99% |
| Class F | -5.68% | -6.36% | -4.60% | 106% | 100% |

For low-delay P configuration, experimental results are shown in Table 2. The proposed method can obtain -4.09%, -3.87%, and -2.75% BD-rate on average for Y, Cb, and Cr components, respectively.

Table 2. Performance over VTM-23.0, LDP, DPFKeyLength=8, DPFNonkeyLength=1

|  |  | Low delay P Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-23.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -3.56% | -2.12% | -1.92% | 104% | 100% |
| Class C | -5.36% | -6.89% | -4.58% | 103% | 99% |
| Class E | -3.29% | -2.74% | -1.69% | 106% | 100% |
| Overall | -4.09% | -3.87% | -2.75% | 104% | 100% |
| Class D | -1.52% | -2.22% | -0.16% | 101% | 99% |
| Class F | -6.71% | -7.60% | -5.47% | 105% | 100% |

The proposed method results in a significant performance gain, despite a slight increase in encoding time due to pre-encoding.

Performance: only key frames pre-encoded

In this configuration, DPFKeyLength is set to 8, while DPFNonkeyLength is set to 0. Only key frames are pre-encoded.

For low-delay B configuration, experimental results are shown in Table 3. The proposed method can obtain -3.78%, -3.09%, and -2.21% BD-rate on average for Y, Cb, and Cr components, respectively.

Table 3. Performance over VTM-23.0, LDB, DPFKeyLength=8, DPFNonkeyLength=0

|  |  | Low delay B Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-23.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -3.50% | -1.26% | -1.24% | 102% | 101% |
| Class C | -4.88% | -5.99% | -4.01% | 101% | 100% |
| Class E | -2.79% | -2.27% | -1.43% | 101% | 100% |
| Overall | -3.78% | -3.09% | -2.21% | 101% | 100% |
| Class D | -1.33% | -1.78% | -0.59% | 100% | 99% |
| Class F | -5.89% | -6.10% | -4.53% | 101% | 100% |

For low-delay P configuration, experimental results are shown in Table 4. The proposed method can obtain -3.72%, -2.96%, and -1.83% BD-rate on average for Y, Cb, and Cr components, respectively.

Table 4. Performance over VTM-23.0, LDP, DPFKeyLength=8, DPFNonkeyLength=0

|  |  | Low delay P Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-23.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -3.32% | -1.36% | -1.02% | 102% | 101% |
| Class C | -5.05% | -6.09% | -4.06% | 102% | 100% |
| Class E | -2.61% | -1.47% | -0.21% | 101% | 100% |
| Overall | -3.72% | -2.96% | -1.83% | 102% | 100% |
| Class D | -1.31% | -1.58% | 0.45% | 101% | 99% |
| Class F | -6.85% | -7.36% | -5.34% | 102% | 99% |

Although there is a slight performance decrease compared to pre-encoding all frames, the method still achieves a significant performance gain. Moreover, the time complexity is lower in this configuration with an almost negligible increase in encoding time.

Performance of larger distortion propagation length

As a comparison, we conducted the experiment with DPFKeyLength=12 for providing further insights on the distortion propagation effects of key frames. The results are shown in Table 5, 6, 7 and 8.

Table 5. Performance over VTM-23.0, LDB, DPFKeyLength=12, DPFNonkeyLength=1

|  |  | Low delay B Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-23.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -3.49% | -3.11% | -2.48% | 104% | 100% |
| Class C | -5.94% | -8.96% | -6.28% | 101% | 99% |
| Class E | -4.01% | -3.06% | -2.30% | 107% | 100% |
| Overall | -4.44% | -5.05% | -3.70% | 104% | 100% |
| Class D | -1.50% | -2.72% | -1.17% | 103% | 100% |
| Class F | -6.55% | -8.29% | -5.98% | 106% | 101% |

Table 6. Performance over VTM-23.0, LDP, DPFKeyLength=12, DPFNonkeyLength=1

|  |  | Low delay P Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-23.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -3.25% | -2.44% | -2.38% | 104% | 100% |
| Class C | -6.01% | -8.62% | -5.70% | 101% | 101% |
| Class E | -3.87% | -3.72% | -1.49% | 107% | 98% |
| Overall | -4.32% | -4.82% | -3.27% | 104% | 100% |
| Class D | -1.54% | -2.49% | -1.28% | 102% | 102% |
| Class F | -7.69% | -9.04% | -6.59% | 105% | 101% |

Table 7. Performance over VTM-23.0, LDB, DPFKeyLength=12, DPFNonkeyLength=0

|  |  | Low delay B Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-23.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -3.44% | -1.78% | -1.58% | 102% | 101% |
| Class C | -5.73% | -7.73% | -5.27% | 101% | 97% |
| Class E | -3.63% | -2.32% | -2.28% | 101% | 96% |
| Overall | -4.25% | -3.90% | -2.99% | 101% | 98% |
| Class D | -1.40% | -1.92% | -1.64% | 100% | 100% |
| Class F | -6.82% | -8.46% | -6.18% | 102% | 101% |

Table 8. Performance over VTM-23.0, LDP, DPFKeyLength=12, DPFNonkeyLength=0

|  |  | Low delay P Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-23.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -3.17% | -2.12% | -1.51% | 102% | 101% |
| Class C | -5.73% | -7.83% | -5.34% | 101% | 100% |
| Class E | -3.55% | -2.16% | -0.57% | 101% | 98% |
| Overall | -4.12% | -4.03% | -2.55% | 101% | 100% |
| Class D | -1.41% | -2.12% | -0.69% | 101% | 103% |
| Class F | -7.69% | -9.18% | -6.85% | 102% | 100% |

It can be observed that a larger distortion propagation length leads to better performance in general. However, for some specific sequences, such as those with scene switching, a larger distortion propagation length results in performance loss. In summary, setting the default distortion propagation length to 8 is a stable choice.

Performance of BIM

The performance of BIM over VTM-23.0 is shown in Table 9 and 10. From Tables 1 to 10, it can be observed that our method slightly increases the encoding time but achieves a more noticeable BD-rate improvement.

Table 9. Performance of BIM over VTM-23.0 (LDB)

|  |  | Low delay B Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-23.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -3.21% | -1.35% | -1.65% | 100% | 100% |
| Class C | -3.22% | -0.07% | 0.42% | 99% | 99% |
| Class E | 0.15% | 2.50% | 0.49% | 102% | 101% |
| Overall | -2.37% | 0.04% | -0.42% | 100% | 100% |
| Class D | -1.57% | 1.41% | 1.37% | 100% | 99% |
| Class F | -2.71% | -2.36% | -0.99% | 101% | 100% |

Table 10. Performance of BIM over VTM-23.0 (LDP)

|  |  | Low delay P Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-23.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.90% | -1.42% | -1.50% | 99% | 100% |
| Class C | -2.94% | -0.03% | 0.43% | 100% | 100% |
| Class E | -0.41% | 1.65% | 0.83% | 101% | 101% |
| Overall | -2.29% | -0.19% | -0.28% | 100% | 100% |
| Class D | -1.38% | 1.15% | 1.73% | 100% | 100% |
| Class F | -2.90% | -2.27% | -1.10% | 101% | 100% |

Subjective Results

A full subjective video quality test was conducted and no compression artifact was observed. Figure 1 to 6 exemplifies visual comparison of frames with different configurations.

Figure 1 - Images of the subjective evaluation (BasketballDrill.Q22.jvet10.lb 220)

Figure 2 - Images of the subjective evaluation (RitualDance.Q27.jvet10 360)

Figure 3 - Images of the subjective evaluation (FourPeople.Q32.jvet10.lp 450)

Figure 4 - Images of the subjective evaluation (SlideShow.Q37.jvet10.lp 250)

Figure 5 - Images of the subjective evaluation (BlowingBubbles.Q22.jvet10.lp 180)

Figure 6 - Images of the subjective evaluation (BQTerrace.Q37.jvet10.lb 200)

Conclusion

It has been shown that CTU-level Lagrange multiplier and QP adaptation for VVC low-delay configuration by estimating the DPF through pre-encoding can provide coding performance benefits compared to the original scheme. The encoding time of the proposed method is slightly increased. The pre-encoding of non-key frames can be skipped to reduce the time complexity. It is recommended to adopt this proposed scheme into VTM under low-delay configuration.

References

- VTM-23.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/releases/VTM-23.0

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Meeting by Teleconference, October 2020.

- H. Guo, C. Zhu, L. Luo, et al, “AhG10: Lagrange multiplier and QP adaptation at CTU-level for VVC”, JVET-AF0089, Hannover Meeting, October 2023.

- H. Guo, C. Zhu, L. Luo, et al, “AhG10: CTU-Level Lagrange Multiplier and QP Adaptation for VVC Low-Delay Configuration”, JVET-AG0055, Teleconference Meeting, January 2024.

- P. Wennersten, C. Hollmann, J. Ström, “[AHG10] Block importance mapping”, JVET-V0057, Meeting by Teleconference, April 2021.

- P. Wennersten, J. Enhorn, C. Hollmann, J. Ström, “AHG10: Block importance mapping”, JVET-Y0077, Meeting by Teleconference, January 2022.

Patent rights declaration(s)

University of Electronic Science and Technology of China may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Shenzhen Transsion Holdings Co., Ltd. (Transsion) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).