| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0080-v1 |
| --- | --- |

| Title: | EE1-1.2: Joint LOP model with inputs transformed |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Du Liu, Jacob Ström, Mitra Damghanian, Per Wennersten (Ericsson) Dmytro Rusanovskyy, Yun Li, Marta Karczewicz (Qualcomm) Tong Shao, Peng Yin, Sean McCarthy (Dolby) Jay N. Shingala, Ajay Shyam, Ajat Suneja, Siddarth P. Badya (Ittiam) | Email: | {du.liu, jacob.strom, mitra.damghanian, per.wennersten}@ericsson.com {dmytror, yli30, martak}@qti.qualcomm.com {tong.shao, pyin}@dolby.com {jay.shingala, ajayshyam}@ittiam.com |
| Source: | Ericsson, Qualcomm, Dolby, Ittiam |  |  |

_____________________________

# Abstract

This contribution reports the results of EE1-1.2, which uses DCT transformed inputs on top of EE1-1.1. The method applies the DCT to the inputs of the model and the inverse DCT to the outputs. The spatial resolution inside the model is reduced by a factor of four, and the reduced complexity is balanced by increasing the number of channels. The proposed model has a complexity of 16.9 kMAC/pixel and 0.2M parameters, while the model from EE1-1.1 has 17.0 kMAC/pixel and 0.05M parameters. Stage 3 training of the proposed model is performed. The performance of the int16 model (including NNIntra) compared to the NNVC-8.0 anchor (NNIntra+LOP) is reported to be:

RA {-0.45%, 0.07%, 1.08%}, AI {-0.29%, -1.09%, -0.50%}, LDB {0.07%, 3.66%, 7.12%}

It is proposed to adopt the method into NNVC.

# Introduction

The current LOP.2 model has been studied and trained in JVET-AF0043 [1]. At the last meeting, a method using transformed inputs for LOP was proposed [2]. It was proposed to apply a 2x2 DCT-II transform and reshaping to the inputs and apply inverse reshaping and an inverse transform to the outputs.

Fig. 1 shows an example of applying the DCT transform to the reconstruction samples and reshaping. Fig. 1(a) shows the current dimension for the reconstruction input in LOP, and Fig. 1(b) shows the placement of the DCT and the dimension of the input. In Fig. 1(b), the luma reconstruction input is transformed for each 2x2 subblocks, and each 2x2 subblock is reshaped into four channels corresponding to the four DCT coefficients. The input luma of size 144x144x1 is therefore transformed and reshaped into 72x72x4, and 4 represents four DCT frequency channels. For chroma reconstruction samples, because of upsampling of 2, each 2x2 subblock is constant and there is no need to apply transform. Thus, it can be directly downsampled by 2 to get to 72x72x2. The upsampling of 2 in chroma is kept to retain compatibility with the current training, due to that in the training data, the chroma data is already upsampled and saved together with luma. In a more efficient implementation, both upsampling and downsampling can be skipped with identical results.

(a) Current dimension for the reconstruction input in LOP.

(b) The proposed placement of DCT-II and the input dimension.

Fig. 1. An example of applying the transform and reshaping to the reconstruction samples.

Similarly, the inputs “Pred” and “BS” follow the same transform and reshaping and obtain a dimension of 72x72x6. For “QPbase” and “QPslice”, since they are constant, there is no need to apply transform and they are directly reshaped to 72x72x1. For “IPB”, as there is only luma information, it is transformed and reshaped into 72x72x4.

Since the spatial dimension going through the network is only a quarter of the current LOP, the complexity in terms of kMAC/pixel is reduced to about a quarter as well. This is because the spatial resolution inside the network is reduced by a factor of four (so kMAC goes down) while the final output still has the same number of pixels (the denominator stays the same). The complexity is not exactly a quarter due to that the input channels have a slightly increased number to accommodate the number of DCT frequency channels. To further adjust the network to meet the current LOP complexity, the number of channels and the number of backbone blocks for luma and chroma need to be modified.

# Proposed method

In EE1-1.1 [3], a combined method using low complexity transition/fusion blocks from JVET-AG0155 [4] and a new backbone structure from JVET-AG0163 [5] is tested. In this EE1-1.2, the method using transformed inputs is tested on top of EE1-1.1. The model architecture is shown in Fig. 2.

Fig. 2. The architecture of EE1-1.2, a combination of JVET-AG0155, AG0163, and JVET-AG0069.

As shown in Fig. 2, the DCT is applied to the inputs and inverse DCT is applied before cropping the outputs. The rest of the architecture changes follow EE1-1.1. The proposed modifications of model parameters are given below:

Head

- d1: 12  16

- d2: 8

- d3: 4

- d4: 2

- d5: 2

- C (=d6): 24  64

Backbone

- NY: 14

- NUV: 4

- CY: 16  32

- CUV: 16  32

- CY1: 64  144

- CUV1:  32  128

- C21: 16  32

This results in a complexity of 16.9 kMAC/pixel and 205108 parameters for the combined model.

The current LOP.2 model from JVET-AF0043 [1] has a complexity of 17.1 kMAC/pixel and the number of parameters is 0.05M. The model from EE1-1.1 has a complexity of 17.0 kMAC/pixel and the number of parameters is 0.05M.

# Training

Training of the model is performed only for stage 3. Training follows EE1-1.0, where the training dataset uses the LOP stage 3 dataset, luma:chroma weighting 12:2, batch size 64, and milestones 41, 51, 55. The only difference is the learning rate. The suggested learning rate in EE1-1.0 is 8e-4. However, it is observed that learning rate 8e-4 is too large to train the proposed model as the training does not converge. Therefore, a smaller learning rate 2e-4 is used in this training.

Fig. 3. Validation loss in log scale for the learning rates 8e-4 and 2e-4. Zoomed in for visibility.
Blue: run 1 using 8e-4, black: run 2 using 8e-4, magenta: using 2e-4.

Fig. 3 shows the validation loss for the learning rates 8e-4 and 2e-4. Two runs using 8e-4 do not show learning improvement, while using 2e-4 shows reasonable performance improvement.

The training information is shown in Table 1.

| Table 1. Network Information for the model in EE1-1.2 in training. |  |  |
| --- | --- | --- |
| Network Information in Training Stage |  |  |
| Mandatory | GPU Type | GPU: NVIDIA A100-PCIE-40GB |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | Stage3: 60 |
|  | Batch size: | Stage3: 64 |
|  | Loss function: | L1 and MSE |
|  | Training time: | Stage3: 7.5 days |
|  | Training data information: | DIV2K, BVI-DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | The LOP stage 3 dataset |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | Stage3: 2e-4, milestones [41, 51, 55] |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

Fig. 4 shows the GPU memory usage for training EE1-1.1 and EE1-1.2, both using the same batch size 64 under the same environment. It can be observed that the memory consumption of EE1-1.2 is 63% (=5260/8314) of EE1-1.1. Although the number of parameters in EE1-1.2 is quadrupled, the spatial size of samples processed by the network is a quarter by using the transformed inputs, and this reduces the memory usage during training.

(a) GPU memory usage during training for EE1-1.1.

(b) GPU memory usage during training for EE1-1.2.

Fig. 4. GPU memory usage for EE1-1.1 and EE1-1.2.

# Inference

| Table 2. Network Information for the input-transformed model in inference. |  |  |
| --- | --- | --- |
| Network Information in Inference Stage |  |  |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 205108 |
|  | Parameter Precision (Bits) | fp32, int16 |
|  | Memory Parameter (MB) | fp32: 0.4MB, int16: 0.2MB |
|  | Multiplay Accumulate (MAC) | 16.9 kMAC/pixel |
| Optional |  |  |
|  | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128x128, 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

The number of parameters and complexity is consistent between the excel sheet attached in the contribution and the thop library [6].

# Results

The proposed model is tested on top of NNVC-8.0. The changes include the NN model and the corresponding transformed inputs and outputs. The parameter selection and residue scaling are kept unchanged. The results are in comparison to the NNVC CTC [7].

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 anchor (NNIntra+LOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.96% | -1.76% | -1.90% | 97% | 94% |
| Class A2 | -0.80% | 0.46% | 1.78% | 100% | 100% |
| Class B | -0.35% | -0.21% | 0.84% | 97% | 96% |
| Class C | 0.07% | 1.49% | 3.08% | 106% | 91% |
| Class E |  |  |  |  |  |
| Overall | -0.45% | 0.07% | 1.08% | 100% | 95% |
| Class D | 0.59% | 2.84% | 4.47% | 105% | 91% |
| Class F | -0.08% | 2.26% | 3.86% | 104% | 101% |
|  | All Intra |  |  |  |  |
|  | BD-rate Over NNVC-8.0 anchor (NNIntra+LOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.68% | -1.78% | -2.99% | 96% | 94% |
| Class A2 | -0.42% | -1.01% | 0.74% | 96% | 100% |
| Class B | -0.29% | -1.22% | -0.90% | 119% | 122% |
| Class C | 0.09% | 0.39% | 1.59% | 128% | 131% |
| Class E | -0.28% | -2.24% | -1.38% | 104% | 113% |
| Overall | -0.29% | -1.09% | -0.50% | 110% | 113% |
| Class D | 0.43% | 1.23% | 3.33% | 107% | 120% |
| Class F | 0.08% | 1.52% | 1.99% | 105% | 107% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 anchor (NNIntra+LOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.20% | 4.11% | 6.06% | 75% | 64% |
| Class C | 0.24% | 2.49% | 7.36% | 67% | 57% |
| Class E | 0.28% | 4.45% | 8.57% | 84% | 69% |
| Overall | 0.07% | 3.66% | 7.12% | 74% | 63% |
| Class D | 0.58% | 3.40% | 8.60% | 94% | 76% |
| Class F | 0.04% | 2.85% | 7.71% | 71% | 62% |

The encoding time and decoding time is not reliable since they are not run on the same machine.

The same model but trained from scratch is reported in EE1-1.3 in JVET-AH0081[8], which shows an additional improvement of {-0.26%, -1.03%, -1.15%} for RA.

# Conclusions

This contribution reports the study of EE1-1.2, which uses transformed inputs on top of EE1-1.1. The complexity of the model is 16.9 kMAC/pixel which is similar to LOP.2 and EE1-1.1, and the number of parameters is increased from 0.05M to 0.2M. The performance compared to NNVC-8.0 anchor shows a BD-rate impact of {-0.45%, 0.07%, 1.08%} for RA and {-0.29%, -1.09%, -0.05%} for AI with the int16 model. It is proposed to adopt the method into NNVC.

# References

[1] AhG11/EE1: Status of the joint EE1-0 (LOP.2) training, JVET-AF0043, D. Rusanovskyy, Y.Li, M. Karczewicz, T. Shao, P. Yin, S. McCarthy, J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya, J. Li, Y. Li, C. Lin, K. Zhang, L. Zhang, R. Chang, L. Wang, X. Xu, S. Liu, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 32nd Meeting, Hannover, DE, 13–20 October 2023.

[2] AhG11: LOP with inputs transformed, JVET-AG0069, D. Liu, J. Ström, M. Damghanian, P. Wennersten, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 33rd Meeting, by teleconference, 17–26 January 2024.

[3] EE1-1.1: LOP candidate with Low Complexity TB and new BB structure, JVET-AH0050, D. Rusanovskyy, Y. Li, M. Karczewicz, T. Shao, P. Yin, S. McCarthy, J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024.

[4] EE1-Related: On Low Complexity Operational Point for In-Loop Filtering, JVET-AG0155, D. Rusanovskyy, Y. Li, M. Karczewicz, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 33rd Meeting, by teleconference, 17–26 January 2024.

[5] EE1-2.3: Further complexity reduction on the joint LOP.2, JVET-AG0163,	T. Shao, P. Yin, S. McCarthy, J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 33rd Meeting, by teleconference, 17–26 January 2024.

[6] https://pypi.org/project/thop/

[7] Common Test Conditions and evaluation procedures for neural network-based video coding technology, E. Alshina, R.-L. Liao, S. Liu, A. Segall, JVET-AF2016, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 32nd Meeting, Hannover, DE, 13–20 October 2023.

[8] EE1-1.3: Training from scratch for the joint LOP model with inputs transformed, JVET-AH0081, D. Rusanovskyy, Y. Li, M. Karczewicz, T. Shao, P. Yin, S. McCarthy, J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Dolby may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ittiam may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).