| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0085-v1 |
| --- | --- |

| Title: | EE2-6.2 Entropy coding extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Galpin, F. Lo Bianco C. Salmon-Legagneur M. Balcilar | Tel: Email: | franck.galpin@interdigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

In contribution JVET-AG0108, it was proposes to add more flexibility to the bins coding in current ECM. First, some bins coded in EP are transformed to be coded with an associated context. Second, an additional coding type is introduced in between the CABAC and the EP coding where the encoding uses a fixed probability without adaptation. The method is tested on some bins which were coded in EP, as well as together with the retraining of the original CABAC bins. Two different training set were used to show the sensitivity to the training set.  It is reported that on top of ECM-12.0, the overall coding performance impact for {Y, U, V} is {-0.03% -0.06% -0.01%} { -0.14% -0.19% -0.24%} in AI and RA configurations respectively when using dataset from CTC sequences together with original CABAC retraining. Using BVI dataset from AhG11 recommended datasets, the overall coding performance impact for {Y, U, V} is {-0.02%	-0.08%	0.02%} {xxx} in AI and RA configurations respectively.

# Introduction

In current ECM, slice data can be coded either using CABAC coding, where a context is associated to the bin to encode and EP coding.

In some cases, the fixed equa-probability of EP coding might not be optimal. It is proposed to add a mode where the probability depends on the syntax element to encode (Non Equi-Probable).

In order to choose when to use this mode, EP coded bins are changed in order to get an associated context. The change does not impact the encoding and decoding of current ECM-12.0 but allows to alter choose which bins can use CABAC, EP or NEP.

# Simulations

In both tests, full encoding in AI, RA, LDB and LDP configurations using the 4 CTCs QPs is used.

Note: Enc/Dec timing are not reliable but complexity is not expected to increase.

## Test 6.2a – using CTC sequences

All the CTC sequences including F and TGM are used.

### 6.2a CABAC and EP contexts

Test retrained EP bins and original CABAC coded bins. EP bins can choose between EP and NEP models and CABAC bins can choose between CABAC, NEP or EP models.

The following statistics is obtained:

| Model | Before | After |
| --- | --- | --- |
| CABAC | 4770 | 4667 |
| NEP | 0 | 923 |
| EP | 2043 | 1223 |

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | 0.00% | 0.02% | 101.6% | 99.3% | 99.1% | 101.6% |
| Class A2 | -0.04% | -0.05% | -0.06% | 101.9% | 98.6% | 101.4% | 102.9% |
| Class B | -0.03% | -0.03% | 0.08% | 101.0% | 101.7% | 97.4% | 101.0% |
| Class C | -0.03% | -0.17% | -0.10% | 101.6% | 104.3% | 100.4% | 100.2% |
| Class E | -0.03% | -0.04% | -0.03% | 102.1% | 104.7% | 103.4% | 100.6% |
| Overall | -0.03% | -0.06% | -0.01% | 101.5% | 101.8% | 100.0% | 101.2% |
| Class D | -0.07% | 0.05% | -0.40% | 99.1% | 100.2% | 100.0% | 100.0% |
| Class F | -0.09% | -0.19% | -0.20% | 99.9% | 99.8% | 102.4% | 100.4% |
| Class TGM | -0.26% | -0.16% | -0.32% | 101.5% | 102.4% | 101.7% | 100.7% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.12% | -0.01% | -0.08% | 100.6% | 102.0% | 100.3% | 100.0% |
| Class A2 | -0.03% | -0.16% | -0.27% | 101.1% | 101.4% | 100.1% | 99.9% |
| Class B | -0.22% | -0.16% | -0.49% | 96.7% | 97.3% | 100.1% | 100.0% |
| Class C | -0.13% | -0.37% | -0.04% | 97.7% | 97.9% | 100.0% | 99.8% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.14% | -0.19% | -0.24% | 98.6% | 99.2% | 100.1% | 99.9% |
| Class D | -0.19% | -0.32% | -0.24% | 96.5% | 96.1% | 100.3% | 100.0% |
| Class F | -0.13% | -0.42% | -0.28% | 96.0% | 92.1% | 99.9% | 100.2% |
| Class TGM | -0.59% | -0.63% | -0.50% | 99.1% | 100.4% | 100.1% | 100.1% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.09% | -0.57% | -0.36% | 99.5% | 98.3% | 99.5% | 100.9% |
| Class C | -0.10% | -0.29% | -0.54% | 94.4% | 94.4% | 100.0% | 100.3% |
| Class E | -0.02% | 0.43% | -0.24% | 92.9% | 92.4% | 100.3% | 99.5% |
| Overall | -0.08% | -0.23% | -0.39% | 96.1% | 95.5% | 99.9% | 100.4% |
| Class D | -0.21% | 0.04% | -0.22% | 92.1% | 94.3% | 99.9% | 100.1% |
| Class F | -0.09% | -0.45% | -0.45% | 96.1% | 96.1% | 100.4% | 101.5% |
| Class TGM | -1.25% | -1.20% | -1.34% | 99.4% | 95.5% | 100.1% | 100.6% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.06% | -0.44% | -0.08% | 102.3% | 101.6% | 100.0% | 99.9% |
| Class C | -0.07% | -0.93% | -0.30% | 97.3% | 99.1% | 100.1% | 100.2% |
| Class E | 0.01% | 0.20% | 0.64% | 97.3% | 101.5% | 99.0% | 100.1% |
| Overall | -0.04% | -0.44% | 0.03% | 99.3% | 100.7% | 99.8% | 100.0% |
| Class D | -0.37% | -1.88% | -0.21% | 96.1% | 98.6% | 99.9% | 100.0% |
| Class F | -0.15% | 0.18% | 0.38% | 97.1% | 97.3% | 101.0% | 100.1% |
| Class TGM | -0.90% | -0.95% | -0.94% | 102.4% | 103.3% | 100.2% | 100.0% |

### 6.2a – additional test with EP only contexts

Test only retrained EP bins without training the original CABAC coded bins. EP bins can choose between EP and NEP model.

The following statistics is obtained:

| Model | Before | After |
| --- | --- | --- |
| NEP | 0 | 820 |
| EP | 2043 | 1223 |

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | 0.13% | 0.02% | 99.2% | 98.8% | 100.3% | 101.5% |
| Class A2 | -0.05% | -0.02% | -0.05% | 98.4% | 98.1% | 101.4% | 101.8% |
| Class B | -0.01% | 0.04% | -0.02% | 99.4% | 98.6% | 98.7% | 100.9% |
| Class C | 0.01% | 0.00% | 0.00% | 98.8% | 98.2% | 99.9% | 100.2% |
| Class E | 0.00% | 0.01% | -0.12% | 99.1% | 99.0% | 100.8% | 100.7% |
| Overall | -0.01% | 0.03% | -0.03% | 99.0% | 98.5% | 100.0% | 101.0% |
| Class D | -0.02% | 0.05% | -0.21% | 97.9% | 97.1% | 100.0% | 100.1% |
| Class F | 0.00% | 0.08% | -0.09% | 98.4% | 97.6% | 101.2% | 100.4% |
| Class TGM | -0.19% | -0.11% | -0.24% | 98.5% | 98.1% | 100.9% | 100.8% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06% | -0.15% | -0.03% | 98.2% | 98.6% | 100.3% | 99.8% |
| Class A2 | -0.04% | -0.08% | -0.03% | 98.0% | 98.1% | 100.1% | 100.0% |
| Class B | -0.15% | -0.13% | -0.25% | 98.9% | 97.6% | 99.8% | 100.0% |
| Class C | -0.04% | -0.11% | 0.11% | 97.6% | 98.5% | 100.0% | 99.8% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.08% | -0.12% | -0.07% | 98.2% | 98.1% | 100.0% | 99.9% |
| Class D | -0.04% | -0.40% | 0.09% | 96.1% | 97.5% | 100.4% | 100.0% |
| Class F | -0.02% | -0.40% | -0.28% | 98.6% | 97.1% | 99.9% | 100.1% |
| Class TGM | -0.61% | -0.61% | -0.61% | 98.9% | 97.5% | 100.6% | 99.3% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.09% | 0.25% | 0.20% | 97.9% | 99.0% | 99.5% | 100.5% |
| Class C | -0.05% | -0.18% | -0.05% | 97.5% | 98.1% | 100.0% | 99.9% |
| Class E | 0.07% | 0.41% | -0.95% | 96.5% | 97.9% | 100.4% | 99.5% |
| Overall | -0.04% | 0.15% | -0.17% | 97.4% | 98.4% | 99.9% | 100.1% |
| Class D | -0.03% | -0.26% | -0.13% | 94.8% | 96.7% | 99.8% | 100.1% |
| Class F | 0.06% | -0.92% | -1.22% | 97.9% | 97.6% | 99.7% | 100.6% |
| Class TGM | -1.11% | -0.94% | -0.96% | 99.5% | 98.0% | 99.8% | 99.0% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.54% | 0.34% | 98.3% | 98.0% | 100.0% | 100.0% |
| Class C | -0.01% | -0.35% | 0.01% | 97.5% | 96.6% | 100.1% | 100.0% |
| Class E | 0.03% | 0.32% | -0.72% | 97.8% | 98.2% | 98.9% | 100.2% |
| Overall | 0.00% | 0.19% | -0.03% | 97.9% | 97.6% | 99.8% | 100.0% |
| Class D | -0.24% | -1.58% | -0.19% | 96.8% | 97.9% | 99.6% | 100.0% |
| Class F | 0.07% | 0.40% | -0.04% | 97.7% | 97.5% | 101.4% | 100.1% |
| Class TGM | -0.77% | -0.69% | -0.53% | 98.4% | 98.1% | 100.3% | 100.0% |

## Test 6.2b – using BVI sequences

All BVI sequences (using the additional last frame as described in AhG11 CTC) are used.

### 6.2b CABAC and EP contexts

Test retrained EP bins and original CABAC coded bins. EP bins can choose between EP and NEP models and CABAC bins can choose between CABAC, NEP or EP models.

The following statistics is obtained:

| Model | Before | After |
| --- | --- | --- |
| CABAC | 4770 | 4667 |
| NEP | 0 | 858 |
| EP | 2043 | 1288 |

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.06% | -0.03% | 101.1% | 100.4% | 99.1% | 100.9% |
| Class A2 | -0.03% | 0.03% | -0.01% | 100.7% | 100.3% | 101.4% | 101.4% |
| Class B | -0.01% | -0.09% | -0.01% | 100.9% | 101.7% | 99.8% | 101.1% |
| Class C | -0.01% | -0.14% | 0.00% | 101.4% | 103.8% | 99.9% | 100.2% |
| Class E | -0.04% | -0.24% | 0.17% | 101.6% | 103.7% | 99.7% | 100.6% |
| Overall | -0.02% | -0.08% | 0.02% | 101.1% | 102.0% | 100.0% | 100.8% |
| Class D | -0.07% | 0.01% | -0.34% | 100.1% | 100.4% | 100.0% | 100.1% |
| Class F | -0.01% | 0.00% | 0.02% | 100.9% | 101.8% | 102.4% | 100.5% |
| Class TGM | -0.11% | -0.04% | -0.14% | 102.2% | 102.6% | 98.5% | 100.8% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.12% | -0.15% | 0.03% | 101.1% | 99.3% | 100.1% | 99.9% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | -0.21% | -0.26% | -0.32% | 100.4% | 97.9% | 100.1% | 100.1% |
| Class C | -0.06% | -0.29% | -0.18% | 99.0% | 98.1% | 99.6% | 99.8% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0.19% | -0.02% | 0.02% | 97.4% | 96.3% | 100.3% | 100.0% |
| Class F | -0.05% | -0.24% | -0.21% | 100.2% | 100.3% | 99.9% | 100.2% |
| Class TGM | -0.25% | -0.27% | -0.22% | 101.7% | 102.3% | 99.3% | 101.6% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.09% | -0.71% | -0.42% | 102.0% | 99.5% | 99.5% | 100.5% |
| Class C | -0.03% | -0.16% | 0.13% | 97.2% | 97.9% | 100.0% | 99.7% |
| Class E | 0.06% | 0.34% | -0.57% | 98.5% | 98.8% | 99.9% | 99.8% |
| Overall | -0.03% | -0.27% | -0.27% | 99.5% | 98.8% | 99.8% | 100.0% |
| Class D | -0.24% | 0.08% | 0.89% | 93.6% | 96.3% | 100.3% | 100.1% |
| Class F | 0.06% | -0.51% | -0.54% | 97.5% | 96.7% | 101.3% | 101.1% |
| Class TGM | -0.57% | -0.70% | -0.61% | 102.0% | 95.2% | 99.9% | 100.4% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.10% | -0.48% | -0.60% | 102.1% | 102.0% | 100.0% | 100.2% |
| Class C | 0.02% | -0.81% | -0.74% | 97.0% | 98.3% | 100.1% | 100.1% |
| Class E | -0.12% | -1.03% | 0.52% | 97.4% | 101.2% | 99.0% | 99.9% |
| Overall | -0.07% | -0.72% | -0.37% | 99.2% | 100.5% | 99.8% | 100.1% |
| Class D | -0.34% | -1.43% | 0.28% | 97.9% | 99.1% | 99.8% | 100.0% |
| Class F | 0.01% | -0.15% | -0.36% | 98.8% | 98.4% | 101.5% | 100.1% |
| Class TGM | -0.47% | -0.55% | -0.45% | 102.4% | 101.7% | 100.2% | 100.0% |

### 6.2b- additional results with EP only contexts

Test only retrained EP bins without training the original CABAC coded bins. EP bins can choose between EP and NEP model.

The following statistics is obtained:

| Model | Before | After |
| --- | --- | --- |
| NEP | 0 | 755 |
| EP | 2043 | 1288 |

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | 0.05% | 0.04% | 99.6% | 98.0% | 98.6% | 101.8% |
| Class A2 | -0.04% | -0.04% | -0.11% | 99.7% | 97.7% | 101.4% | 101.6% |
| Class B | -0.01% | -0.02% | 0.01% | 98.3% | 97.6% | 99.1% | 101.0% |
| Class C | 0.01% | -0.06% | 0.02% | 98.8% | 99.1% | 100.8% | 100.3% |
| Class E | 0.01% | -0.02% | -0.10% | 100.0% | 99.6% | 100.4% | 100.6% |
| Overall | -0.01% | -0.02% | -0.02% | 99.2% | 98.3% | 100.0% | 101.0% |
| Class D | -0.02% | -0.13% | -0.14% | 97.1% | 96.3% | 100.1% | 100.1% |
| Class F | 0.02% | 0.09% | 0.08% | 98.1% | 97.5% | 103.9% | 100.4% |
| Class TGM | -0.04% | 0.00% | -0.15% | 99.5% | 99.8% | 99.7% | 100.7% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06% | 0.02% | 0.00% | 98.6% | 99.0% | 100.4% | 100.2% |
| Class A2 | 0.01% | 0.02% | -0.02% | 98.7% | 99.0% | 99.5% | 99.8% |
| Class B | -0.15% | -0.01% | -0.08% | 99.1% | 96.9% | 99.9% | 100.2% |
| Class C | -0.02% | -0.21% | -0.07% | 97.8% | 97.8% | 100.0% | 99.9% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.07% | -0.05% | -0.05% | 98.6% | 98.0% | 100.0% | 100.0% |
| Class D | -0.09% | -0.09% | 0.37% | 96.4% | 97.9% | 100.4% | 100.0% |
| Class F | -0.05% | -0.12% | -0.20% | 98.6% | 96.4% | 99.9% | 100.1% |
| Class TGM | -0.08% | -0.03% | 0.02% | 98.9% | 98.2% | 99.9% | 101.2% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.02% | -0.19% | -0.53% | 99.0% | 98.6% | 99.5% | 99.9% |
| Class C | -0.03% | -0.12% | 0.03% | 96.3% | 97.6% | 100.0% | 100.4% |
| Class E | -0.04% | 1.24% | -0.26% | 96.4% | 98.2% | 100.1% | 99.5% |
| Overall | -0.03% | 0.19% | -0.27% | 97.5% | 98.1% | 99.8% | 100.0% |
| Class D | -0.17% | 0.03% | -0.18% | 93.5% | 96.8% | 100.5% | 100.0% |
| Class F | 0.07% | -0.10% | -0.96% | 95.8% | 96.2% | 101.2% | 101.2% |
| Class TGM | -0.40% | -0.38% | -0.29% | 99.1% | 97.5% | 100.1% | 99.5% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.02% | 0.26% | 0.53% | 99.2% | 98.9% | 100.0% | 100.0% |
| Class C | 0.03% | -0.05% | 0.27% | 97.4% | 98.2% | 100.1% | 100.2% |
| Class E | 0.08% | 0.46% | -0.24% | 96.4% | 98.0% | 97.9% | 100.1% |
| Overall | 0.04% | 0.20% | 0.25% | 97.9% | 98.4% | 99.5% | 100.0% |
| Class D | -0.11% | -0.88% | 0.45% | 95.9% | 97.4% | 99.9% | 100.0% |
| Class F | 0.15% | 1.21% | 1.10% | 97.8% | 97.2% | 101.2% | 100.3% |
| Class TGM | -0.43% | -0.28% | -0.33% | 98.9% | 99.0% | 100.2% | 100.0% |

# Conclusion

This contribution proposes to add more flexibility to the current bin coding in ECM. Training on different datasets shows little difference in term of results and it is proposed to use the CTC training process in order to ease the training process for the group. It is proposed to adopt the tool in next version of ECM.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).