| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0087-v1 |
| --- | --- |

| Title: | AhG11 LOP/HOP training strategy study |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Galpin, T. Dumas P. Bordes | Tel: Email: | franck.galpin@interdigital.com |
| Source: | InterDigital Inc. |  |  |

_________ ____________________

# Abstract

In contributions JVET-AG0174 and JVET-AF0043, HOP3 and LOP2 filters respectively were adopted. They are the results of several rounds of joint design, training and optimization. The HOP filter requires 466 kMACs/pixel. The LOP filter requires 17.2 kMACs/pixel. Each model uses its own training set, the training is made of 3 stages. The last stage requires to encode the same set of sequences using NNVC, one with a HOP stage 2 model and the other one with a LOP stage 2 model. While HOP requires 20 epochs to train, LOP requires 60 epochs to train for stage III. In this contribution it is proposed to show the impact of unifying some aspects of the training for both models.

# Introduction

In AhG11, a joint effort has been made to develop neural-network based loop-filters. Two operating points are under development. Models share a lot of common characteristics:

- Input datasets are the same.

- Inputs are the same.

- Outputs are the same.

- Training scripts are the same.

- Inference process is the same.

- Some parameters are the same for stage III: luma-chroma balance.

The following points differ:

- The training dataset extracted after encoding the dataset with NNVC+NNLF is not the same.

- Some training hyper-parameters are different: number of epochs, schedule learning rate epochs etc.

- Model architecture: 1 branch for luma/chroma for HOP, 2 branches for LOP. HOP has wider convolutional layer, etc.

In this contribution it is proposed to see the impact of the following 2 aspects on the performance of LOP/HOP at stage III:

- The number of epochs to train (and the associated L1/L2 epochs switch and learning rate decay).

- The dataset used at stage III.

## NNVC-8.0 HOP model

Figure 1 HOP3 architecture

| Specifications |  |
| --- | --- |
| Framework: | SADL |
| Number of Parameters (Each Model) | 1 model: 1.4M |
| Parameter Precision (Bits) | int: 16 |
| Memory Parameter (MB) | 2.8MB |
| Multiplay Accumulate (MAC)/pixel | 466 kMAC/pix (assuming block-level input) |
| Total Conv. Layers | 196 |
| Total number of layers | 1026 |
| Training |  |
| Training set stage 3 | HOPds3: Use HOP1.st2 int16 model on NNVC 5.1 |
| Number of samples | 6.8M |
| Number of epochs | 18+2 |

## NNVC-8.0 LOP model

Figure 2 LOP2 architecture

| Specifications |  |
| --- | --- |
| Framework: | SADL |
| Number of Parameters (Each Model) | 1 model: 51k |
| Parameter Precision (Bits) | int: 16 |
| Memory Parameter (MB) | 102kB |
| Multiplay Accumulate (MAC)/pixel | 17.2 kMAC/pix (assuming block-level input) |
| Total Conv. Layers | 196 |
| Total number of layers | 595 |
| Training |  |
| Training set stage 3 | LOPds3: Use LOP2.st2 int16 model on NNVC 6.0 |
| Number of samples | 6.8M |
| Number of epochs | 58+2 |

# Simulations overview

## Training description for stage III

The HOP training has the following hyper-parameters:

- Number of L1 epochs: 18

- Number of additional L2 epochs: 2.

- Learning rate: 2e-4

- Learning rate decay of 1/10 at epoch 15, 17, 18

- Batch size: 32.

- Dataset: HOPds3.

The LOP training has the following hyper-parameters:

- Number of L1 epochs: 58

- Number of additional L2 epochs: 2

- Learning rate: 4e-4

- Learning rate decay of 1/10 at epoch 55, 57, 58

- Batch size: 128

- Dataset: LOPds3.

## Parameters modifications

The following modifications were performed on the original training:

- HOP: “LOP training”

  - Number of L1 epochs: 58

  - Number of additional L2 epochs: 2

  - Learning rate decay of 1/10 at epoch 55, 57, 58

- LOP: “HOP training”

  - Number of L1 epochs: 18

  - Number of additional L2 epochs: 2

  - Learning rate decay of 1/10 at epoch 15, 17, 18

  - Dataset: HOPds3

# Results

All results have been obtained with float model using NNVC-8.0 in VTM mode with either HOP or LOP models. Results on B/C/D in RA and AI.

## HOP with LOP training

### Validation loss

L2

L2

The above graph shows the validation loss when using “LOP training”. It shows that additional epochs allow to reduce both final L1 and L2 norms.

| L1 norm | luma | chroma | total |
| --- | --- | --- | --- |
| HOP training | 0.0084801 | 0.0045452 | 0.007918 |
| LOP training | 0.0084734 | 0.0045342 | 0.0079106 |

| L2 norm | luma | chroma | total |
| --- | --- | --- | --- |
| HOP training | 0.00021629 | 5.1255e-05 | 0.00019271 |
| LOP training | 0.00021592 | 5.1019e-05 | 0.00019236 |

### Performance

| HOP3 int | (official) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RA | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |  | AI | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |
| Class B | -11.10% | -12.81% | -11.89% | -11.31% |  | Class B | -8.53% | -6.91% | -7.58% | -8.32% |
| Class C | -12.15% | -16.49% | -15.36% | -12.78% |  | Class C | -9.75% | -9.89% | -12.10% | -9.96% |
| Class D | -13.43% | -16.07% | -16.36% | -13.89% |  | Class D | -9.42% | -9.42% | -12.58% | -9.68% |
| Overall | -12.23% | -15.12% | -14.54% | -12.66% |  | Overall | -9.24% | -8.74% | -10.75% | -9.32% |
| HOP3 float |  |  |  |  |  |  |  |  |  |  |
| RA | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |  | AI | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |
| Class B | -11.35% | -13.30% | -12.92% | -11.64% |  | Class B | -8.63% | -5.25% | -7.27% | -8.23% |
| Class C | -12.36% | -16.79% | -16.35% | -13.06% |  | Class C | -9.85% | -10.02% | -12.15% | -10.05% |
| Class D | -13.63% | -16.35% | -17.25% | -14.16% |  | Class D | -9.51% | -9.53% | -12.49% | -9.76% |
| Overall | -12.44% | -15.48% | -15.51% | -12.95% |  | Overall | -9.33% | -8.27% | -10.64% | -9.35% |
| HOP3 float – LOP train |  |  |  |  |  |  |  |  |  |  |
| RA | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |  | AI | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |
| Class B | -11.52% | -13.03% | -12.99% | -11.77% |  | Class B | -8.74% | -4.85% | -6.96% | -8.26% |
| Class C | -12.59% | -17.47% | -17.54% | -13.41% |  | Class C | -10.02% | -10.43% | -12.77% | -10.28% |
| Class D | -13.84% | -17.33% | -19.09% | -14.57% |  | Class D | -9.63% | -9.87% | -13.30% | -9.96% |
| Overall | -12.65% | -15.95% | -16.54% | -13.25% |  | Overall | -9.46% | -8.38% | -11.01% | -9.50% |

The performance on B/C/D shows an improvement of -0.3% for RA and -0.15% BD-rate gains when using longer training for official HOP3 model.

## LOP with HOP training

Several LOP architecture were tested to see the impact of the number of parameters on the gap between long and short training.

The trained architectures are the following:

| Name | N luma | N chroma |
| --- | --- | --- |
| LOP14/4 (original LOP2) | 14 | 4 |
| LOP24/8 | 24 | 8 |
| LOP40/12 | 40 | 12 |
| LOP96/28 | 96 | 28 |

### Validation loss

LOP 14/4

| L1 norm | luma | chroma | total |
| --- | --- | --- | --- |
| HOP training | 0.0086593 | 0.0047215 | 0.0080967 |
| LOP training | 0.0086433 | 0.0047104 | 0.0080815 |

| L2 norm | luma | chroma | total |
| --- | --- | --- | --- |
| HOP training | 0.00022533 | 5.5435e-05 | 0.00020106 |
| LOP training | 0.00022451 | 5.5136e-05 | 0.00020031 |

LOP 28/8

| L1 norm | luma | chroma | total |
| --- | --- | --- | --- |
| HOP training | 0.0086363 | 0.0046865 | 0.008072 |
| LOP training | 0.0086232 | 0.0046723 | 0.0080588 |

| L2 norm | luma | chroma | total |
| --- | --- | --- | --- |
| HOP training | 0.00022425 | 5.466e-05 | 0.00020002 |
| LOP training | 0.00022359 | 5.4311e-05 | 0.0001994 |

LOP 40/12

| L1 norm | luma | chroma | total |
| --- | --- | --- | --- |
| HOP training | 0.0086089 | 0.0046661 | 0.0080457 |
| LOP training | 0.0085919 | 0.004648 | 0.0080285 |

| L2 norm | luma | chroma | total |
| --- | --- | --- | --- |
| HOP training | 0.00022287 | 5.4136e-05 | 0.00019877 |
| LOP training | 0.00022201 | 5.3714e-05 | 0.00019797 |

LOP 96/28

| L1 norm | luma | chroma | total |
| --- | --- | --- | --- |
| HOP training | 0.008571 | 0.0046584 | 0.0080121 |
| LOP training | 0.0085588 | 0.0046433 | 0.0079995 |

| L2 norm | luma | chroma | total |
| --- | --- | --- | --- |
| HOP training | 0.00022111 | 5.4e-05 | 0.00019724 |
| LOP training | 0.00022047 | 5.3577e-05 | 0.00019663 |

### Performance

### Comparison

The gains of LOP training (longer training) are slightly decreasing with the complexity of the model: from -0.8% gains in RA for the original 14/4 model to -0.5% for the 96/28 model. In AI configuration, the gains change from -0.5% to -0.3%.

### LOP training

| LOP14/4 | (official) | Int |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RA | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |  | AI | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |
| Class B | -4.99% | -13.20% | -12.06% | -6.27% |  | Class B | -4.39% | -10.32% | -10.64% | -5.40% |
| Class C | -5.12% | -13.75% | -13.25% | -6.52% |  | Class C | -4.64% | -11.11% | -11.70% | -5.77% |
| Class D | -6.02% | -14.05% | -13.96% | -7.35% |  | Class D | -5.00% | -10.10% | -12.34% | -6.04% |
| Overall | -5.38% | -13.67% | -13.09% | -6.71% |  | Overall | -4.68% | -10.51% | -11.56% | -5.74% |
| LOP14/4 |  |  |  |  |  |  |  |  |  |  |
| RA | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |  | AI | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |
| Class B | -4.92% | -11.38% | -10.62% | -5.93% |  | Class B | -4.33% | -9.67% | -10.22% | -5.27% |
| Class C | -5.10% | -12.51% | -11.79% | -6.27% |  | Class C | -4.61% | -10.33% | -10.78% | -5.60% |
| Class D | -6.15% | -12.50% | -12.99% | -7.25% |  | Class D | -4.96% | -9.35% | -11.46% | -5.87% |
| Overall | -5.39% | -12.13% | -11.80% | -6.48% |  | Overall | -4.63% | -9.78% | -10.82% | -5.58% |
| LOP28/8 |  |  |  |  |  |  |  |  |  |  |
| RA | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |  | AI | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |
| Class B | -5.50% | -14.89% | -13.58% | -6.96% |  | Class B | -4.75% | -12.29% | -13.23% | -6.08% |
| Class C | -5.78% | -15.73% | -14.95% | -7.37% |  | Class C | -5.05% | -12.62% | -13.33% | -6.37% |
| Class D | -6.61% | -15.57% | -16.22% | -8.16% |  | Class D | -5.35% | -11.70% | -14.01% | -6.60% |
| Overall | -5.97% | -15.40% | -14.92% | -7.50% |  | Overall | -5.05% | -12.21% | -13.53% | -6.35% |
| LOP40/12 |  |  |  |  |  |  |  |  |  |  |
| RA | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |  | AI | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |
| Class B | -6.61% | -16.09% | -15.25% | -8.12% |  | Class B | -5.33% | -12.90% | -13.94% | -6.68% |
| Class C | -6.92% | -17.27% | -16.20% | -8.56% |  | Class C | -5.80% | -13.15% | -14.31% | -7.12% |
| Class D | -7.98% | -17.17% | -17.31% | -9.52% |  | Class D | -6.00% | -12.27% | -15.00% | -7.27% |
| Overall | -7.17% | -16.84% | -16.25% | -8.73% |  | Overall | -5.71% | -12.78% | -14.42% | -7.02% |
| LOP96/28 |  |  |  |  |  |  |  |  |  |  |
| RA | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |  | AI | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |
| Class B | -7.64% | -15.65% | -14.67% | -8.90% |  | Class B | -5.99% | -12.76% | -13.13% | -7.15% |
| Class C | -7.97% | -17.11% | -16.00% | -9.40% |  | Class C | -6.64% | -12.95% | -13.90% | -7.77% |
| Class D | -9.15% | -16.85% | -16.57% | -10.41% |  | Class D | -6.67% | -12.26% | -14.46% | -7.79% |
| Overall | -8.25% | -16.54% | -15.75% | -9.57% |  | Overall | -6.43% | -12.66% | -13.83% | -7.57% |

### HOP training

| LOP14/4 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RA | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |  | AI | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |
| Class B | -4.41% | -10.12% | -9.84% | -5.34% |  | Class B | -4.02% | -8.92% | -9.85% | -4.92% |
| Class C | -4.68% | -11.66% | -10.63% | -5.76% |  | Class C | -4.24% | -9.65% | -10.15% | -5.18% |
| Class D | -5.48% | -11.79% | -11.88% | -6.54% |  | Class D | -4.61% | -8.75% | -10.88% | -5.47% |
| Overall | -4.86% | -11.19% | -10.78% | -5.88% |  | Overall | -4.29% | -9.10% | -10.29% | -5.19% |
| LOP28/8 |  |  |  |  |  |  |  |  |  |  |
| RA | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |  | AI | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |
| Class B | -5.10% | -13.34% | -12.11% | -6.37% |  | Class B | -4.50% | -11.39% | -11.98% | -5.70% |
| Class C | -5.27% | -13.99% | -13.60% | -6.69% |  | Class C | -4.71% | -11.67% | -12.31% | -5.92% |
| Class D | -6.13% | -14.19% | -14.52% | -7.50% |  | Class D | -5.05% | -10.43% | -12.82% | -6.15% |
| Overall | -5.50% | -13.84% | -13.41% | -6.85% |  | Overall | -4.75% | -11.16% | -12.37% | -5.92% |
| LOP40/12 |  |  |  |  |  |  |  |  |  |  |
| RA | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |  | AI | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |
| Class B | -6.07% | -15.12% | -13.74% | -7.47% |  | Class B | -4.99% | -12.38% | -13.25% | -6.30% |
| Class C | -6.32% | -15.74% | -14.97% | -7.82% |  | Class C | -5.38% | -12.45% | -13.38% | -6.64% |
| Class D | -7.32% | -15.74% | -16.34% | -8.77% |  | Class D | -5.63% | -11.67% | -14.29% | -6.85% |
| Overall | -6.57% | -15.53% | -15.02% | -8.02% |  | Overall | -5.33% | -12.17% | -13.64% | -6.60% |
| LOP96/28 |  |  |  |  |  |  |  |  |  |  |
| RA | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |  | AI | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |
| Class B | -7.30% | -14.94% | -13.64% | -8.46% |  | Class B | -5.73% | -12.36% | -12.65% | -6.86% |
| Class C | -7.51% | -16.10% | -14.98% | -8.85% |  | Class C | -6.27% | -12.44% | -13.34% | -7.38% |
| Class D | -8.56% | -16.39% | -16.13% | -9.85% |  | Class D | -6.39% | -11.80% | -14.22% | -7.49% |
| Overall | -7.79% | -15.81% | -14.92% | -9.05% |  | Overall | -6.13% | -12.20% | -13.41% | -7.24% |

## LOP with HOP dataset

### Performance

#### LOP dataset

| LOP14/4 | (official) | Int LOP |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RA | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |  | AI | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |
| Class B | -4.99% | -13.20% | -12.06% | -6.27% |  | Class B | -4.39% | -10.32% | -10.64% | -5.40% |
| Class C | -5.12% | -13.75% | -13.25% | -6.52% |  | Class C | -4.64% | -11.11% | -11.70% | -5.77% |
| Class D | -6.02% | -14.05% | -13.96% | -7.35% |  | Class D | -5.00% | -10.10% | -12.34% | -6.04% |
| Overall | -5.38% | -13.67% | -13.09% | -6.71% |  | Overall | -4.68% | -10.51% | -11.56% | -5.74% |

| LOP14/4 | official | float |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RA | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |  | AI | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |
| Class B | -5.05% | -13.22% | -12.28% | -6.33% |  | Class B | -4.43% | -10.39% | -10.72% | -5.45% |
| Class C | -5.16% | -13.73% | -13.34% | -6.55% |  | Class C | -4.72% | -11.05% | -11.56% | -5.81% |
| Class D | -6.07% | -14.00% | -14.05% | -7.40% |  | Class D | -5.09% | -10.10% | -12.38% | -6.11% |
| Overall | -5.43% | -13.65% | -13.22% | -6.76% |  | Overall | -4.74% | -10.51% | -11.55% | -5.79% |

#### HOP dataset

| LOP14/4 | float |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RA | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |  | AI | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |
| Class B | -4.92% | -11.38% | -10.62% | -5.93% |  | Class B | -4.33% | -9.67% | -10.22% | -5.27% |
| Class C | -5.10% | -12.51% | -11.79% | -6.27% |  | Class C | -4.61% | -10.33% | -10.78% | -5.60% |
| Class D | -6.15% | -12.50% | -12.99% | -7.25% |  | Class D | -4.96% | -9.35% | -11.46% | -5.87% |
| Overall | -5.39% | -12.13% | -11.80% | -6.48% |  | Overall | -4.63% | -9.78% | -10.82% | -5.58% |

The float model trained on HOP stage 3 dataset as a bdrate difference of -0.25% in RA and -0.2% in AI on YUV metrics. Given that the official model is the best of 4 models where the difference in gains was -0.15% in RA and AI, the difference is quite small between the training with LOP dataset and HOP dataset.

# Conclusion

The above results show some impact of the training to take into consideration for future HOP/LOP training:

- longer training is beneficial for more complex models. For a “final” HOP training, a longer training is still worth it. About 0.2%-0.3% are still in the margin of error of the training time and process.

- Impact of dataset on the performance seems minor while increasing the burden for proponents to train both LOP and HOP.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).