| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0093-v2 |
| --- | --- |

| Title: | EE2-2.6: Adaptive MRL Fusion |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Saverio Blasi Jani Lainema | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document reports on experiment EE2-2.6 on Adaptive MRL Fusion.

The impact on coding efficiency and runtimes of the EE2 tests over ECM-12.0 are reportedly {for Y, U, V, EncT, DecT, EncVmPeak, DecVmPeak}:

Test 2.3a:

AI {0.03%, 0.04%, 0.05%, 100.8%, 101.0%, 100.1%, 100.6%}

Test 2.3b:

AI {-0.04%, 0.02%	, 0.01%	, 100.7%, 101.1%, 99.8%, 100.5%}

RA {-0.01%, -0.02%, -0.08%, 100.5%, 100.9%, 100.4%, 100.5%}

# Proposed Method

In test EE2-2.6, Adaptive MRL Fusion [1] is tested. When using Adaptive MRL Fusion the MRL indexes used to obtain the additional MRL Fusion predictors are selected depending on the current MRL index. The MRL Fusion process is modified so that two additional predictors are computed, rather than only one.

When the current MRL index i = 0, then MRL indexes 1 and 2 are used to compute the additional predictors. Conversely, when the current MRL index is i > 0, then the MRL indexes i + 1 and i – 1 are used. Also when the current MRL index is i > 0, then fixed weights set to W1 = 2/8, W2 = 2/8 and Wmain = 4/8 are used to blend the predictors.

In Test 2.6a, fixed weights equal to 5/8, 2/8 and 1/8 are used if i = 0. In Test 2.6b the fusion weights when the current MRL index i = 0 are determined adaptively for each block using a template-based search.

# Experimental results

The proposed methods are implemented on top of ECM-12.0 [2] and tested based on the common test conditions [3]. The simulation results are summarized in the following Tables.

Table 1: Coding performance of EE2-2.6a

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | -0.02% | 0.04% | 100.7% | 101.0% | 100.7% | 100.6% |
| Class A2 | 0.00% | 0.06% | -0.04% | 100.3% | 101.2% | 100.4% | 100.5% |
| Class B | 0.00% | 0.08% | 0.09% | 100.5% | 101.7% | 99.3% | 100.7% |
| Class C | 0.07% | -0.05% | 0.10% | 101.2% | 99.7% | 100.1% | 100.6% |
| Class E | 0.10% | 0.13% | 0.02% | 101.2% | 101.6% | 100.3% | 100.5% |
| Overall | 0.03% | 0.04% | 0.05% | 100.8% | 101.0% | 100.1% | 100.6% |
| Class D | 0.05% | 0.30% | 0.11% | 101.4% | 103.7% | 100.7% | 100.7% |
| Class F | 0.14% | 0.09% | -0.01% | 101.4% | 101.0% | 100.4% | 100.7% |

Table 2: Coding performance of EE2-2.6b

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04% | 0.04% | 0.03% | 100.3% | 101.5% | 100.4% | 100.5% |
| Class A2 | -0.04% | 0.01% | -0.03% | 100.6% | 101.0% | 100.4% | 100.5% |
| Class B | -0.05% | 0.02% | -0.01% | 100.4% | 101.0% | 98.2% | 100.6% |
| Class C | -0.01% | 0.01% | 0.06% | 101.3% | 100.5% | 100.4% | 100.6% |
| Class E | -0.04% | 0.05% | -0.14% | 101.1% | 101.5% | 100.3% | 100.5% |
| Overall | -0.04% | 0.02% | -0.01% | 100.7% | 101.1% | 99.8% | 100.5% |
| Class D | -0.02% | -0.03% | -0.08% | 101.6% | 101.2% | 100.5% | 100.6% |
| Class F | -0.04% | 0.04% | -0.13% | 102.0% | 100.8% | 100.4% | 100.7% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.10% | -0.01% | 99.5% | 101.0% | 100.2% | 100.1% |
| Class A2 | 0.02% | 0.13% | 0.00% | 99.4% | 101.0% | 100.6% | 100.9% |
| Class B | -0.02% | 0.07% | -0.14% | 101.4% | 100.4% | 99.9% | 100.5% |
| Class C | 0.01% | -0.19% | -0.13% | 100.9% | 101.4% | 101.0% | 100.6% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.01% | -0.02% | -0.08% | 100.5% | 100.9% | 100.4% | 100.5% |
| Class D | 0.02% | 0.18% | 0.57% | 100.5% | 102.8% | 100.4% | 100.7% |
| Class F | -0.04% | -0.31% | -0.31% | 100.1% | 100.5% | 99.8% | 101.1% |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- S. Blasi, J. Lainema, “AHG12: Adaptive MRL Fusion”, JVET-AG0075, Jan. 2024.

- ECM12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-12.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct, 2023.