| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0094-v1 |
| --- | --- |

| Title: | EE2-2.8: Combination of tests EE2-2.6b and EE2-2.7 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Saverio Blasi, Jani Lainema (Nokia) Zhipin Deng, Kai Zhang, Li Zhang (Bytedance) | Email: | firstname.lastname@nokia.com {zhipin.deng, zhangkai.video, lizhang.idm}@bytedance.com |
| Source: | Nokia, Bytedance Inc. |  |  |

_____________________________

# Abstract

This document reports on experiment EE2-2.8, which is a combination of tests EE2-2.6b on Adaptive MRL Fusion and EE2-2.7 on relaxing line buffer restriction. The impact on coding efficiency and runtimes over ECM-12.0 is reportedly {for Y, U, V, EncT, DecT, EncVmPeak, DecVmPeak}:

AI {-0.06%	, -0.05%, -0.03%, XX%, XX%, XX%, XX%}

RA {-0.03%, -0.04%, -0.11%, XX%, XX%, XX%, XX%}

# Proposed Method

In test EE2-2.8, test EE2-2.6b on Adaptive MRL Fusion [1] and test EE2-2.7 on relaxing line buffer restriction [2] are combined together. In particular in Test EE2-2.6b the MRL indexes used to obtain the additional MRL Fusion predictors are selected depending on the current MRL index, and the fusion weights are determined adaptively for each block using a template-based search. Moreover, Test EE2-2.7 proposes relaxing the top line of CTU restriction to align with the principle of other coding tools in ECM that can access more than one line of reconstructed reference samples for CUs on the CTU top boundary.

# Experimental results

The proposed methods are implemented on top of ECM-12.0 [3] and tested based on the common test conditions [4]. The simulation results are summarized in the following Tables.

Table 1: Coding performance of EE2-2.8

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06% | -0.06% | 0.01% |  |  | 100.4% | 100.5% |
| Class A2 | -0.06% | -0.08% | -0.01% |  |  | 100.4% | 100.5% |
| Class B | -0.09% | -0.03% | 0.03% |  |  | 98.2% | 100.7% |
| Class C | -0.04% | -0.04% | 0.05% |  |  | 100.2% | 100.7% |
| Class E | -0.06% | -0.06% | -0.30% |  |  | 100.4% | 100.7% |
| Overall | -0.06% | -0.05% | -0.03% |  |  | 99.7% | 100.6% |
| Class D | -0.04% | 0.01% | -0.09% |  |  | 100.6% | 100.7% |
| Class F | -0.06% | -0.01% | -0.19% |  |  | 100.4% | 100.7% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05% | 0.05% | -0.16% |  |  | 99.9% | 100.0% |
| Class A2 | -0.04% | -0.06% | -0.01% |  |  | 100.2% | 100.6% |
| Class B | -0.05% | -0.04% | -0.22% |  |  | 100.1% | 100.4% |
| Class C | 0.03% | -0.11% | 0.00% |  |  | 101.3% | 100.6% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.03% | -0.04% | -0.11% |  |  | 100.4% | 100.4% |
| Class D | 0.01% | -0.12% | 0.42% |  |  | 100.1% | 100.7% |
| Class F | 0.00% | -0.05% | 0.03% |  |  | 99.2% | 100.9% |

# Reference

- S. Blasi, J. Lainema, “AHG12: Adaptive MRL Fusion”, JVET-AG0075, Jan. 2024.

- Z. Deng, K. Zhang, L. Zhang, “Non-EE2: On line buffer restriction”, JVET-AG0120, Jan. 2024.

- ECM12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-12.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct, 2023.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).