| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0098_v2 |
| --- | --- |

| Title: | EE2-5.2: Coding Information based Classification for ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Yang Wang1, Zhipin Deng1, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA Nan Hu, Marta Karczewicz, Vadim Seregin 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com nanh@qti.qualcomm.com martak@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | Bytedance Inc. and Qualcomm Incorporated |  |  |

_____________________________

Abstract

In this contribution, the results of EE2-5.2 are reported. In EE2-5.2, a coding-information based classification method is applied for Luma Adaptive Loop Filter (ALF). In the proposed method, the partitioning and prediction information are introduced into the Luma-ALF classification process to build up an additional rule. There are 3 subtests in EE2-5.2. Test 5.2a only uses partitioning information and test 5.2b only uses prediction information. Test 5.2c uses both partitioning and prediction information.

On top of ECM-12.0, simulation results of EE2-5.2 are reported as below:

EE2-5.2a:

AI:  0.00%, -0.01%, -0.01%, 99.9%, 100.5%.

RA: -0.03%, -0.02%, -0.06%, 99.7%, 100.5%.

LB: -0.07%, -0.05%, 0.06%, 99.6%, 101.1%.

EE2-5.2b:

AI:  0.00%, -0.01%, -0.01%, 99.1%, 99.7%.

RA: 0.00%, -0.02%, -0.06%, 99.8%, 100.1%.

LB: 0.04%, -0.39%, 0.01%, 100.5%, 100.0%.

EE2-5.2c:

AI:  0.01%, -0.01%, -0.01%, 99.9%, 100.3%.

RA: -0.04%, -0.03%, -0.06%, 99.8%, 100.5%.

LB: -0.08%, -0.21%, -0.10%, 100.5%, 100.4%.

Introduction

In the design of Luma-ALF classification in ECM-12.0, there are totally 3 classifiers which are texture-based, band-based and residual-based classifiers. The encoder collects corresponding statistical data for each classifier and selects the most suitable classifier for each CTU. In current classification design, coding information such as the partitioning and prediction information are not well considered and utilized.

Proposed Method

In EE2-5.2, an additional rule for Luma-ALF classification is proposed. By using the partitioning and  prediction information, each classification unit is classified into 2 noise levels by the newly added classification rule. For each classification unit (2x2), it would be classified into noise level 1 if it is located at a block boundary position or coded by MDOE_INTRA. Otherwise, the unit would be classified into noise level 0.

Then the generated noise levels are further combined with the existing classifiers to output the final classification results. The class number of existing classifiers is reduced to 12 from 25. For the band and residual classifier, the range of 0 to 2bitdepth – 1 are quantized into 12 classes. For the texture classier, a LUT is used to map 25 classes to 12 classes. The 12 classes are further combined with the proposed 2 noise levels. Finally, for each classifier, the number of classes is 12 x 2 = 24.

The proposed method always appied and no signaling is required. In such a design, the number of classifiers is kept as 3 and no additional encoder selection is introduced.

There are 3 subtests inside EE2-5.2. In test 5.2a, only the boundary information is utilized. In test 5.2b, only the prediction information is utilized. In test 5.2c, both boundary and prediction information are utilized.

Simulation Results

The proposed method was implemented on top of the ECM-12.0 [1]. Simulations were performed following the common test conditions [2]. More detailed results could be found in the attached excel sheets. The results compared to ECM-12.0 are shown in Table 1, Table 2 and Table 3 for test 5.2a, test 5.2b and 5.2c respectively.

Table 1: Performance of 5.2a

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.00% | 0.00% | 99.4% | 100.4% | 100.9% | 101.2% |
| Class A2 | -0.02% | 0.00% | 0.00% | 99.8% | 100.5% | 99.6% | 101.1% |
| Class B | 0.01% | -0.01% | -0.01% | 100.5% | 101.7% | 99.6% | 100.4% |
| Class C | 0.00% | 0.00% | 0.00% | 100.1% | 100.9% | 98.4% | 100.1% |
| Class E | 0.00% | -0.01% | -0.01% | 99.4% | 98.0% | 102.7% | 100.3% |
| Overall | 0.00% | -0.01% | -0.01% | 99.9% | 100.5% | 100.1% | 100.6% |
| Class D | 0.00% | 0.00% | 0.00% | 99.5% | 101.6% | 99.7% | 100.1% |
| Class F | 0.02% | -0.02% | -0.02% | 101.0% | 99.4% | 102.0% | 100.2% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | 0.00% | -0.09% | 98.6% | 100.8% | 99.9% | 100.5% |
| Class A2 | 0.00% | -0.02% | -0.07% | 99.3% | 100.7% | 100.0% | 100.7% |
| Class B | -0.02% | -0.03% | -0.08% | 100.8% | 100.3% | 99.4% | 100.4% |
| Class C | -0.08% | -0.02% | 0.01% | 99.5% | 100.5% | 99.2% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.03% | -0.02% | -0.06% | 99.7% | 100.5% | 99.6% | 100.4% |
| Class D | -0.02% | 0.05% | 0.02% | 100.6% | 99.5% | 99.7% | 100.1% |
| Class F | 0.02% | 0.02% | -0.09% | 100.2% | 100.9% | 99.4% | 100.2% |
|  | Low Delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.01% | -0.02% | 0.11% | 99.6% | 101.1% | 99.4% | 99.5% |
| Class C | -0.18% | 0.06% | 0.02% | 98.6% | 99.8% | 100.1% | 100.5% |
| Class E | -0.02% | -0.23% | 0.05% | 100.7% | 102.6% | 100.0% | 100.1% |
| Overall | -0.07% | -0.05% | 0.06% | 99.6% | 101.1% | 99.8% | 100.0% |
| Class D | -0.08% | 0.15% | -0.56% | 99.2% | 99.9% | 99.7% | 100.0% |
| Class F | -0.03% | -0.48% | -0.90% | 98.7% | 100.6% | 101.2% | 100.4% |

Table 2: Performance of 5.2b

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.02% | -0.01% | -0.01% | 99.5% | 100.4% | 100.8% | 101.1% |
| Class A2 | -0.02% | 0.00% | 0.00% | 99.7% | 99.6% | 99.6% | 101.1% |
| Class B | 0.01% | -0.01% | -0.01% | 98.3% | 100.5% | 98.5% | 100.4% |
| Class C | 0.00% | 0.00% | 0.00% | 98.2% | 100.0% | 98.4% | 100.1% |
| Class E | 0.00% | -0.01% | -0.01% | 100.4% | 97.5% | 101.5% | 100.3% |
| Overall | 0.00% | -0.01% | -0.01% | 99.1% | 99.7% | 99.5% | 100.6% |
| Class D | 0.00% | 0.00% | 0.00% | 98.7% | 99.3% | 99.7% | 100.1% |
| Class F | 0.02% | -0.03% | -0.03% | 99.3% | 100.6% | 102.0% | 100.2% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | -0.06% | -0.10% | 99.8% | 99.2% | 100.1% | 100.4% |
| Class A2 | -0.01% | 0.01% | -0.10% | 99.0% | 99.4% | 100.1% | 100.5% |
| Class B | 0.00% | -0.02% | -0.07% | 99.8% | 99.9% | 99.3% | 100.2% |
| Class C | 0.01% | -0.01% | 0.01% | 100.6% | 101.6% | 99.2% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.06% | 99.8% | 100.1% | 99.6% | 100.3% |
| Class D | -0.01% | -0.16% | -0.06% | 101.0% | 99.4% | 100.0% | 100.1% |
| Class F | 0.06% | 0.04% | -0.02% | 100.0% | 99.7% | 99.4% | 100.2% |
|  | Low Delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.01% | -0.27% | -0.04% | 100.6% | 99.8% | 99.3% | 100.2% |
| Class C | 0.02% | -0.39% | -0.03% | -0.39% | -0.03% | 100.2% | 101.7% |
| Class E | 0.10% | -0.57% | 0.15% | -0.57% | 0.15% | 100.9% | 98.4% |
| Overall | 0.04% | -0.39% | 0.01% | 100.5% | 100.0% | 99.5% | 100.3% |
| Class D | -0.12% | 0.35% | -0.25% | 97.9% | 98.7% | 99.8% | 100.0% |
| Class F | 0.52% | -0.19% | -0.64% | 101.2% | 99.9% | 101.7% | 100.9% |

Table 3: Performance of 5.2c

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.02% | -0.02% | -0.02% | 103.6% | 100.9% | 100.8% | 101.1% |
| Class A2 | 0.00% | -0.01% | -0.01% | 99.7% | 100.9% | 98.8% | 101.1% |
| Class B | 0.01% | -0.01% | -0.01% | 99.2% | 101.5% | 98.6% | 100.5% |
| Class C | 0.00% | 0.00% | 0.00% | 98.9% | 99.8% | 98.4% | 100.1% |
| Class E | 0.00% | -0.01% | -0.01% | 98.9% | 98.0% | 102.7% | 100.3% |
| Overall | 0.01% | -0.01% | -0.01% | 99.9% | 100.3% | 99.6% | 100.6% |
| Class D | 0.00% | 0.00% | 0.00% | 99.0% | 100.5% | 99.7% | 100.1% |
| Class F | 0.01% | -0.02% | -0.02% | 99.9% | 100.8% | 101.2% | 100.2% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04% | -0.21% | -0.14% | 99.7% | 101.0% | 100.2% | 100.6% |
| Class A2 | -0.01% | 0.10% | -0.03% | 99.2% | 100.4% | 99.8% | 100.3% |
| Class B | -0.05% | -0.03% | -0.02% | 100.1% | 100.2% | 99.4% | 100.3% |
| Class C | -0.06% | -0.01% | -0.06% | 100.1% | 100.4% | 99.6% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.04% | -0.03% | -0.06% | 99.8% | 100.5% | 99.7% | 100.3% |
| Class D | -0.04% | -0.09% | 0.06% | 99.9% | 99.4% | 99.8% | 100.1% |
| Class F | 0.03% | 0.03% | -0.09% | 99.6% | 101.7% | 99.4% | 100.2% |
|  | Low Delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.02% | -0.04% | 0.24% | 101.6% | 100.7% | 99.9% | 100.0% |
| Class C | -0.16% | 0.03% | 0.02% | 98.7% | 101.5% | 100.1% | 100.4% |
| Class E | -0.09% | -0.82% | -0.83% | 101.1% | 98.6% | 100.1% | 100.2% |
| Overall | -0.08% | -0.21% | -0.10% | 100.5% | 100.4% | 100.0% | 100.2% |
| Class D | -0.12% | 0.06% | -0.46% | 97.3% | 98.5% | 99.9% | 100.0% |
| Class F | 0.26% | -0.04% | -0.20% | 102.0% | 99.4% | 101.4% | 100.4% |

Conclusions

In EE2-5.2, an additional classification rule based on coding-information is proposed for Luma-ALF. The proposed method can achieve promising coding performance with negligible encoding and decoding time increase. It is recommended to include the proposed method into the next version of ECM.

Reference

- ECM-12.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-12.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AF2017, DE, October 2023.

Patent Rights Declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).