| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0121-v1 |
| --- | --- |

| Title: | AHG9: On the SEI processing order SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Francesco Cricri Jill Boyce Nokia Technologies, Tampere, Finland | Email: | {miska.hannuksela ¦ francesco.cricri ¦ jill.boyce}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This input document proposes the following items for the SEI processing order (SPO) SEI message:

- It is proposed to allow region-wise packing (RWP) SEI message in an SPO SEI message and specify the derivation of its interface variable, ChromaFormatIdc, in a similar manner as done for the film grain characteristics SEI message in JVET-AG2027.

- It is proposed to indicate if an SEI message type in an SPO SEI message indicates a process or a property as follows:

  - It is proposed to add a flag, po_sei_process_flag[ i ], to indicate whether the i-th type of an SEI message indicates a process or a property.

  - It is proposed that when po_sei_process_flag[ i ] is equal to 0, it indicates a property of the output of the previous process that is associated with the same processing order value.

  - It is proposed to clarify that when the region-wise packing SEI message is used as a process, the output of the process is a projected picture as specified by the region-wise packing SEI message.

- The following aspects are proposed in relation to processes that have the same processing order value:

  - It is proposed to add a flag (po_sei_alternative_flag[ i ]) that enables the encoder to indicate the intended processing order for the processes with the same processing order value to be either a cascade in any order or a selection among the processes.

  - Since the proposed semantics of po_sei_alternative_flag[ i ] refer to the semantics of po_sei_processing_order[ i ], po_sei_processing_order[ i ] is proposed to be moved up in the syntax so that it appears before po_sei_alternative_flag[ i ].

  - It is proposed that all the types of SEI messages within an SPO SEI message that indicate processes (rather than properties) and have the same processing order value shall either be alternatives to each other or be processed in any order. This is asserted to simplify the proposed semantics.

  - Depending on adoption of item 2, two options are proposed to identify alternative processes among the types of SEI messages that have the same processing order value:

    - If item 2 is adopted, the alternative processes have po_sei_process_flag[ i ] equal to 1.

    - If item 2 is not adopted, it is proposed that alternative processes have subsequent indices in the SPO SEI message. This is asserted to simplify the semantics and enable the decoding system to identify processes from properties even if the decoding system were unaware of some SEI message payload type values. However, it is not possible to provide SEI messages to indicate properties for the alternative processes individually.

- It is proposed to adopt the mechanism used in the SEI message syntax to represent the payload type value in the SPO SEI message. The reserved zero bits that result from this change are proposed to be specified in a way that allows future extensions.

- It is proposed to fix the ChromaFormatIdc derivation for the use of the colour transform information SEI message as a part of a processing chain in a similar manner as done for the film grain characteristics SEI message in JVET-AG2027.

- It is proposed to fix the interface variable derivation for the use of the NNPFC and NNPFA SEI messages as a part of a processing chain in a similar manner as done for the film grain characteristics SEI message in JVET-AG2027.

# Allowing region-wise packing SEI message in an SPO SEI message

## Problem statement

It is asserted that the region-wise packing (RWP) SEI message included in an SPO and PON SEI messages can be used to indicate region boundaries applying after the preceding processing stage(s), such as spatial resampling.

## Overview of the proposal

It is proposed to allow region-wise packing (RWP) SEI message in an SPO SEI message and specify the derivation of its interface variable, ChromaFormatIdc, in a similar manner as done for the film grain characteristics SEI message in JVET-AG2027.

## Proposed specification text

#### D.11.1.2		SEI processing order SEI message semantics

SeiProcessingOrderSeiList is set to consist of the payloadType values 3, 4, 5, 19, 137, 142, 144, 147, 148, 149, 155, 165, 177, 210, and 211. The value of po_sei_payload_type[ i ] for each i in the range of 0 to po_num_sei_messages_minus2 + 1, inclusive, shall be equal to a value in SeiProcessingOrderSeiList.

### D.xx.5	Use of the equirectangular projection, generalized cubemap projection, and region-wise packing SEI messages

For purposes of interpretation of the equirectangular projection, and generalized cubemap projection, and region-wise packing SEI message, the following variable is specified:

–	ChromaFormatIdc is set equal to sps_chroma_format_idc.

For purposes of interpretation of the region-wise packing SEI message, the variable ChromaFormatIdc is specified as follows:

–	If the region-wise packing SEI message is not applied as part of a processing chain indicated by an SEI processing order SEI message or is applied as the first processing step of the processing chain indicated by an SEI processing order SEI message, ChromaFormatIdc is set equal to sps_chroma_format_idc.

–	Otherwise (the region-wise packing SEI message is applied as the second or a later processing step of the processing chain indicated by an SEI processing order SEI message), prevPic is a picture resulting from the previous processing step of the processing chain indicated by the SEI processing order SEI message, and ChromaFormatIdc is set equal to the picture chroma format indicator of prevPic.

# Indicating if an SEI message is descriptive or transformative

## Problem statement

A type of an SEI message may imply a process to be performed or indicate a property according to: JVET-AG2027:

po_sei_processing_order[ m ] equal to po_sei_processing_order[ n ] indicates that there is no preferred order of processing between the types of SEI messages associated with indexes m and n (e.g., they can indicate different properties that are both applicable at that stage, or alternative processes that can be applied, or one can indicate a property and the other can indicate a process).

It is asserted that the region-wise packing (RWP) SEI message could be intended for either implying a process to be performed or indicating a property:

- When implying a process, the RWP SEI message causes unpacking a region-wise-packed picture to an omnidirectional picture.

- When indicating a property, the RWP SEI message can be used to indicate region boundaries applying after the preceding processing stage(s), such as spatial resampling.

Figure 1 presents an example how an encoding system rearranges an equirectangular projected picture into a packed picture prior to encoding the packed picture. The top and bottom stripes of the projected picture are resampled and positioned in the packed picture. The encoding system may include an RWP SEI message as an SEI message type in an SPO SEI message. When the RWP SEI message is interpreted as a property, it indicates the region boundaries within the packed picture and the corresponding regions in the projected picture. When the RWP SEI message is interpreted as a process, it maps the packed picture back to a projected picture and all subsequent processing steps are performed to the projected picture.

Figure 1 – An example of encoding system operation for region-wise packing.

It is believed that other SEI messages with similar ambiguity whether they imply a process or indicate a property may be introduced in the future.

## Overview of the proposal

The following aspects are proposed:

- Addition of a flag, po_sei_process_flag[ i ], to indicate if the i-th type of an SEI message indicates a process or a property.

- When po_sei_process_flag[ i ] is equal to 0, it indicates a property of the output of the previous process that is associated with the same processing order value.

- Clarification that when the region-wise packing SEI message is used as a process, the output of the process is a projected picture as specified by the region-wise packing SEI message.

## Proposed specification text

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_id | u(8) |
| po_num_sei_messages_minus2 | u(8) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++ ) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| po_sei_process_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(12 13) |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| for( i = 0; i < po_num_sei_messages_minus2 + 2; i++ ) |  |
| if( po_sei_prefix_flag[ i ] ) { |  |
| po_num_bits_in_prefix_indication_minus1[ i ] | u(8) |
| for( j = 0; j <= po_num_bits_in_prefix_indication_minus1[ i ]; j++ ) |  |
| po_sei_prefix_data_bit[ i ][ j ] | u(1) |
| while( !byte_aligned( ) ) |  |
| po_byte_alignment_bit_equal_to_one /* equal to 1 */ | f(1) |
| } |  |
| } |  |

po_sei_process_flag[ i ] equal to 1 specifies that the i-th type of an SEI message indicates a process intended to be performed as a part of the processing chain specified by this SEI message. po_sei_process_flag[ i ] equal to 0 specifies that the i-th type of an SEI message indicates a property.

When po_sei_process_flag[ i ] is equal to 0, the following applies:

–	It is a requirement of bitstream conformance that there shall be at least one value of j less than i such that po_sei_process_flag[ j ] is equal to 1 and po_processing_order[ j ] is equal to po_processing_order[ i ]

–	The i-th type of an SEI message characterizes the result of the process indicated by the k-th type of an SEI message for the greatest value of k less than i such that po_sei_process_flag[ k ] is equal to 1 and po_processing_order[ k ] is equal to po_processing_order[ i ].

When po_sei_importance_flag[ i ] is equal to 1, po_sei_process_flag[ i ] shall be equal to 1.

…

When po_sei_payload_type[ i ] is equal to 155 and po_sei_process_flag[ i ] is equal to 1, the output of the process to be performed is a projected picture as specified by the region-wise packing SEI message.

# Indicating between alternative processing steps and cascading in any order

## Problem statement

JVET-AG2027 specifies the following:

po_sei_processing_order[ m ] equal to po_sei_processing_order[ n ] indicates that there is no preferred order of processing between the types of SEI messages associated with indexes m and n (e.g., they can indicate different properties that are both applicable at that stage, or alternative processes that can be applied, or one can indicate a property and the other can indicate a process).

If both loop entries m and n imply processing (rather than properties), the semantics in JVET-AG2027 allows the decoder to perform any of the following:

- Cascade processes m and n in any order:

  - process m  process n

  - process n  process m

- Selection of either process m or n:

  - process m only

  - process n only

Case 1 (cascade processes m and n in any order) is applicable for two neural-network post-filters (NNPFs) that complement each other but do not require a specific order, for example a quality enhancement NNPF and a picture rate upsampling NNPF.

Case 2 (selection of either process m or n) is applicable for example in the following cases:

- Two NNPFs that have the same purpose but different characteristics, such as different complexity.

- A colour transform information SEI message and NNPFC and/or NNPFA SEI messages are associated with the same processing order value, and the NNPF performs a colour transform as a part of its operation, indicated by nnpfc_separate_colour_description_present_flag equal to 1.

It is asserted that without an indication in the SPO SEI message syntax, the decoder might not be able to conclude whether the encoder intended case 1 or case 2 to be applied.

## Overview of the proposal

This proposal item has the following ingredients:

- It is proposed to add a flag (po_sei_alternative_flag[ i ]) that enables the encoder to indicate between cascade of processes in any order and selection among the alternative processes with the same processing order value.

- The semantics of po_sei_alternative_flag[ i ] refer to the semantics of po_sei_processing_order[ i ]. Consequently, po_sei_processing_order[ i ] is proposed to be moved up in the syntax so that it appears before po_sei_alternative_flag[ i ].

- It is proposed that all the types of SEI messages within an SPO SEI message that indicate processes (rather than properties) and have the same processing order value shall either be alternatives to each other or be processes in any order. This is asserted to simplify the proposed semantics.

- Depending on adoption of item 2, two options are proposed to identify alternative processes among the types of SEI messages that have the same processing order value:

  - If item 2 is adopted, the alternative processes have po_sei_process_flag[ i] equal to 1.

  - If item 2 is not adopted, it is proposed that alternative processes have subsequent indices in the SPO SEI message. This is asserted to simplify the semantics and enable the decoding system to identify processes from properties even if the decoding system were unaware of some SEI message payload type values. However, it is not possible to provide SEI messages to indicate properties for the alternative processess individually.

## Proposed specification text changes (option 1)

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_id | u(8) |
| po_num_sei_messages_minus2 | u(8) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++ ) { |  |
| po_sei_processing_order[ i ] | u(8) |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(11 13) |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_process_flag[ i ] | u(1) |
| po_sei_alternative_flag[ i ] | u(1) |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| for( i = 0; i < po_num_sei_messages_minus2 + 2; i++ ) |  |
| if( po_sei_prefix_flag[ i ] ) { |  |
| po_num_bits_in_prefix_indication_minus1[ i ] | u(8) |
| for( j = 0; j <= po_num_bits_in_prefix_indication_minus1[ i ]; j++ ) |  |
| po_sei_prefix_data_bit[ i ][ j ] | u(1) |
| while( !byte_aligned( ) ) |  |
| po_byte_alignment_bit_equal_to_one /* equal to 1 */ | f(1) |
| } |  |
| } |  |

po_sei_alternative_flag[ i ] equal to 1 for i greater than 0 specifies that the processing of the i-th SEI message type is an alternative to the processing of the j-th SEI message type where the value of j is the greatest value less than i for which po_sei_processing_order[ j ] is equal to po_sei_processing_order[ i ] and po_sei_process_flag[ j ] is equal to 1. po_sei_alternative_flag[ i ] equal to 0 for i greater than 0 specifies that the processing of the i-th SEI message type is not an alternative to the j-th SEI message type for any value of j in the range of 0 to i − 1, inclusive. The semantics of po_sei_alternative_flag[ 0 ] are unspecified.

When po_sei_process_flag[ i ] is equal to 0, po_sei_alternative_flag[ i ] shall be equal to 0.

When poIdx is set equal to po_processing_order[ i ], a list of values procList[ poIdx ] consists of the values of j in increasing order such that po_sei_processing_order[ j ] is equal to poIdx and po_sei_process_flag[ j ] is equal to 1, and maxProcIdx is set equal to the count of entries in procList[ poIdx ] minus 1, the following applies:

–	po_sei_alternative_flag[ procList[ poIdx ][ 0 ] ] shall be equal to 0.

–	When po_sei_alternative_flag[ procList[ poIdx ][ k ] ] is equal to 1 for any value of k in the range of 0 to maxProcIdx, inclusive, po_sei_alternative_flag[ procList[ poIdx ][ k ] ] shall be equal to 1 for all values of k in the range of 1 to maxProcIdx, inclusive.

–	When po_sei_alternative_flag[ procList[ poIdx ][ k ] ] is equal to 0 for any value of k in the range of 1 to maxProcIdx, inclusive, the SEI messages types with index equal to procList[ poIdx ][ k ] for all values of k in the range of 0 to maxProcIdx, inclusive, should be processed but there is no preferred order between them.

## Proposed specification text changes (option 2)

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_id | u(8) |
| po_num_sei_messages_minus2 | u(8) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++ ) { |  |
| po_sei_processing_order[ i ] | u(8) |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(12 13) |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_alternative_flag[ i ] | u(1) |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| for( i = 0; i < po_num_sei_messages_minus2 + 2; i++ ) |  |
| if( po_sei_prefix_flag[ i ] ) { |  |
| po_num_bits_in_prefix_indication_minus1[ i ] | u(8) |
| for( j = 0; j <= po_num_bits_in_prefix_indication_minus1[ i ]; j++ ) |  |
| po_sei_prefix_data_bit[ i ][ j ] | u(1) |
| while( !byte_aligned( ) ) |  |
| po_byte_alignment_bit_equal_to_one /* equal to 1 */ | f(1) |
| } |  |
| } |  |

po_sei_alternative_flag[ i ] equal to 1 specifies that the processing of the i-th SEI message type is an alternative to the processing of the (i − 1)-th SEI message type. po_sei_alternative_flag[ i ] equal to 0 specifies that the processing of the i-th SEI message type is not an alternative to the processing of the (i − 1)-th SEI message type. When i is equal to 0 or po_sei_processing_order[ i ] is not equal to po_sei_processing_order[ i − 1 ], po_sei_alternative_flag[ i ] shall be equal to 0. When i is greater than 0, po_sei_processing_order[ i ] is equal to po_sei_processing_order[ i − 1 ], and po_sei_alternative_flag[ i ] is equal to 0, both the i-th and ( i − 1 )-th SEI message types should be processed but there is no preferred order between them. When po_sei_alternative_flag[ i ] is equal to 1, the i-th SEI message type and the (i − 1)-th SEI message type shall indicate processes.

# Using extensible SEI payload type

## Problem statement

As of today, the SEI payload type values are not larger than 8 bits. The fixed-length coding of the SEI payload type value as u(13) therefore causes the use of 5 unused bits with the current SEI payload type values. It is asserted that these 5 bits could be used for other information for the i-th type of an SEI message or reserved for metadata extensions for the i-th type of an SEI message.

The SEI message syntax uses a byte equal to 0xFF as an escape code to add another SEI message payload type value byte.

## Overview of the proposal

It is proposed to adopt the mechanism used in the SEI message syntax to represent the payload type value in the SPO SEI message. The reserved zero bits that result from this change are proposed to be specified in a way that allows future extensions.

## Proposed specification text

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_id | u(8) |
| po_num_sei_messages_minus2 | u(8) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++ ) { |  |
| poSeiPayloadType[ i ] = 0 |  |
| do { |  |
| po_sei_payload_type_byte | u(8) |
| poSeiPayloadType[ i ] += po_sei_payload_type_byte |  |
| } while( po_sei_payload_type_byte = = 0xFF ) |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(13) |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_reserved_zero_5bits[ i ] | u(5) |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| for( i = 0; i < po_num_sei_messages_minus2 + 2; i++ ) |  |
| if( po_sei_prefix_flag[ i ] ) { |  |
| po_num_bits_in_prefix_indication_minus1[ i ] | u(8) |
| for( j = 0; j <= po_num_bits_in_prefix_indication_minus1[ i ]; j++ ) |  |
| po_sei_prefix_data_bit[ i ][ j ] | u(1) |
| while( !byte_aligned( ) ) |  |
| po_byte_alignment_bit_equal_to_one /* equal to 1 */ | f(1) |
| } |  |
| } |  |

po_sei_payload_type_byte is a byte of the payload type of an SEI message.

po_sei_payload_type[ i ] specifies the payloadType value of the i-th type of SEI message.

In the semantics of SPO SEI and PON SEI message, replace po_sei_payload_type[ i ] with poSeiPayloadType[ i ].

po_sei_reserved_zero_5bits[ i ] shall be equal to 0. Values greater than 0 for po_sei_reserved_zero_5bits[ i ] are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore the value of po_sei_reserved_zero_5bits[ i ].

# Use of the colour transform information SEI message as a part of a processing chain

## Problem statement

The colour transform information uses an interface variable ChromaFormatIdc, which may depend on earlier processing steps in the processing chain.

## Overview of the proposal

It is proposed to fix the ChromaFormatIdc derivation for the use of the colour transform information SEI message as a part of a processing chain in a similar manner as done for the film grain characteristics SEI message in JVET-AG2027.

## Proposed specification text

### D.xx.9	Use of the colour transform information SEI message

For purposes of interpretation of the colour transform information SEI message, the following variable ChromaFormatIdc is specified as follows:

–	If the colour transform information SEI message is not applied as part of a processing chain indicated by an SEI processing order SEI message or is applied as the first processing step of the processing chain indicated by an SEI processing order SEI message, ChromaFormatIdc is set equal to sps_chroma_format_idc.

–	Otherwise (the colour transform information SEI message is applied as the second or a later processing step of the processing chain indicated by an SEI processing order SEI message), prevPic is a picture resulting from the previous processing step of the processing chain indicated by the SEI processing order SEI message, and ChromaFormatIdc is set equal to the picture chroma format indicator of prevPic.

# Use of the NNPFC and NNPFA SEI messages as a part of a processing chain

## Problem statement

The NNPFC SEI messages uses interface variables, which may depend on earlier processing steps in the processing chain.

## Overview of the proposal

It is proposed to fix the interface variable derivation for the use of the NNPFC and NNPFA SEI messages as a part of a processing chain in a similar manner as done for the film grain characteristics SEI message in JVET-AG2027.

The proposed text is asserted to be similar to what has been included in the technologies under consideration (JVET-AG2032) but attempts to have even fewer text changes for the legacy case when NNPFC and NNPFA SEI message are not applied as a part of a processing chain so that it is reportedly easier to verify that there is no technical change relative to VVC v3.

## Proposed specification text

### D.xx.11	Use of the neural network post-filter characteristics SEI message and the neural network post-filter activation SEI message

Let currPic be the cropped decoded output picture for which the neural-network post-processing filter (NNPF) defined by the neural-network post-filter characteristics (NNPFC) SEI message is activated by a neural-network post-filter activation (NNPFA) SEI message, and currLayerId be the nuh_layer_id value of currPic.

It is a requirement of bitstream conformance that when a picture unit contains an NNPFA SEI message, the value of ph_pic_output_flag in the picture header contained in that picture unit shall be equal to 1.

NOTE – Since only cropped decoded output pictures are used as input pictures of the NNPF, the value of ph_pic_output_flag in the picture header of the coded picture corresponding to each input picture of the NNPF is equal to 1.

The variable pictureRateUpsamplingFlag is set equal to ( ( nnpfc_purpose & 0x08 ) > 0 ) ? 1 : 0.

The variable numInputPics is set equal to nnpfc_num_input_pics_minus1 + 1.

The variable numInferences is derived as follows:

–	If all of the following conditions are true, the variable numPostRoll is set equal to the value of i such that nnpfc_interpolated_pics[ i ] is greater than 0 and the variable numInferences is set equal to 1 + numPostRoll:

–	nnpfc_purpose is equal to 8 (i.e., the only purpose for the NNPF is picture rate upsampling).

–	nnpfa_persistence_flag is equal to 1.

–	nnpfc_interpolated_pics[ i ] is greater than 0 only for a single value of i that is greater than 0.

–	Either of the following conditions is true:

–	currPic is the last picture of the bitstream in output order that has nuh_layer_id equal to currLayerId.

–	currPic is the last picture in the CLVS in output order and nnpfa_no_foll_clvs_flag is equal to 1.

–	Otherwise, if all of the following conditions are true, the variable numPostRoll is set equal to InpIdx[ i ] for the value of i such that nnpfa_output_flag[ i ] is equal to 1, and the variable numInferences is set equal to 1 + numPostRoll:

–	pictureRateUpsamplingFlag is equal to 0.

–	numInputPics is greater than 1.

–	nnpfa_persistence_flag is equal to 1.

–	nnpfa_output_flag[ idx ] is equal to 1 for a single value of idx in the range of 0 to NumInpPicsInOutputTensor − 1, inclusive, and for that single value of idx, InpIdx[ idx ] is greater than 0.

–	Either of the following conditions is true:

–	currPic is the last picture of the bitstream in output order that has nuh_layer_id equal to currLayerId.

–	currPic is the last picture in the CLVS in output order and nnpfa_no_foll_clvs_flag is equal to 1.

–	Otherwise, the variable numInferences is set equal to 1.

The list candInputPicList, which is a list of pictures from which the input pictures to the NNPF process are selected, is derived as follows:

–	If the NNPFA SEI message is not applied as part of a processing chain indicated by an SEI processing order SEI message or is applied as the first processing step of the processing chain indicated by an SEI processing order SEI message, candInputPicList is the list of those cropped decoded pictures of the bitstream in output order that belong to the current layer.

–	Otherwise (the NNPFA SEI message is applied as the second or a later processing step of the processing chain indicated by an SEI processing order SEI message), candInputPicList is derived as follows:

–	candInputPicList is initially the list of the cropped decoded pictures of the bitstream in output order that belong to the current layer

–	For each processing step in non-decreasing po_processing_order[ i ] values, candInputPicList is updated by replacing pictures with the corresponding processed pictures, if any, resulting from the processing step and inserting the other pictures, if any, resulting from the processing step in candInputPicList so that the output order is obeyed.

–	It is a requirement of bitstream conformance that within candInputPicList there shall be no more than one picture pertaining to any particular output time instance.

Let currCandPic be the picture in candInputPicList that corresponds to currPic, i.e., has the same output time as currPic.

For each value of j in the range of 0 to numInferences − 1, inclusive, the following applies:

–	The arrays inputPic[ i ] and inputPresentFlag[ i ] for i in the range of 0 to numInputPics − 1, inclusive, representing all the input pictures and the presence of input pictures, respectively, are specified as follows:

–	When j is greater than 0, for each value of k in the range of 0 to j − 1, inclusive, inputPic[ k ] is set to be currCandPic and inputPresentFlag[ k ] is set equal to 0.

–	The j-th input picture, inputPic[ j ], is set to be currCandPic and inputPresentFlag[ j ] is set equal to 1.

–	When numInputPics is greater than 1, the following applies for each value of i in the range of j + 1 to numInputPics − 1, inclusive, in increasing order of i:

–	If both of the following conditions are true, inputPic[ i ] is set to be prevPic and inputPresentFlag[ i ] is set equal to 1:

–	Either of the following conditions is true:

–	pictureRateUpsamplingFlag is equal to 1 and currPic is associated with a frame packing arrangement SEI message with frame_packing_arrangement_type equal to 5 and a particular value of fp_current_frame_is_frame0_flag, and there is a cropped decoded output picture prevPic that is the last picture in output order among the pictures that are present in candInputPicList all cropped decoded output pictures that have nuh_layer_id equal to currLayerId, precede inputPic[ i − 1 ] in output order, and are associated with a frame packing arrangement SEI message with frame_packing_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag.

–	pictureRateUpsamplingFlag is equal to 0 or currPic is not associated with a frame packing arrangement SEI message with frame_packing_arrangement_type equal to 5, and there is a cropped decoded output picture prevPic that is the last picture in output order among the pictures that are present in candInputPicList all cropped decoded output pictures that have nuh_layer_id equal to currLayerId and precede inputPic[ i − 1 ] in output order.

–	nnpfa_no_prev_clvs_flag is equal to 0 or the coded picture corresponding to prevPic and the current picture are present in the same CLVS.

–	Otherwise, the following applies:

–	inputPic[ i ] is set to be the same picture as inputPic[ i − 1 ] and inputPresentFlag[ i ] is set equal to 0.

–	It is a requirement of bitstream conformance that, when pictureRateUpsamplingFlag is equal to 1, nnpfc_interpolated_pics[ i − 1 ] shall be equal to 0.

–	It is a requirement of bitstream conformance that when inputPresentFlag[ i ] is equal to 0 and nnpfc_input_pic_output_flag[ i ] is equal to 1, the value of nnpfa_output_flag[ idx ] shall be equal to 0 for the value of idx such that InpIdx[ idx ] is equal to i.

–	For purposes of interpretation of the NNPFC SEI message, the following variables are specified:

–	If the NNPFA SEI message is not applied as part of a processing chain indicated by an SEI processing order SEI message, the following applies:

–	If numInputPics is greater than 1 and there is a second NNPF that is defined by at least one NNPFC SEI message, is activated by an NNPFA SEI message for currPic, and has nnpfc_purpose equal to 4, the following applies:

–	CroppedWidth is set equal to nnpfcOutputPicWidth defined for the second NNPF.

–	CroppedHeight is set equal to nnpfcOutputPicHeight defined for the second NNPF.

–	Otherwise, the following applies:

–	CroppedWidth is set equal to the value of pps_pic_width_in_luma_samples − ‌SubWidthC * ‌ ( pps_conf_win_left_offset + pps_conf_win_right_offset ) for currPic.

–	CroppedHeight is set equal to the value of pps_pic_height_in_luma_samples − ‌SubHeightC * ‌ ( pps_conf_win_top_offset + pps_conf_win_bottom_offset ) for currPic.

–	Otherwise (the NNPFA SEI message is applied as part of a processing chain indicated by an SEI processing order SEI message), the following applies:

–	CroppedWidth is set equal to the width of currCandPic in luma samples.

–	CroppedHeight is set equal to the height of currCandPic in luma samples.

–	The luma sample arrays CroppedYPic[ i ] and the chroma sample arrays CroppedCbPic[ i ] and CroppedCrPic[ i ], when present, are derived as follows for each value of i in the range of 0 to numInputPics − 1, inclusive:

–	The variable sourcePic is derived as follows:

–	If inputPresentFlag[ i ] is equal to 1 or nnpfc_absent_input_pic_zero_flag is equal to 0, sourcePic is set to be inputPic[ i ].

–	Otherwise(inputPresentFlag[ i ] is equal to 0 and nnpfc_absent_input_pic_zero_flag is equal to 1), sourcePic is set to be a picture with a luma sample array of CroppedWidth × CroppedHeight samples equal to 0 and Cb and Cr sample arrays of ( CroppedWidth / SubWidthC ) × ( CroppedHeight / SubHeightC ) samples equal to 0.

–	If numInputPics is equal to 1 or the NNPFA SEI message is applied as part of a processing chain indicated by an SEI processing order SEI message, the following applies:

–	The luma sample array CroppedYPic[ i ] and the chroma sample arrays CroppedCbPic[ i ] and CroppedCrPic[ i ], when present, are set to be the 2-dimensional arrays of decoded sample values of the Y, Cb and Cr components, respectively, of sourcePic.

–	Otherwise (numInputPics is greater than 1), the following applies:

–	The variable sourceWidth is set equal to the value of pps_pic_width_in_luma_samples − ‌SubWidthC * ( pps_conf_win_left_offset + pps_conf_win_right_offset ) for sourcePic.

–	The variable sourceHeight is set equal to the value of pps_pic_height_in_luma_samples − ‌SubHeightC * ( pps_conf_win_top_offset + pps_conf_win_bottom_offset ) for sourcePic.

–	If sourceWidth is equal to CroppedWidth and sourceHeight is equal to CroppedHeight, resampledPic is set to be the same as sourcePic.

–	Otherwise (sourceWidth is not equal to CroppedWidth or sourceHeight is not equal to CroppedHeight), the following applies:

–	There shall be an NNPF, hereafter referred to as the super resolution NNPF, that is defined by at least one NNPFC SEI message, is activated by an NNPFA SEI message for sourcePic, and has nnpfc_purpose equal to 4, nnpfcOutputPicWidth equal to CroppedWidth and nnpfcOutputPicHeight equal to CroppedHeight.

–	resampledPic is set to be the output of the neural-network inference of the super resolution NNPF with sourcePic being an input.

–	The luma sample array CroppedYPic[ i ] and the chroma sample arrays CroppedCbPic[ i ] and CroppedCrPic[ i ], when present, are set to be the 2-dimensional arrays of decoded sample values of the Y, Cb and Cr components, respectively, of resampledPic.

–	If the NNPFA SEI message is not applied as part of a processing chain indicated by an SEI processing order SEI message, the following applies:

–	BitDepthY and BitDepthC are both set equal to BitDepth.

–	ChromaFormatIdc is set equal to sps_chroma_format_idc.

–	Otherwise (the NNPFA SEI message is applied as part of a processing chain indicated by an SEI processing order SEI message), the following applies:

–	BitDepthY and BitDepthC are set equal to the luma bit depth and chroma bit depth of currCandPic, respectively.

–	ChromaFormatIdc is set equal to the picture chroma format indicator of currCandPic.

–	The array StrengthControlVal[ i ] for all values of i in the range of 0 to numInputPics − 1, inclusive, specifying the filtering strength control value for the input pictures for the NNPF, is derived as follows:

–	StrengthControlVal[ i ] is set equal to the value of ( firstSliceQpY + QpBdOffset ) ÷ ( 63 + QpBdOffset ), where firstSliceQpY is equal to SliceQpY of the first slice of the coded picture corresponding to inputPic[ i ].

There shall not be more than two NNPFC SEI messages present in a picture unit with the same value of nnpfc_id. When there are two NNPFC SEI messages present in a picture unit with the same value of nnpfc_id, these SEI messages shall have different content. When two NNPFC SEI messages with the same nnpfc_id and different content are present in the same picture unit, both of these NNPFC SEI messages shall be in the same SEI NAL unit.

# Combined syntax of items 2, 3 and 4

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_id | u(8) |
| po_num_sei_messages_minus2 | u(8) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++ ) { |  |
| po_sei_processing_order[ i ] | u(8) |
| poSeiPayloadType[ i ] = 0 |  |
| do { |  |
| po_sei_payload_type_byte | u(8) |
| poSeiPayloadType[ i ] += po_sei_payload_type_byte |  |
| } while( po_sei_payload_type_byte = = 0xFF ) |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(13) |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_process_flag[ i ] | u(1) |
| po_sei_alternative_flag[ i ] | u(1) |
| po_sei_reserved_zero_3bits[ i ] | u(3) |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| for( i = 0; i < po_num_sei_messages_minus2 + 2; i++ ) |  |
| if( po_sei_prefix_flag[ i ] ) { |  |
| po_num_bits_in_prefix_indication_minus1[ i ] | u(8) |
| for( j = 0; j <= po_num_bits_in_prefix_indication_minus1[ i ]; j++ ) |  |
| po_sei_prefix_data_bit[ i ][ j ] | u(1) |
| while( !byte_aligned( ) ) |  |
| po_byte_alignment_bit_equal_to_one /* equal to 1 */ | f(1) |
| } |  |
| } |  |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).