| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0137-v1 |
| --- | --- |

| Title: | AHG9: Indicating timing information for generative face video (GFV) output pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen Olena Chubach Yu-Wen Huang Shawmin Lei | Email: | {lulin.chen, olena.chubach, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes changes to JVET-AG2032 for indicating timing information for the generative face video output pictures generated from the GFV SEI messages.

# Introduction

In the last JVET AG meeting, the generative face video (GFV) SEI message and related improvements from JVET-AG0203 and JVET-AG0087 were agreed and included in JVET-AG2032 - Technologies under consideration for future extensions of VSEI (version 3). One of those improvements is the addition of the syntax element gfv_cnt that specifies a GFV SEI message instance count value for a gfv_id value within a picture unit (PU) in that multiple GFV SEI messages may be contained within a PU. For a particular value of gfv_id, the generating process is specified in increasing order of value of gfv_cnt to generate a video picture per each GFV SEI message that has gfv_base_pic_flag equal to 0 and a unique value of gfv_cnt within a PU.

As commented in principle and for further study in the meeting notes of the last JVET AG meeting, timing information is needed for those GFV output picture generated from the GFV SEI messages with the values of gfv_cnt greater than 0.

# Proposal

This contribution proposes changes to JVET-AG2032 for indicating timing information for GFV output pictures generated from each GFV SEI message, for a particular value of gfv_id, with a unique value of gfv_cnt greater than 0 within a PU.

In general, a PU may be in one of three cases as illustrated in Figure 1.

Figure 1. Three PU cases as PU 1, PU 2, and PU 3

PU 1 contains one or more GFV SEI messages of

The 1st GFV SEI message with gfv_cnt = 0, gfv_base_pic_flag = 1, and optionally

The 2nd GFV SEI messages with gfv_cnt > 0, gfv_base_pic_flag = 0, gfv_drive_pic_fusion_flag = 0.

PU 2 contains one or more GFV messages of

The 1st GFV SEI message with gfv_cnt = 0, gfv_base_pic_flag = 0, gfv_drive_pic_fusion_flag = 1, and optionally

The 2nd GFV SEI messages with gfv_cnt > 0, gfv_base_pic_flag = 0, gfv_drive_pic_fusion_flag = 0.

PU 3 contains one or more GFV messages of

The 1st GFV SEI message with gfv_cnt = 0, gfv_base_pic_flag = 0, gfv_drive_pic_fusion_flag = 0, and optionally

The 2nd GFV SEI messages with gfv_cnt > 0, gfv_base_pic_flag = 0, gfv_drive_pic_fusion_flag = 0.

For a low delay application, a PU may contain a coded picture and a single GFV SEI message with the value of gfv_cnt equal to 0. The timing of the generated output picture is the same as the timing of the decoded picture within PU.

In the above three cases, timing information is needed for GFV output pictures generated from those second GFV SEI messages in the current PU. The proposed new syntax elements and semantics over JVET-AG2032 are added below as highlighted in green.

8.37 Generative face video SEI message

8.37.1 Generative face video SEI message syntax

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| gfv_cnt | ue(v) |
| if ( gfv_cnt ) { \\add timing info |  |
| gfv_timing_extrapolates_pic_timing_flag | u(1) |
| if( !gfv_timing_extrapolates_pic_timing_flag ) { |  |
| gfv_time_scale | u(32) |
| gfv_num_units_in_pic_interval | u(32) |
| } |  |
| } |  |
| … |  |

8.37.1 Generative face video SEI message semantics

…

gfv_cnt specifies a GFV SEI message instance count value for this gfv_id value within a picture unit.

…

For a low delay application, a PU may contain a coded picture and a single GFV SEI message with the value of gfv_cnt equal to 0 within a PU.

gfv_timing_extrapolates_pic_timing_flag equal to 1, for a particular value of gfv_id, indicates that the timing of generated GFV output picture is extrapolated from and based on the timing of the decoded picture within the PU that contains the current GFV SEI message. gfv_timing_extrapolates_pic_timing_flag equal to 0 indicates the timing of generated GFV output picture may not be extrapolated from and based on the timing of the decoded picture within the PU that contains the current GFV SEI message.

NOTE 1 - The decoded picture may correspond to a base picture, a driving picture that may be used for fusion, or a driving picture that may not be used for fusion.

gfv_time_scale is the number of time units that pass in one second. For example, a time coordinate system that measures time using a 27 MHz clock has a time_scale of 27 000 000. The value of time_scale shall be greater than 0.

gfv_num_units_in_pic_interval, when present, specifies the number of time units of a clock operating at the frequency gfv_time_scale Hz that corresponds to the indicated GFV output picture interval for the generated GFV output pictures in output order in the CVS.

The indicated GFV output picture interval, denoted by the variable GfvOpPicInterval, in units of seconds, is equal to the quotient of gfv_num_units_in_pic_interval divided by gfv_time_scale. For example, for a 25 Hz gfv video sequence, gfv_time_scale and gfv_num_units_in_pic_interval could be equal to 27 000 000 and 1 080 000, respectively, and consequently a GFV output picture interval would be equal to 0.04 seconds.

When gfv_timing_extrapolates_timing_flag is equal to 1 and a picture timing SEI message is present for the decoded picture, The generated GFV output picture timing could be determined from information conveyed in the picture timing SEI message. As the extrapolation, the value of GfvOpPicInterval is equal to that of the picture interval for the decoded picture in the current PU.

The GFV output picture intervals associated with GFV output pictures generated from GFV SEI messages with a particular value of gfv_id and the values of gfv_cnt greater than 0, denoted by the variable GfvOpPicIntervals[ i ][ j ], in units of seconds, for i equal to gfv_id, j less than or equal to gfv_cnt, is derived as follows:

GfvOpPicIntervals[ i ][ j ] = GfvOpPicInterval * j				8-X

…

For a particular gfv_id value, the following process is used in increasing order of value of gfv_cnt to generate a video picture per each GFV SEI message that has gfv_base_pic_flag equal to 0 and a unique value of gfv_cnt within a picture unit:

…

In the above process, timing of the generated GFV output picture per each GFV SEI message with a particular value of gfv_id equal to i and a particular value of gfv_cnt equal to j, j is greater than 0, is indicated by the GFV output picture intervals, i.e., GfvOpPicIntervals[ i ][ j ] of 8-X, which indicates the temporal distances, for i and j, in units of seconds, from the decoded picture in the current PU.

# References

- “Technologies under consideration for future extensions of VSEI (version 3)”, JVET-AG2032, JVET 33rd (AG) meeting, January 2024, online.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).