| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0142 |
| --- | --- |

| Title: | AHG4: HDR 4K content for JVET CTCs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | E. François, P. de Lagrange, S. Puri, D. Doyen | Tel: Email: | edouard.francois@interdigital.com philippe.delagrange@interdigital.com saurabh.puri@interdigital.com didier.doyen@interdigital.com |  |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes to consider including in the HDR common test conditions (CTCs) 4K HDR PQ content. 6 HDR 4K clips are proposed, to be added to or to partly replace the current HDR PQ test set that only comprises 7 clips of 1080p or lower resolution. Adding 4K HDR PQ content is asserted to be more in line with current commercial deployment of HDR. It is proposed to add 4 4K clips among the 6 proposed ones, and to reduce the HD HDR PQ set to 4 or 5 clips.

AI and RA coding results are reported for HM18.0 versus VTM23.0, and VTM11_ECM12 versus  ECM12.0 in tables below.

HM18.0 versus VTM23.0

|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| AI | -41.29% | -23.76% | -23.37% | -47.48% | -68.42% | -22.64% | -44.28% | -67.12% | 632% | 88% |
| RA | -47.28% | -37.83% | -37.13% | -52.33% | -65.70% | -36.14% | -50.21% | -64.45% | 181% | 96% |

VTM11_ECM12 versus ECM12.0

|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| AI | -30.74% | -12.58% | -11.65% | -43.98% | -55.31% | -11.60% | -43.23% | -53.95% | 971% | 537% |
| RA | -43.66% | -21.60% | -20.14% | -57.15% | -69.51% | -20.13% | -56.36% | -68.43% | 925% | 949% |

# Introduction

The HDR PQ content from the current JVET HDR CTCs (JVET-AC2011) is made of 7 clips of 1080p (HD) or lower resolution. Considering that HDR 4K is more in line with current commercial deployment of HDR, it is proposed to consider updating the HDR PQ test set by adding 4K clips. Some of the 7 HD clips could be removed.

# HDR PQ 4K clips description

The following 6 clips are proposed for consideration. They have been selected from JVET or 3GPP SA4 content repositories. It is suggested to consider 300 frames for each clip. Details on copyright, access, md5sum are given in Annex.

All clips have in common the following parameters:

- Resolution 	3840 x 2160

- Scan 		progressive

- Frame Rate 	60

- Bit Depth 	10

- YUV format 	4:2:0

- Colour params 	ITU-T BT.2100

| Clip name | Original location | snapshot |
| --- | --- | --- |
| Chimera7 | jvet@ftp.ient.rwth-aachen.de |  |
| Meridian1 | jvet@ftp.ient.rwth-aachen.de |  |
| Meridian2 | jvet@ftp.ient.rwth-aachen.de |  |
| Life Untouched | https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences |  |
| Meridian | https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences |  |
| Nocturne | https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences |  |

# Experiments with the proposed 4K HDR PQ clips

The considered clip length for all clips is 300 frames. Encoding was performed with HM18.0, VTM23.0, VTM11_ECM12, ECM12.0. AI and RA configurations were used.

The full results are attached to the contribution.

## HM18.0 versus VTM23.0

|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| AI | -41.29% | -23.76% | -23.37% | -47.48% | -68.42% | -22.64% | -44.28% | -67.12% | 632% | 88% |
| RA | -47.28% | -37.83% | -37.13% | -52.33% | -65.70% | -36.14% | -50.21% | -64.45% | 181% | 96% |

Per clip AI

|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| --- | --- | --- | --- | --- | --- |
| ChimeraHDR7_3840x2160 | -36.93% | -29.58% | -28.07% | -35.12% | -50.40% |
| MeridianHDR1_3840x2160 | -53.64% | -31.62% | -30.92% | -62.26% | -73.33% |
| MeridianHDR2_3840x2160 | -32.25% | -17.50% | -17.82% | -42.91% | -70.44% |
| life_untouched | -47.35% | -22.28% | -21.46% | -32.50% | -66.59% |
| meridian_22412-22738 | -45.35% | -19.27% | -19.67% | -70.21% | -88.30% |
| nocturne | -32.22% | -22.28% | -22.29% | -41.91% | -61.46% |
| overall | -41.29% | -23.76% | -23.37% | -47.48% | -68.42% |

Per clip RA

|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| --- | --- | --- | --- | --- | --- |
| ChimeraHDR7_3840x2160 | -44.85% | -41.66% | -40.88% | -44.34% | -51.00% |
| MeridianHDR1_3840x2160 | -47.99% | -42.08% | -41.23% | -52.64% | -53.31% |
| MeridianHDR2_3840x2160 | -49.17% | -46.28% | -45.56% | -44.42% | -66.13% |
| life_untouched | -55.86% | -36.10% | -33.94% | -44.18% | -68.27% |
| meridian_22412-22738 | -39.32% | -28.14% | -28.79% | -53.66% | -87.12% |
| nocturne | -46.49% | -32.70% | -32.39% | -74.73% | -68.38% |
| overall | -47.28% | -37.83% | -37.13% | -52.33% | -65.70% |

## VTM11_ECM12 versus ECM12.0

|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| AI | -30.74% | -12.58% | -11.65% | -43.98% | -55.31% | -11.60% | -43.23% | -53.95% | 971% | 537% |
| RA | -43.66% | -21.60% | -20.14% | -57.15% | -69.51% | -20.13% | -56.36% | -68.43% | 925% | 949% |

Per clip AI

|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| --- | --- | --- | --- | --- | --- |
| ChimeraHDR7_3840x2160 | -26.66% | -14.00% | -12.40% | -26.81% | -36.65% |
| MeridianHDR1_3840x2160 | -44.46% | -19.33% | -17.93% | -58.76% | -63.20% |
| MeridianHDR2_3840x2160 | -23.39% | -7.67% | -7.39% | -32.37% | -55.73% |
| life_untouched | -39.56% | -10.91% | -10.06% | -39.94% | -52.93% |
| meridian_22412-22738 | -18.31% | -7.12% | -7.16% | -55.21% | -63.99% |
| nocturne | -32.07% | -16.42% | -14.96% | -50.78% | -59.36% |
| overall | -30.74% | -12.58% | -11.65% | -43.98% | -55.31% |

Per clip RA

|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| --- | --- | --- | --- | --- | --- |
| ChimeraHDR7_3840x2160 | -39.23% | -25.46% | -24.30% | -39.81% | -48.15% |
| MeridianHDR1_3840x2160 | -58.91% | -25.64% | -23.76% | -70.85% | -77.71% |
| MeridianHDR2_3840x2160 | -41.92% | -26.55% | -25.53% | -52.47% | -66.92% |
| life_untouched | -49.25% | -18.09% | -16.74% | -51.62% | -62.60% |
| meridian_22412-22738 | -38.50% | -13.59% | -12.00% | -77.93% | -94.66% |
| nocturne | -34.18% | -20.27% | -18.48% | -50.24% | -67.04% |
| overall | -43.66% | -21.60% | -20.14% | -57.15% | -69.51% |

## RD-plots for RA configuration

wPSNR-Y RD-plots are depicted in figures below, for HM18.0, VTM23.0 and ECM12.0, RA configuration.

The following observations can be made:

- MeridianHDR1 has the lowest coding complexity (HM18.0 rate around 3.5Mbps for QP 22).

- ChimeraHDR7, MeridianHDR2 are relatively easy to code (HM18.0 rate around 5-6Mbps for QP 22).

- life_untouched is of medium coding complexity (HM18.0 rate around 10Mbps for QP 22).

- meridian_22412-22738, nocturne have the highest coding complexity (HM18.0 rate above 20Mbps for QP 22).

# Results with the current CTC HDR PQ clips (Class H1)

As a reference, below are the results for the current HDR PQ CTCs clips (class H1). It may be observed that the performance for the new 4K PQ content is of same range as for current CTCs HD PQ content.

## HM18.0 versus VTM23.0

|  | Over HM18.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| AI | -41.45% | -27.31% | -26.79% | -57.74% | -52.79% | -24.00% | -52.93% | -45.13% | 1537% | 117% |
| RA | -39.50% | -37.55% | -36.66% | -54.93% | -48.46% | -33.67% | -49.42% | -40.87% | 288% | 101% |

## VTM11_ECM12 versus ECM12.0

|  | Over VTM-11.0ecm12.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| AI | -28.95% | -11.88% | -11.02% | -35.74% | -41.44% | -11.04% | -34.39% | -40.12% | 888% | 468% |
| RA | -30.15% | -12.64% | -17.74% | -43.42% | -48.91% | -17.74% | -41.92% | -47.12% | 834% | 772% |

# Comments

One suggestion is to pick up 4 4K HDR PQ clips among the 6 ones considered in this contribution, covering the full coding complexity range, from the lowest to the highest ones. For example, MeridianHDR1,  MeridianHDR2, life_untouched, Nocturne.

It is also suggested to remove some of the following current HD clips: BalloonFestival, EBU_Starting, SunRise (for which we find these clips provide lower HDR experience than the other clips), Cosmos1_TreeTrunk (that shows weird chroma samples statistics, and in addition is not HD but 1920x896 resolution).

# References

JVET-AC2011 A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, VTM and HM common test conditions and evaluation procedures for HDR/WCG video, Jan 2023

# Patent rights declaration(s)

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.

# Annex: detailed description of the clips

Chimera7 from jvet@ftp.ient.rwth-aachen.de

Cf contribution JVET-D0083	m39086	Netflix HDR and SDR test sequences (AHG4 and AHG7 related), Andrey Norkin, Anne Aaron (Netflix)

| Parameter | Value |
| --- | --- |
| Resolution | 3840 x 2160 |
| Scan | progressive |
| Frame Rate | 60/1.001 |
| Bit Depth | 10 |
| Length | 300 |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT.2020 |
| Colour space | HDR PQ |

Copyright ©

The proposed sequences will be given the same license as their 4K versions, namely the Creative Commons Attribution-NonCommercial-NoDerivatives 4.0 International  License. To view a copy of this license, visit http://creativecommons.org/licenses/by-nc-nd/4.0/. The license also allows hosting the sequences at another site, provided that the license terms (http://creativecommons.org/licenses/by-nc-nd/4.0/) are fulfilled.

Access	jvet@ftp.ient.rwth-aachen.de

folder /ahg/candidates_hdr/ChimeraHDR

Conversion

The original sequence is in tiff format, 4096x2160, P3

It is converted to YUV 3840x2160 10b 2020 using HDRTools19.1 by the following steps:

- Convert from Tiff to YUV 4096x2160 10b 2020

HDRConvert -f HDRConvertP3D65TiffToYCbCr420.cfg

-p SourceFile="./tiff/Chimera_DCI4k5994p_HDR_P3PQ_0%05d.tif"

-p StartFrame=092156 -p NumberOfFrames=300 -p OutputFile=convert.yuv

- Cropped to YUV 3840x2160 10b 2020

HDRConvert -f crop_4096x2160_to_3840x2160.cfg

-p SourceFile=convert.yuv'

-p OutputFile='ChimeraHDR7_3840x2160_60p_10b_P3_ct2020_420.yuv'

-p SourceWidth=4096 -p SourceHeight=2160

-p OutputWidth=3840 -p OutputHeight=2160 -p NumberOfFrames=300

Md5sum of converted YUV 10b clip: 	3bbe6a5790ad6aed6995420a1473b3ae  ChimeraHDR7_3840x2160_60p_10b_P3_ct2020_420.yuv

Meridian1 from jvet@ftp.ient.rwth-aachen.de

Cf contribution JVET-D0083	m39086	Netflix HDR and SDR test sequences (AHG4 and AHG7 related), Andrey Norkin, Anne Aaron (Netflix)

| Parameter | Value |
| --- | --- |
| Resolution | 3840 x 2160 |
| Scan | progressive |
| Frame Rate | 60/1.001 |
| Bit Depth | 10 |
| Length | 300 |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT.2020 |
| Colour space | HDR PQ |

Copyright ©

The proposed sequences will be given the same license as their 4K versions, namely the Creative Commons Attribution-NonCommercial-NoDerivatives 4.0 International  License. To view a copy of this license, visit http://creativecommons.org/licenses/by-nc-nd/4.0/. The license also allows hosting the sequences at another site, provided that the license terms (http://creativecommons.org/licenses/by-nc-nd/4.0/) are fulfilled.

Access	jvet@ftp.ient.rwth-aachen.de

folder /ahg/candidates_hdr/MeridianHDR

Conversion

The original sequence is in tiff format, 4096x2160, P3

It is converted to YUV 3840x2160 10b 2020 using HDRTools19.1 by the following steps:

- Convert from Tiff to YUV 4096x2160 10b 2020

HDRConvert -f HDRConvertP3D65TiffToYCbCr420.cfg

-p SourceFile="./tiff/Meridian_UHD4k5994p_HDR_P3PQ_%05d.tif"

-p StartFrame=1782 -p NumberOfFrames=300 -p OutputFile=convert.yuv

- Cropped to YUV 3840x2160 10b 2020

HDRConvert -f crop_4096x2160_to_3840x2160.cfg

-p SourceFile=convert.yuv'

-p OutputFile='MeridianHDR1_3840x2160_60p_10b_P3_ct2020_420.yuv'

-p SourceWidth=4096 -p SourceHeight=2160

-p OutputWidth=3840 -p OutputHeight=2160 -p NumberOfFrames=300

Md5sum of converted YUV 10b clip:	063c3f98b5ef94c9e25a9f89dac16bd1  MeridianHDR1_3840x2160_60p_10b_P3_ct2020_420.yuv

Meridian2 from jvet@ftp.ient.rwth-aachen.de

Cf contribution JVET-D0083	m39086	Netflix HDR and SDR test sequences (AHG4 and AHG7 related), Andrey Norkin, Anne Aaron (Netflix)

| Parameter | Value |
| --- | --- |
| Resolution | 3840 x 2160 |
| Scan | progressive |
| Frame Rate | 60/1.001 |
| Bit Depth | 10 |
| Length | 300 |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT.2020 |
| Colour space | HDR PQ |

Copyright ©

The proposed sequences will be given the same license as their 4K versions, namely the Creative Commons Attribution-NonCommercial-NoDerivatives 4.0 International  License. To view a copy of this license, visit http://creativecommons.org/licenses/by-nc-nd/4.0/. The license also allows hosting the sequences at another site, provided that the license terms (http://creativecommons.org/licenses/by-nc-nd/4.0/) are fulfilled.

Access	jvet@ftp.ient.rwth-aachen.de

folder /ahg/candidates_hdr/MeridianHDR

Conversion

The original sequence is in tiff format, 4096x2160, P3

It is converted to YUV 3840x2160 10b 2020 using HDRTools19.1 by the following steps:

- Convert from Tiff to YUV 4096x2160 10b 2020

HDRConvert -f HDRConvertP3D65TiffToYCbCr420.cfg

-p SourceFile="./tiff/Meridian_UHD4k5994p_HDR_P3PQ_%05d.tif"

-p StartFrame=11872 -p NumberOfFrames=300 -p OutputFile=convert.yuv

- Cropped to YUV 3840x2160 10b 2020

HDRConvert -f crop_4096x2160_to_3840x2160.cfg

-p SourceFile=convert.yuv'

-p OutputFile='MeridianHDR2_3840x2160_60p_10b_P3_ct2020_420.yuv'

-p SourceWidth=4096 -p SourceHeight=2160

-p OutputWidth=3840 -p OutputHeight=2160 -p NumberOfFrames=300

Md5sum of converted YUV 10b clip:	0ff0affb5d3fe8cc1c98991d6584636b  MeridianHDR2_3840x2160_60p_10b_P3_ct2020_420.yuv

3GPP SA4 Life Untouched

| Parameter | Value |
| --- | --- |
| Resolution | 3840 x 2160 |
| Scan | progressive |
| Frame Rate | 60/1.001 |
| Bit Depth | 10 |
| Length | 450 |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT.2020 |
| Colour space | HDR PQ |

Copyright ©

CableLabs LifeUntouched sequence is licensed under a Creative Commons Attribution-NonCommercial-NoDerivs 3.0 Unported License. https://creativecommons.org/licenses/by-nc-nd/3.0/deed.en_US.

Access

https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Life-Untouched/Life-Untouched.json

also available for download at https://media.xiph.org/video/av2/.

Md5sum of YUV 10b clip:	e01292d0db0d74dc931506acb61dbcac  life_untouched_3840x2160_59_94fps_PQ_P3D65_BT2100_10b_420.yuv

3GPP SA4 Meridian

| Resolution | 3840 x 2160 |
| --- | --- |
| Scan | progressive |
| Frame Rate | 60/1.001 |
| Bit Depth | 10 |
| Length | 327 |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT.2020 |
| Colour space | HDR PQ |

Copyright ©

The video sequence provided above and all intellectual property rights therein remain the property of Netflix Inc. This video sequence is licensed under the Creative Commons Attribution-NonCommercial-NoDerivatives 4.0 International License. To view a copy of this license, visit http://creativecommons.org/licenses/by-nc-nd/4.0/

Access

https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Meridian/Meridian.json

also available for download at https://media.xiph.org/video/av2/.

Md5sum of YUV 10b clip:	d055a94f35f7594776186fc5d09a9fa4 
meridian_22412-22738_3840x2160_59_94fps_PQ_P3D65_BT2100_10b_420.yuv

3GPP SA4 Nocturne

| Parameter | Value |
| --- | --- |
| Resolution | 3840 x 2160 |
| Scan | progressive |
| Frame Rate | 60/1.001 |
| Bit Depth | 10 |
| Length | 370 |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT.2020 |
| Colour space | HDR PQ |

Copyright ©

The video sequence provided above and all intellectual property rights therein remain the property of Netflix Inc. This video sequence is licensed under the Creative Commons Attribution-NonCommercial-NoDerivatives 4.0 International License. To view a copy of this license, visit http://creativecommons.org/licenses/by-nc-nd/4.0/

Access

https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/Nocturne/Nocturne.json

also available for download at https://media.xiph.org/video/av2/.

Md5sum of YUV 10b clip:	9b8b72753926f11a7c7fc8545b5c78b7 
nocturne_27740-28109_3840x2160_60fps_PQ_P3D65_BT2100_10b_420.yuv