| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0144-v1 |
| --- | --- |

| Title: | AHG12: Hierarchical Subblock based DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, C. Hollmann | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

DMVR can refine a bi-directional block motion vector on subblocks of size 16x16. Considering that the largest block size in ECM is 256x256, subblocks of size 16x16 are very small and can in some cases have difficulties to find correct refinements of the block motion vectors. This proposal combats the large size difference between block and subblocks by introducing hierarchical subblock-based DMVR. The reference blocks are divided into subblocks of reduced size in each dimension and compared to the block size at each level of the hierarchy until the subblock size reaches the ECM subblock size. Integer DMVR search is performed on each level of the hierarchy on subsampled subblocks of size 16x16. In the last level, besides integer DMVR, also subpixel DMVR search as in ECM-12.0 is performed. The search range is kept the same as in ECM-12.0. The final subblock refinement is biased towards the larger subblock refinements by penalizing refinements that differs from the corresponding larger subblock refinement.

The objective BDR is asserted to be:

-0.02% (Y), -0.01% (U), -0.06% (V) for random access configuration with most impact at 4K resolution. More importantly, it is also asserted that subjective quality can be improved by a more coherent motion field.

# Introduction

In ECM-12.0 the largest block size is 256x256. DMVR is a powerful technology to reduce the signalling of motion information and refines a bi-directional block motion vector on block basis but also on subblocks of 16x16 samples. The large gap between the size of the block and the size of the subblock can make the refinements on subblocks incoherent. This contribution addresses that issue by deploying DMVR search in a hierarchical manner.

# Proposal

When subblock DMVR is applicable and the width or height of the current block is equal to or greater than 64, hierarchical subblock-based DMVR is enabled. The reference areas and search range given by the respective block motion vector of the bi-directional motion vector are of the same size as in ECM.

The current block (e.g., 64x64) is divided into subblocks of half size in each dimension when possible. The corresponding reference subblocks and search areas are down-sampled so that each subblock has a size of 16x16 samples. The integer DMVR search is then applied on the subsampled subblocks to derive a refinement vector for each subblock.

Then the current subblock (e.g., 32x32) is divided into smaller subblocks of half size in each dimension when possible. The corresponding reference subblocks and search area are down-sampled so that each smaller subblock has a size of 16x16 samples. The integer DMVR search is then applied on the subsampled smaller subblocks to derive a refinement vector for each smaller subblock.

When a subblock reaches size 16x16 no further down-sampling is done and integer DMVR is applied, followed by sub-pixel DMVR as in ECM. The final subblock refinement is biased towards the higher level subblock refinements by penalizing refinements that differ from the corresponding larger subblocks motion. This is similar to the current ECM which penalizes refinements from the block motion vector. The refined subblock motion vector for a 16x16 subblock for reference picture A is derived by adding the initial motion vector for reference picture A and the corresponding refinements motion vectors. The corresponding motion vector for reference picture B is derived by taking the difference between the initial motion vector for the reference picture B and the corresponding refinement motion vectors. One example the hierarchical subblock based DMVR is shown for a block size of 64x64 based on bi-directional motion vector (Mv_refA, Mv_refB) in Figure 1.

Figure 1: Hierarchical subblock based DMVR.

# Results

Results have been compared to ECM-12.0 on random access configuration where DMVR is applied. Encoding times are not accurate. Decoding time is accurate and show a 2% increase in decoding time. The code can likely be optimized.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,08% | 0,00% | -0,12% | 91,7% | 102,5% | 100,3% | 99,7% |
| Class A2 | -0,03% | -0,08% | -0,08% | 85,6% | 102,5% | 100,0% | 99,6% |
| Class B | -0,02% | 0,03% | -0,01% | 98,1% | 102,3% | 100,2% | 100,2% |
| Class C | 0,01% | -0,02% | -0,06% | 99,6% | 102,1% | 99,6% | 100,0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0,02% | -0,01% | -0,06% | 94,6% | 102,3% | 100,0% | 99,9% |
| Class D | -0,03% | -0,07% | 0,02% | 99,1% | 102,8% | 100,1% | 100,0% |

# Conclusion

It has been shown that hierarchical subblock based DMVR can provide an objective benefit, especially for higher resolutions. It is suggested to study the method further in next round of EE2.

# References

[1] M. Karczewice, Y. Yan, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Hannover, Germany, October 2023.

[2] ECM-12.0, available at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-12.0

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).