| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0151-v3 |
| --- | --- |

| Title: | EE2-related: IntraTMP merge candidates clustering based on refinement window |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Damian Ruiz Coll, Jung-Kyung Lee, Karam Naser Fabrice Le Léannec Tangi Poirier Hassane Guermoud Thierry Dumas. | Tel: Email: | duizcoll@ofinno.com, jlee@ofinno.com karam.naser@InterDigital.com |
| Source: | Ofinno, InterDigital |  |  |

_____________________________

# Abstract

In IntraTMP mode, extend the sparse candidate list with additional merge candidates acquired from the spatial and non-spatial neighboring PUs. Following this, a refinement stage is carried out using a local search window around each BV candidate in the candidates' list. The refinement window size varies based on the candidate type, whether a merge or a sparse candidate. The refinement window for merge candidates may overlap with other merge candidates and sparse candidates.

This contribution proposes clustering the merge candidates whose refinement windows overlap with other candidates, determining a new refinement window that includes both candidates' local windows. This contribution also extends the merge candidate list with ARBVP candidates based on sparse candidate list The proposed methods have been implemented on top of ECM-11.0, and the subsequent test results are as follows {for Y, U, V, EncT, DecT}:

AI: Overall {-0.09%, -0.06%, -0.08%, 102.5%, 105.0%},

RA: Overall {-0.xx%, -0.xx%, -0.xx%, 10x.x%, 10x.x%}.

# Introduction

IntraTMP mode [1] predicts the current unit (CU) in a constrained search region from the reconstructed samples within the current frame. The best reference block is determined by matching a template neighboring samples of CU in a predefined search range that comprises several adjacent regions (R1 to R6 in Fig. 1). Candidate searching in IntraTMP is performed in a two-step process. First, the spare searching is conducted across all regions utilizing a subsampling factor (SI), and the 30 BV candidates exhibiting the lowest template cost are selected and ranked in ascending order in the sparse list for further refinement.

In addition to the sparse candidates, IntraTMP selects up to 10 merge candidates among the adjacent and non-adjacent PUs encoded using IBC or IntraTMP mode [2]. The merge candidates compete with the sparse candidates and the top 30 candidates are held in the merge-sparse list for the second stage.

Second, the merge and sparse candidates are refined using a local search window around the BVs. The refinement window size for the sparse candidates is ±SI2, consequently, the refinement windows of the sparse candidates never overlap. The refinement window size for the merge BV candidates (BV-M) is 11x11,   and they may point in or out of the IntraTMP search region; thus, the inner merge candidates' refinement window may overlap the sparse candidates' search window. Furthermore, the distance between the BV merge candidates is not constrained, so the merge’s refinement window may overlap.

Figure 1. Example of TMP merge candidates (BV-M)

After the refinement stage, the top 19 candidates among the merge and sparse refined candidates are selected for a "short IntraTMP list." Finally, an IntraTMP mode (single BV, Fusion of BVs, lineal filter mode, sub-pel precision, LIC) and an index to the best candidate in the list are signaled to the decoder.

In EE2-2.11[3], IntraTMP Auto-relocated block vector prediction (AR-BVP) based on merge candidates is investigated. Up to 10 merge candidates are derived from the neighboring PUs and those are used as guiding BVs to construct AR-BVPs by adding the BVs of the reference blocks. If an AR-BVP candidate is selected for refinement, it uses a refinement searching window of 3x3.

# Proposal

Refining candidates with overlapping search windows may be inefficient because it performs the exact cost computation twice for the overlapped positions. Furthermore, holding candidates on the list with overlapping windows reduces candidate diversity.

This contribution proposes clustering the candidates whose refinement windows overlap with other candidates. When a merge candidate overlaps a sparse candidate in the sparse list, the BV candidate with the lower template cost is selected and ranked in the sparse list for further refinement. In addition, a new refinement window that comprises both individual refinement windows is determined for the winning candidate.

Figure 2. Merge and sparse candidate refinement window overlapping example

This contribution proposes the auto-relocated BVP (AR-BVP) based on the sparse candidates instead of the merged candidates. The candiadates derived from the sparse candidate list are used as guiding BVs to construct AR-BVPs by adding the BVs of the reference blocks. The AR-BVP candidates are added to the merge list and the best 10 candidates compete with the sparse candidate using the TMP cost.

Additionally, it is observed that an overlap may occur between the IntraTMP block vectors in default and LIC mode. Although the two use different cost metric: SAD for default and MR-SAD for LIC, the overlap may occur in the sparse search as the the same starting point is used. It is therefore proposed to shift the LIC search region by half of the subsampling factor, as shown in the figure below:

Figure 3 Modified Search Range for metric. Positions labeled with “x” are the default positions used for the default metric. The others labeled with “o” are the ones used for the additional metric. The new positions are shifted by half of the subsampling range.

# Experimental Results

The proposed methods are tested on top of ECM-12.0 software [4] under JVET ECM common test conditions [5] in AI and RA configurations. The following results were obtained:

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | 0.04% | 0.02% | 100.3% | 98.5% | 100.3% | 99.1% |
| Class A2 | -0.09% | -0.03% | -0.12% | 102.7% | 103.9% | 98.8% | 98.4% |
| Class B | -0.11% | -0.05% | -0.03% | 103.2% | 108.7% | 99.4% | 99.0% |
| Class C | -0.09% | -0.15% | -0.02% | 102.1% | 104.8% | 100.1% | 99.8% |
| Class E | -0.14% | -0.07% | -0.31% | 103.7% | 107.3% | 101.0% | 99.5% |
| Overall | -0.09% | -0.06% | -0.08% | 102.5% | 105.0% | 99.9% | 99.2% |
| Class D | -0.04% | 0.23% | -0.15% | 102.7% | 103.0% | 99.9% | 100.0% |
| Class F | -0.19% | -0.12% | -0.15% | 102.1% | 106.3% | 98.9% | 99.5% |
| Class TGM | 0.00% | 0.00% | 0.00% | 97.1% | 97.1% | 100.0% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

# Conclusion

This proposal suggests clustering the merge and sparse candidates based on the overlapped reference window to determine a new refinement search window dimension for the clustered candidates. The proposed method yields coding gains with a slight increase in runtime. It is recommended for adoption in the upcoming EE2.

# References

- K. Naser, T. Poirier, F. Le Léannec, G. Martin-Cocher, “EE2: Intra Template Matching”, JVET-V0130, April 2021.

- K. Naser, M. Radosavljević, F. Le Léannec, Y. Chen, T. Dumas, T. Poirier,  " EE2-1.2: IntraTMP with Merge Candidates ", JVET-AG0151, January 2024.

- N. Qiu, J. Huo,Y. Ma, F. Yang, L. Zhang, Y. Yu, F. Wang, H. Yu, D. Wang, “EE2-2.11: Auto-relocated BVP for IntraTMP merge candidates”, JVET-AH0055, April 2024.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-12.0

- M. Karczewicz and Y. Ye, "Common Test Conditions and evaluation procedures for enhanced compression tool testing," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-AF2017, 32nd Meeting, Hannover, Oct. 2023.

Ofinno, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigitall may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).