| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0152-v1 |
| --- | --- |

| Title: | Ahg12: adaptive dual-tree coding in B slices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec, K. Naser, T. Dumas, T. Poirier, F. Urban, E. François | Tel: Email: | fabrice.leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution introduces adaptive dual tree selection for the coding of intra regions in inter slices for ECM, in a similar way as formerly proposed in JVET-L0424. That is, single tree representation is used from the CTU root down to a shared luma/chroma coding tree leaf. Resulting CU, if signaled in intra mode, can serve as the root of a dual tree representation of luma and chroma in considered intra region.

It is observed that proposed dual tree mode is frequently used for regions of inter slices where motion compensation fails in efficiently predicting blocks. Following objective performances are obtained in random access over ECM-12.0:

RA: 0,01% / -1,22% / -0,82% (Y / Cb / Cr)	101,4% (EncT)	99,4% (DecT)

# Introduction

The partitioning system of ECM is similar to that of VVC, where several constraints are removed for higher compression capability.

- The maximum CTU size is increased to 256 in class A.

- The latency constraint for small blocks in inter slices is removed.

- The VPDU constraint is removed for BT and TT partitions splitting and is no more considered for CCLM latency restriction or LMCS processing.

Thus, in ECM, dual tree is used by default for the coding of intra slices, whereas single tree is systematically employed in inter slices.

On the other hand, contribution JVET-L0424 proposes adaptive single/dual tree selection to code intra regions in inter slices, separate and shared luma/chroma tree support in intra slices, or separate tree selection at CTU level for intra-coded CTUs in inter slices.

This contribution proposes to use adaptive dual tree coding of intra regions in B slices, for ECM. This involves the following technical aspects.

- In inter slices, single tree coding is systematically used at least down to block size 64x64. Then, for a non-split CU with size at most 64x64 and corresponding to a leaf of the single tree, skip flag and coding mode are signaled. They are followed by a separate-tree CU-level flag, which, if true, indicates that the CU is the root of a further separate tree representation of luma and chroma. All coding units coded in this local dual tree mode in B slices are in intra mode. Thus, both IBC and Inter CUs are systematically coded in single tree mode.

- Dual tree usage is inferred for CU sizes higher than some pre-defined threshold in luma samples.

- Single tree usage is inferred for CU sizes lower than some pre-defined threshold in luma samples.

- In intra slices, only dual tree representation is used, regardless the Intra/IBC coding mode.

The principle of adaptive dual-tree/single-tree coding for intra regions in inter slices is illustrated by Figure 1.

Figure 1: principle of adaptive intra coding trees in inter slices.

Figure 2 illustrates ECM codec behavior obtained when using adaptive dual tree/single tree coding at coding tree node level, in an exemplary inter picture. Green partitions correspond to luma blocks, where intra prediction and dual tree are used. Blue partitions correspond to chroma blocks and are superimposed over green partitions where single tree is used.

One sees a significant number of blocks are selected in dual-tree mode. This typically corresponds to a high proportion of the picture areas where inter prediction fails in efficiently coding the blocks.

Figure 2: example of a JVET CTC B picture coded with adaptive dual tree. Green: luma block partitions. Blue: chroma block boundaries, superimposed over luma blocks boundaries when single tree is used.

# Experimental results

The following Table 1 shows the performances obtained over ECM-12.0 when using proposed adaptive dual tree coding of intra regions in inter slices.

Average coding gain of -1.2%/-0.8% is observed in chroma components in random access, with negligible average impact on luma compression efficiency, and limited encoder runtime increase. No decoding runtime change over ECM-12.0 is obtained.

Table 1: experimental results of proposed selective dual-tree coding of intra regions in inter slices over ECM-12.0

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0,03% | -2,02% | -0,45% | 103,5% | 96,4% | 102,3% | 100,6% |
| Class A2 | 0,07% | -0,64% | -0,50% | 103,4% | 97,4% | 102,3% | 100,8% |
| Class B | -0,01% | -1,70% | -1,60% | 98,7% | 100,0% | 102,2% | 100,7% |
| Class C | -0,04% | -0,45% | -0,35% | 101,8% | 102,6% | 101,9% | 100,6% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0,01% | -1,22% | -0,82% | 101,4% | 99,4% | 102,2% | 100,7% |
| Class D | 0,04% | -0,24% | -0,23% | 100,2% | 103,6% | 102,6% | 100,8% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

# Conclusion

Adaptive dual tree/single tree selection is proposed for the coding of intra rectangular regions in inter slices, in ECM.

The proposed extension of ECM partitioning leads to -1.2%/-0.8% coding gain in chroma components. Negligible impact on encoder runtime is observed, and decoding runtime is unchanged compared to ECM-12.0 anchor.

It is suggested to study the proposed adaptive dual/single tree coding of intra region in inter slices, in next cycle of EE2.

# Cited References

- JVET-L0424, “CE1 tests 3.1.1, 3.1.2, 3.2.1, 3.2.2: Separate intra trees”, 	K. Misra, A. Segall, F. Bossen (Sharp), 12th Meeting: Macao, CN, 3–12 Oct. 2018.

- JVET-AF2017, “Common test conditions and evaluation procedures for enhanced compression tool testing”, M. Karczewicz, Y. Ye, 32nd Meeting, Hannover, DE, 13–20 October 2023.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).