| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0188-v1 |
| --- | --- |

| Title: | EE1-2.1: HOP separate models |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Chaoyi Lin, Junru Li, Kai Zhang, Li Zhang (Bytedance) Franck Galpin (InterDigital) Dmytro Rusanovskyy (Qualcomm) Renjie Chang (Tencent) | Email: | yue.li@bytedance.com franck.galpin@interdigital.com dmytror@qti.qualcomm.com renjiechang@tencent.com |
| Source: | Bytedance, InterDigital, Qualcomm, Tencent |  |  |

_____________________________

- Abstract

This contribution presents the test result of EE1-2.1, where separate models are designed for filtering luma and chroma in HOP filter. Inference process is aligned with HOP.3 in NNVC-8.0. BD-rate changes on top of NNVC-8.0 HOP anchor (NN intra + HOP.3) are reportedly summarized as below:

RA: 0.03%, -2.17%, -2.84%, AI: -0.04%, -3.12%, -2.97%

kMAC/pixel: 459

Introduction

At JVET-AG meeting, a new filter architecture as shown in Fig. 1 was adopted as the HOP single model in NNVC-8.0 [1]. This HOP single model has a simplified residual block design compared with its predecessor. In JVET-AG0175 [2], considering that luma and chroma have different characteristics, separate models are proposed to filter luma and chroma components for HOP, as shown in Fig. 2 and Fig. 3.  In this EE test, the simplified residual block design from existing HOP single model is integrated into the HOP separate models.

Fig. 1. HOP architecture in NNVC-8.0.

Fig. 2. HOP luma model in JVET-AG0175.

Fig. 3. HOP chroma model in JVET-AG0175.

Proposed HOP Filter Architectures

In this contribution, separate filtering architectures for luma and chroma, as shown in Fig. 4 and Fig. 5, are developed on top of existing HOP single model.

Fig. 4. Proposed HOP luma model in EE1-2.1 test.

Fig. 5. Proposed HOP chroma model in EE1-2.1 test.

Inference

SADL is used for performing the inference of the proposed NN filters in VTM. Both weights and feature maps are represented with int16 precision using a static quantization method. The network information in the inference stage is provided in Table 1.

Table 1. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 1.40M |
|  | Parameter Precision (Bits) | 16 (I) |
|  | Memory Parameter (MB) | Total Parameters Number × 2 |
|  | Multiply Accumulate (kMAC/pixel) | 459 |
| Optional | Total Conv. Layers | 154 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144×144, 272×272 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Training

Same training procedure as HOP3 in NNVC-8.0 is applied. The network information in the training stage is provided in Table 2.

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: A100-SXM-80GB |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 20 |
|  | Batch size: | 64 |
|  | Training time: | 180h |
|  | Training data information: | DIV2K, BVI-DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | NNVC, QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |

Experimental results

The proposed model is integrated into NNVC-8.0 and tested according to the common test conditions defined in [3].

Models from Bytedance

The training has been performed independently by Bytedance.

Compared with NNVC-8.0 HOP anchor (nn intra + hop.3), the proposed method brings on average {0.03%, -2.17%, -2.84% } and {-0.04%, -3.12%, -2.97%} BD-rate changes for {Y, Cb, Cr} under RA and AI configurations.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 HOP3 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.04% | -1.16% | -0.70% | 100% | 101% |
| Class A2 | -0.07% | -2.69% | -7.93% | 99% | 103% |
| Class B | 0.05% | -4.09% | -2.07% | 97% | 100% |
| Class C | 0.06% | -0.12% | -1.60% | 100% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -2.17% | -2.84% | 99% | 102% |
| Class D | -0.01% | -0.64% | -0.75% | 99% | 104% |
| Class F | 0.12% | 1.00% | -0.80% | 102% | 105% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 HOP3 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.19% | -3.15% | 0.19% | 98% | 100% |
| Class A2 | -0.04% | -4.55% | -5.49% | 102% | 107% |
| Class B | -0.09% | -3.53% | -3.86% | 99% | 102% |
| Class C | 0.05% | -0.94% | -1.86% | 99% | 112% |
| Class E | 0.07% | -3.92% | -3.62% | 104% | 106% |
| Overall | -0.04% | -3.12% | -2.97% | 100% | 105% |
| Class D | 0.09% | -1.45% | -1.38% | 96% | 98% |
| Class F | -0.04% | -1.46% | -3.63% | 95% | 113% |

Models from InterDigital

Training

The same training has been performed independently by Interdigital. Validation loss profiles across epochs are consistent with HOP3 training. Final L2 stage seems to have better convergence with separated model.

Figure 1 Luma validation loss

Figure 2 Chroma validation loss

Performance

Model have been quantized on int16. The results are shown below.

Note: encoding/decoding times are not reliable.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 HOP |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.15% | -0.55% | 1.42% | 0.22% | -1.91% | 0.87% | 111% | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.03% | -4.33% | -1.19% | 0.24% | -4.81% | -1.34% | 112% | 125% |
| Class C | 0.13% | 0.00% | -0.78% | 0.16% | -0.61% | -1.35% | 111% | 127% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.10% | 0.05% | 0.71% | 0.44% | -1.68% | 0.97% | 110% | 128% |
| Class F | 0.24% | 1.76% | 1.04% | 0.18% | 0.06% | -0.37% | 107% | 127% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 HOP |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.20% | -3.34% | 0.72% | 0.28% | -4.29% | -0.70% | 113% | 123% |
| Class A2 | 0.00% | -5.19% | -6.70% | 0.10% | -5.68% | -5.89% | 116% | 126% |
| Class B | -0.15% | -4.98% | -3.81% | -0.14% | -5.27% | -3.74% | 109% | 124% |
| Class C | -0.04% | -0.96% | -1.38% | -0.01% | -1.84% | -2.63% | 108% | 127% |
| Class E | 0.05% | -3.72% | -3.09% | 0.03% | -3.81% | -3.59% | 105% | 127% |
| Overall | -0.07% | -3.64% | -2.88% | 0.03% | -4.17% | -3.32% | 110% | 125% |
| Class D | 0.01% | -1.42% | -0.83% | 0.02% | -3.00% | -2.80% | 103% | 119% |
| Class F | -0.17% | -0.45% | -2.13% | -0.38% | -1.60% | -3.48% | 108% | 127% |

Conclusion

This contribution presents a separate models-based design for HOP. The proposed filtering method brings coding gains compared with existing single model-based HOP.

References

- Y. Li, C. Lin, J. Li, K. Zhang, L. Zhang, D. Rusanovskyy, Y. Li, M. Karczewicz, R. Chang, L. Wang, X. Xu, S. Liu, EE1-1.1: Report on training with HOP architecture change for EE1-0 (variant 1), JVET-AG0174, Teleconference, January 2024.

- Y. Li, C. Lin, J. Li, K. Zhang, L. Zhang, F. Galpin, D. Rusanovskyy, R. Chang, EE1-1.3: Separate models for HOP filter, JVET-AG0175, Teleconference, January 2024.

- E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AF2016, Hannover, DE, October 2023.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).