| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0204-v1 |
| --- | --- |

| Title: | AGH9: Optical lens correction SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stephan Wenger Gilles Teniou | Email: | {swenger,teniou}@global.tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

Action cameras are more and more used in content acquisition, either targeting professional productions or consumer markets. Many of them have capabilities to capture scenes with a wider angle – field of view – than the conventional linear mode. Some of them even have fisheye capture capabilities resulting in longer depth of field. This contribution proposes to signal those camera and lenses parameters to enable the correction of the images to the rectilinear mode.

# Problem statement

## Ultra wide angle and fisheye cameras characteristics

The Fisheye lenses have an extremely wide field of view, often exceeding 180 degrees. They capture a spherical or hemispherical view, including a significant amount of distortion. Such a distortion results in either barrel distortion (outward curvature) or pincushion distortion (inward curvature) near the edges of the frame. Fisheye lenses also typically have very short focal lengths. The shorter the focal length, the greater potential depth of field.

Ultra-wide-angle lenses have also a wide field of view but do not typically reach the extreme degrees of coverage seen in fisheye lenses. They usually provide a field of view between 90 degrees and 120 degrees.

Although many action cameras have a linear mode for capturing a scene without distorsion, it remains a post-processing step on the captured picture with a native wide-angle lens as shown in the figure below:

In content authoring, when post processing is needed (e.g., color and contrast calibration, transition insertion, video effects…) it is usually better to operate on the native video instead of a post-processed version.

Lens distorsion parameters are then needed to be transmitted (when known) so that correction can be done on the last stage.

## Radial lens distortion

The radial distortion introducted by wide angle optics can be corrected with various correction models. Those models express the relation between the distance of a point (pixel) from the focal point (usually the center of the image) in the distorted image (noted rd) and in the undsitorted image (noted ru) as illustrated bellow:

Among the most popular models some of them are:

- The Poly3 model is a 3rd order polynomial model, defined as:

With k1 being the correction term. k1 value is in the range [-1,1] with a default value of 0 (i.e., no distortion). When k1 > 0 the barrel distortion is observed whereas k1 < 0 	corresponds to the pincushion effect.

- The Poly5 model is the 5th order polynomial model defined as:

With k1 being the quadratic correction term, k2 being the double quadratic term. k1 and k2 	values range in [-1,1] with a default value of 0 (i.e., no distortion).

- The Ptlens model is a more sophisticated model expressed as:

With “a”, “b”, and “c” being parameters corresponding to the physical properties of the 	lens in the range [-1,1] and equal to 0 by default (i.e., no correction).

With the transmission of the above parameters, the video can be corrected with post filters such that the "lenscorrection" filter from ffmpeg [1].

The identification of the parameters usually requires a lens calibration but those from the most popular cameras can be found in the lensfun database [2].

NOTE: It is proposed in the SEI message syntax to use a signed integer using a multiplication factor of 1000, since by experience, parameters are floats with 1/1000th precision. Other approaches may be chosen if found more appropriate by the group.

## Chromatic aberration

Light is electromagnetic radiation depending on its wavelength. Colors light with a long wavelength appears red the shorter this wavelength gets it first becomes yellow then green then blue purple until we can't see it anymore. When going through the edge of the lens the rays are oriented to a different angle depending on their wavelenght, that is called diffraction as shown below:

This effect is the cause of chromatic aberration, also known as color fringing, when the colors start to separate at the edges of a picture as illustrated below:

In this case transversal chromatic aberration (TCA) correction can compensate this effect.

There are several models, that all take the green component as the reference and try to align the red and blue with it.

- Linear TCA model is expressed as:

Where rd,R and rd,B are respectively the red and blue coordinates of a point in the distorted image, ru,R and ru,B respectively the red and blue coordinates in the undistorted corrected image, kr and kb the TCA parameters that default to 1.

- 3rd order polynomial TCA model is expressed as:

Where rd,R and rd,B are respectively the red and blue coordinates of a point in the distorted image, ru,R and ru,B respectively the red and blue coordinates in the undistorted corrected 	image, bR, cR, bB, vB, vR and vB the TCA parameters. bR, cR, bB, vB default to 0 and vR and vB are default to 1.

NOTE: It is proposed in the SEI message syntax to use a signed integer using a multiplication factor of 1000, since by experience, parameters are floats with 1/1000th precision. Other approaches may be chosen if found more appropriate by the group.

## Other effects

Other effects such as vignetting (the effect of light fall-off in the corners of images due to lens characteristics) are well known distorsions that can be corrected with the trnasmission of a limited number of parameters. Such aspects are proposed to be for further study, in case relevant. Also other radial and TCA correction models such as the Adobe Camera Model [3] could be addressed and signalled, this contribution is limited to the most popular correction models.

# Proposal

It is proposed to document the following closes (Syntax and Semantics) into the next version of the TuC.

## VSEI

| lens_optical_correction( payloadSize ) { | Descriptor |
| --- | --- |
| loc_cancel_flag | u(1) |
| if( !loc_cancel_flag ) { |  |
| loc_focal_parameters_present_flag | u(1) |
| if( loc_focal_parameters_present_flag ) { |  |
| loc_focal_center_x | u(16) |
| loc_focal_center_y | u(16) |
| loc_focal_length | u(16) |
| } |  |
| loc_radial_distortion_parameters_present_flag | u(1) |
| if( loc_radial_distortion_parameters_present_flag ) { |  |
| loc_radial_distortion_model_id | u(8) |
| if( loc_radial_distortion_model==0 ) { |  |
| rd_poly3_k1_times1000 | se(v) |
| } |  |
| if( loc_radial_distortion_model==1 ) { |  |
| rd_poly5_k1_times1000 | se(v) |
| rd_poly5_k2_times1000 | se(v) |
| } |  |
| if( loc_radial_distortion_model==2 ) { |  |
| rd_ptl_a_times1000 | se(v) |
| rd_ptl_b_times1000 | se(v) |
| rd_ptl_c_times1000 | se(v) |
| } |  |
| loc_tca_parameters_present_flag | se(v) |
| if( loc_tca_parameters_present_flag ) { |  |
| loc_tca_model_id | u(8) |
| if( loc_tca_model_id==0 ) { |  |
| tca_kr_times1000 | se(v) |
| tca_kb_times1000 | se(v) |
| } |  |
| if( loc_tca_model_id==1 ) { |  |
| tca_br_times1000 | se(v) |
| tca_cr_times1000 | se(v) |
| tca_vr_times1000 | se(v) |
| tca_bb_times1000 | se(v) |
| tca_cb_times1000 | se(v) |
| tca_vb_times1000 | se(v) |
| } |  |
| } |  |
| loc _persistence_flag | u(1) |
| } |  |
| } |  |

## Lens optical correction semantics

This SEI message provides the decoder with a lens distortion model to enable image correction.

loc_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous lens optical correction SEI message in output order that applies to the current layer. loc_cancel_flag equal to 0 indicates that lens optical correction follows.

loc_focal_parameters_present_flag equal to 1 indicates that focal parameters of the lens are present in the lens optical correction SEI message syntax. loc_focal_parameters_present_flag equal to 0 indicates that no focal parameter is present.

loc_focal_center_x defines the horizontal location in the picture of the focal point. It is by default half of the width of the picture.

loc_focal_center_y defines the vertical location in the picture of the focal point. It is by default half of the height of the picture.

loc_focal_lenght defines the focal length of the lens, i.e., the distance from the lens in which rays of light that fall parallel to the optical axis into the lens are converged. It is expressed in millimiters and is set by default to 0, meaning it is unkown.

loc_radial_distortion_parameters_present_flag equal to 1 indicates that radial distortion parameters are present in the lens optical correction SEI message syntax. loc_radial_distortion_parameters_present_flag equal to 0 indicates that no radial correction is expected.

loc_radial_distortion_model_id identifies the type of radial distortion model as specified in Table 1. The value of loc_radial_distortion_model_id shall be in the range of 0 to 2, inclusive. The values greater than 2 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore lens optical correction SEI messages with loc_radial_distortion_model_id greater than 2.

Table 1 – loc_radial_distortion_model_id

| Value | Description |
| --- | --- |
| 0 | Poly3 |
| 1 | Poly5 |
| 2 | Ptlens |

When loc_radial_distortion_model_id equals to 0 the polynomial 3rd order model is specified as

rd=ru(1−k1+k1r2u)		(1)

with rd referring to the distorted radius (normalised distance to image center), ru referring to the corrected undistorted radius, and k1 being the distortion coefficient.

When loc_radial_distortion_model_id equals to 1 the polynomial 5th order model is specified as

rd=ru⋅(1+k1r2u+k2r4u)		(2)

with rd referring to the distorted radius (normalised distance to image center), ru referring to the corrected undistorted radius, and k1 and k2 being the distortion coefficients.

When loc_radial_distortion_model_id equals to 2 the Ptlens model is specified as

rd=ru⋅(ar3u+br2u+cru+1−a−b−c) 		(3)

with rd referring to the distorted radius (normalised distance to image center), ru referring to the corrected undistorted radius, and a, b and c being the distortion coefficients.

rd_poly3_k1_times1000 indicates 1000 times the value of the k1 parameter from the Poly3 radial distorsion model from equation (1). rd_poly3_k1_times1000 is comprised between -1000 and 1000 and is by default set to 0.

rd_poly5_k1_times1000 indicates 1000 times the value of the k1 parameter from the Poly5 radial distorsion model from equation (2). rd_poly5_k1_times1000 is comprised between -1000 and 1000 and is by default set to 0.

rd_poly5_k2_times1000 indicates 1000 times the value of the k2 parameter from the Poly5 radial distorsion model from equation (2). rd_poly5_k2_times1000 is comprised between -1000 and 1000 and is by default set to 0.

rd_ptl_a_times1000 indicates 1000 times the value of the 'a' parameter from the Ptlens radial distorsion model from equation (3). rd_ptl_a_times1000 is comprised between -1000 and 1000 and is by default set to 0.

rd_ptl_b_times1000 indicates 1000 times the value of the 'b' parameter from the Ptlens radial distorsion model from equation (3). rd_ptl_b_times1000 is comprised between -1000 and 1000 and is by default set to 0.

rd_ptl_c_times1000 indicates 1000 times the value of the 'c' parameter from the Ptlens radial distorsion model from equation (3). rd_ptl_c_times1000 is comprised between -1000 and 1000 and is by default set to 0.

loc_tca_parameters_present_flag equal to 1 indicates that transversal chromatic aberration parameters are present in the lens optical correction SEI message syntax. loc_tca_parameters_parameters_present_flag equal to 0 indicates that no transversal chromatic aberration correction is expected.

loc_tca_model_id identifies the type of radial distortion model as specified in Table 2. The value of loc_tca_model_id shall be in the range of 0 to 1, inclusive. The values greater than 1 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore lens optical correction SEI messages with loc_tca_model_id greater than 1.

Table 2 – loc_tca_model_id

| Value | Description |
| --- | --- |
| 0 | Linear |
| 1 | Poly3 |

When loc_tca_model_id equals to 0 the linear model is specified as

rd,R =ru,RkR		(4)

rd,B =ru,BkB		(5)

with rd,R and rd,B referring to the distorted radius (normalised distance to image center) of the red and blue components respectively, ru,R and ru,B referring to the corrected undistorted radius of the red and blue components respectively, and kR and kB being the transversal chromatic aberration coefficients of the red and blue components respectively. kR and kB are equal to 1 by default.

NOTE: the transversal chromatic aberration is corrected in the linear model on the red and blue components whith the green component as the reference.

When loc_tca_model_id equals to 1 the polynomial 3rd order (Poly3) model is specified as

rd,R =ru,R⋅(bRr2u,R+cRru,R+vR)		(6)

rd,B =ru,B⋅(bBr2u,B+cBru,B+vB)		(7)

with rd,R and rd,B referring to the distorted radius (normalised distance to image center) of the red and blue components respectively, ru,R and ru,B referring to the corrected undistorted radius of the red and blue components respectively, and {br, cr, vr} and {bb, cb, vb} being the transversal chromatic aberration coefficients of the red and blue components respectively. kR and kB are equal to 1 by default. The br, cr, bb, cb coefficients are by default equal to 0 and the vr and vb coefficients are by default to 1.

NOTE: the transversal chromatic aberration is corrected in the Poly3 model on the red and blue components whith the green component as the reference.

tca_kr_ times1000 indicates 1000 times the value of the kr parameter from the linear transversal chromatic aberration model from equation (4). kr_ tca_times1000 is by default set to 1.

tca_kb_ times1000 indicates 1000 times the value of the kb parameter from the linear transversal chromatic aberration model from equation (5). kb_ tca_times1000 is by default set to 1.

tca_br_ times1000 indicates 1000 times the value of the br parameter from the Poly3 transversal chromatic aberration model from equation (6). tca_br_ times1000 is by default set to 0.

tca_cr_ times1000 indicates 1000 times the value of the cr parameter from the Poly3 transversal chromatic aberration model from equation (6). tca_cr_ times1000 is by default set to 0.

tca_vr_ times1000 indicates 1000 times the value of the vr parameter from the Poly3 transversal chromatic aberration model from equation (6). tca_vr_ times1000 is by default set to 1.

tca_bb_ times1000 indicates 1000 times the value of the bb parameter from the Poly3 transversal chromatic aberration model from equation (7). tca_br_ times1000 is by default set to 0.

tca_cb_ times1000 indicates 1000 times the value of the cb parameter from the Poly3 transversal chromatic aberration model from equation (7). tca_cr_ times1000 is by default set to 0.

tca_vb_ times1000 indicates 1000 times the value of the vb parameter from the Poly3 transversal chromatic aberration model from equation (7). tca_vr_ times1000 is by default set to 1.

loc_persistence_flag specifies the persistence of the lens optical correction SEI message for the current layer.

loc_persistence_flag equal to 0 specifies that the lens optical correction SEI message applies to the current decoded picture only.

loc_persistence_flag equal to 1 specifies that the lens optical correction SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a lens optical correction SEI message is output that follows the current picture in output order.

# References

[1] https://ffmpeg.org/ffmpeg-filters.html#lenscorrection

[2]	https://lensfun.github.io/lenslist/

[3]	http://download.macromedia.com/pub/labs/lensprofile_creator/lensprofile_creator_cameramodel.pdf.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).