| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0223-v1 |
| --- | --- |

| Title: | AHG12: Context retraining for ECM-12.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Muhammed Coban, Marta Karczewicz | Tel: Email: | vseregin @ qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents the results for all CABAC contexts retraining for ECM-12.0. Using ECM-12.0 as the anchor, test results are summarized as follows.

|  | Y | U | V | EncT | DecT | VmPeakE | VmPeakD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| All intra | -0.01% | -0.04% | 0.06% | 99.7% | 99.5% | 99.8% | 100.0% |
| Random access |  |  |  |  |  |  |  |
| Low delay B | -0.07% | -0.17% | 0.04% | 98.4% | 98.9% | 100.3% | 100.1% |
| Low delay P | -0.03% | -0.70% | 0.13% | 98.9% | 99.8% | 100.0% | 100.1% |

# Proposal

CABAC contexts are retrained for ECM-12.0 using the training script provided with ECM software package, which is applied to CTC bitstreams. Retrained contexts file is attached to this contribution.

Retrained contexts were implemented on top of ECM-12.0 and tested following common test condition described in JVET-AF2017.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | 0.04% | 0.02% | 99.5% | 100.3% | 100.0% | 100.0% |
| Class A2 | -0.01% | 0.04% | 0.06% | 99.0% | 98.7% | 100.0% | 99.9% |
| Class B | -0.02% | -0.01% | 0.05% | 100.7% | 100.1% | 99.3% | 100.0% |
| Class C | -0.02% | -0.06% | 0.05% | 100.1% | 99.5% | 100.0% | 100.0% |
| Class E | 0.00% | -0.20% | 0.14% | 98.7% | 98.8% | 100.0% | 100.0% |
| Overall | -0.01% | -0.04% | 0.06% | 99.7% | 99.5% | 99.8% | 100.0% |
| Class D | -0.07% | -0.01% | -0.21% | 101.4% | 102.2% | 100.0% | 100.0% |
| Class F | -0.06% | -0.17% | -0.38% | 97.3% | 88.0% | 100.0% | 100.0% |
| Class TGM | -0.12% | -0.09% | -0.18% | 97.9% | 88.7% | 100.0% | 99.9% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.08% | 0.01% | -0.07% | 100.0% | 100.7% | 99.9% | 100.2% |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.07% | -0.13% | -0.27% | 100.1% | 100.3% | 99.4% | 100.0% |
| Class C | -0.08% | -0.13% | 0.00% | 100.2% | 100.9% | 100.4% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.17% | -0.16% | 0.03% | 99.3% | 100.6% | 99.8% | 100.0% |
| Class F | -0.02% | -0.12% | -0.20% | 98.0% | 84.2% | 100.0% | 99.9% |
| Class TGM | 0.00% | -0.07% | -0.05% | 97.9% | 79.2% | 100.0% | 99.9% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.05% | -0.58% | -0.36% | 100.1% | 101.1% | 101.2% | 100.0% |
| Class C | -0.11% | -0.48% | -0.30% | 96.5% | 96.4% | 98.7% | 100.2% |
| Class E | -0.04% | 0.93% | 1.18% | 98.4% | 98.7% | 101.1% | 100.1% |
| Overall | -0.07% | -0.17% | 0.04% | 98.4% | 98.9% | 100.3% | 100.1% |
| Class D | -0.22% | -0.12% | 0.08% | 99.0% | 100.3% | 100.2% | 100.1% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.04% | -0.79% | -0.02% | 99.4% | 100.0% | 100.1% | 100.2% |
| Class C | -0.04% | -1.15% | -0.22% | 98.4% | 99.4% | 100.0% | 100.0% |
| Class E | -0.01% | 0.07% | 0.84% | 98.8% | 100.2% | 100.0% | 100.0% |
| Overall | -0.03% | -0.70% | 0.13% | 98.9% | 99.8% | 100.0% | 100.1% |
| Class D | -0.25% | -1.59% | -0.39% | 99.0% | 99.8% | 100.0% | 100.0% |
| Class F | -0.01% | 0.34% | 0.53% | 98.8% | 82.6% | 100.4% | 100.1% |
| Class TGM | -0.09% | -0.30% | -0.37% | 99.2% | 82.8% | 98.4% | 99.9% |

# Patent rights declaration(s)

Qualcomm Incorporated does not have any current or pending patent rights relating to the technology described in this contribution.