| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0246 |
| --- | --- |

| Title: | EE2-2.10h: MR-SAD/SATD based IntraTMP BV search with signalling and BV reordering with ARBVP for IntraTMP merge |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser, Fabrice Le Léannec ,Tangi Poirier, Hassane Guermoud ,Thierry Dumas Wei Chen, Xiaoyu Xiu, Xianglin Wang Nengfu Qiu, Junyan Huo, Yanzhuo Ma, Fuzheng Yang (Xidian University) Lai Zhang, Yue Yu, Fan Wang, Haoping Yu, Dong Wang (OPPO) | Email: | karam.naser@InterDigital.com chenwei06 @kwai.com nfqiu@stu.xidian.edu.cn zhanglai@oppo.com |
| Source: | InterDigital, Kwai, Xidian University, OPPO |  |  |

_____________________________

# Abstract

This contribution presents the simulation results of the EE test 2.10h which is a combination of Test-2.10a, 2.10c, 2.10d and 2.11c. The first test (2.10a) proposes a SATD based cost function to replace the current IntraTMP BV search on refinement search. The second test (2.10c) proposes additional cost metric for IntraTMP BV search for the sparse search. Specifically, a flag is signaled to the decoder to indicate the choice between the default metric (SAD) and the additional one (MR-SAD). The test 2.10d proposes a reordering method to reduce the signalling overhead of IntraTMP BV. In Test-2.11c, the auto-relocated BVP for IntraTMP merge candidates is introduced with sorting.

On top of the ECM12.0, the simulation results of EE2-2.10h are reported as:

AI: -0.19%	-0.20%	-0.19% 102.4% 102.9%

# Introduction

Intra template matching prediction (IntraTMP) is an intra prediction mode that predicts the current CU from the reconstructed regions within the current frame. The best prediction block is determined by matching the L-shaped reconstructed neighboring samples of the CU (based on the SAD cost) in a predefined search range. In ECM-12.0 [1], in the initial search, the IntraTMP search regions is subsampled a subsampling factor. After finding the best match, a refinement process is further applied by refining the BV around the best match within a reduced range.

In ECM-12.0, sub-pel precision is also enabled for IntraTMP, which includes three sub-pel positions (1/2-pel, 1/4-pel and 3/4-pel). For each sub-pel position, eight directions are supported. In the current design, the selected sub-pel BV is explicitly signaled in bitstream, which takes up to 6-bit signaling overhead.

There are multiple modes of IntraTMP in ECM: regular, fusion, sub-pel, FLM and LIC. By default, SAD is used as the template matching cost metric. Exceptionally, MR-SAD is used as a template cost when LIC is used. In ECM-12.0, the encoder generates two BV candidate lists: one for the default case and the other for LIC mode. The decoder generates one list depending on whether LIC usage.

In JVET-AG0063 [2] and JVET-AG0080 [3], the auto-relocated BVP (AR-BVP) for IntraTMP merge candidates were proposed. More specifically, in JVET-AG0063 the AR-BVP candidates are sorted together with other merge candidates per the template matching cost. In JVET-AG0080 the AR-BVP candidates are used as additional candidates on top of the original merge candidates. These two proposals are tested together in EE2 test 2.11.

# Proposed Method

In test 2.10a [4], it is proposed to use the SATD as the cost function for the integer BV search. To reduce the complexity, the SATD based cost function is only applied in the refinement process while the initial search still utilizes the SAD. Additionally, the SATD is always enabled at the refinement stage without any control signalling.

Test-2.10c proposes generalizing the idea of multiple metrics for IntraTMP. A CU-level signaling is proposed to alternate between SAD and MR-SAD. The encoder performs two template searches to generate two BV candidate lists (SAD and MR-SAD based), while the decoder performs SAD or MR-SAD template search depending on the CU flag.  The test further proposes reducing redundancies between the two lists by alternating the template search starting point for each metric. Specifically, MR-SAD template search is shift by half of the subsampling factor that is used for the sparce search.

In Test-2.10d [4], it is proposed to enable the reordering to reduce the signaling overhead of IntraTMP BVs. Specifically, a candidate list, which includes the integer-pel BV (output from the integer BV search) and 16 sub-pel BVs (which includes the BVs at 1/2-pel and 1/4-pel sub-pel positions with eight directions) around the selected integer-pel BV, is generated. The list is sorted based on the SAD cost between the template samples of the current CU and their corresponding prediction samples. After the reordering, the first two candidates are allowed to be selected with one single flag being signaled from encoder to decoder.

In test 2.11c [2], the auto-relocated BVP for IntraTMP merge candidates and the template matching based sorting process for IntraTMP merge candidates are tested. Up to 50 merge candidates can be derived from the neighboring PUs and by constructing AR-BVPs. Then the best 10 candidates are kept by ranking the template matching cost. The BVs of the reference blocks can also be directly used as AR-BVPs to get more available candidates. If an AR-BVP candidate is selected for refinement, it would have a refinement range of 3x3. SAD or MR-SAD can be used for calculating the cost depending on the same CU flags used in test  2.10c.

# Experimental Results

The proposed method is implemented on top of ECM-12.0 software. The following results are obtained under the common testing conditions [5]:

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.02% | -0.01% | 102.0% | 97.2% | 100.2% | 100.7% |
| Class A2 | -0.21% | -0.19% | -0.18% | 101.8% | 104.5% | 99.9% | 102.0% |
| Class B | -0.18% | -0.15% | -0.18% | 102.2% | 101.9% | 97.5% | 100.9% |
| Class C | -0.21% | -0.27% | -0.10% | 102.4% | 104.1% | 101.3% | 101.6% |
| Class E | -0.30% | -0.37% | -0.51% | 103.9% | 107.1% | 102.5% | 101.5% |
| Overall | -0.19% | -0.20% | -0.19% | 102.4% | 102.9% | 100.0% | 101.3% |
| Class D | -0.07% | 0.07% | -0.08% | 101.8% | 101.3% | 101.1% | 101.6% |
| Class F | -0.44% | -0.44% | -0.42% | 100.1% | 104.1% | 101.8% | 101.3% |
| Class TGM | -0.83% | -0.76% | -0.88% | 99.7% | 106.4% | 97.8% | 102.0% |

# References

[1] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-12.0

[2]	L. Zhang, Y. Yu, F. Wang, H. Yu, D. Wang, “EE2-1.2 related: AR-BVP for IntraTMP merge candidates”, JVET-AG0063, January 2024

[3]	N. Qiu, J. Huo, Y. Ma, F. Yang, “EE2-related: Extend block vector prediction for IntraTMP merge candidates”, JVET-AG0080, January 2024

[4] W. Chen, et. al., “EE2-2.10a and 2.10d: SATD based IntraTMP BV search without signalling and BV reordering”, JVET-AH0200, April. 2024.

[5] M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for the implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).