| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0249-v2 |
| --- | --- |

| Title: | AHG8: An adaptive spatial resampling method for machine vision |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shurun Wang Binzhe Li Jie Chen Yan Ye | Tel: Email: | {shurun.wsr, libinzhe.lbz, jiechen.cj, yan.ye}@alibaba-inc.com |
| Source: | Alibaba |  |  |

_____________________________

- Abstract

This contribution proposes an adaptive spatial resampling algorithm towards machine vision. For videos with high resolutions, spatial down-sampling is applied to the original video sequence before encoding to save coding bits and up-sampling is applied after decoding before feeding the videos to the machine vision model. It’s reported that by doing this, the following performance could be achieved on Class A and Class B sequences on SFU-HW dataset in terms of Pareto BD-rate:

For Class A, 35.58% (RA), -9.43% (LD), -14.38% (AI)

For Class B, -21.96% (RA), -23.01% (LD), -12.62% (AI)

For videos with low resolutions, spatial up-sampling is applied to the reconstruction after decoding, before feeding the videos to the machine vision model. It’s reported the following performance could be achieved on Class D sequences on SFU-HW dataset in terms of Pareto BD-rate:

For Class D, -5.35% (RA), -7.03% (LD), -2.13% (AI)

Description of adaptive spatial resampling for machine vision

In this contribution, an adaptive spatial resampling algorithm is proposed for the video sequences with resolution larger than or equal to 1920×1080 for machine vision. The flowchart of the proposed algorithm is shown in Fig. 1. The input visual data I is rescaled by a spatial down-sampling model before encoding, and then down-sampled data ID is encoded with the VTM encoder. The resampling ratio is set to 2. At the decoder side, the decoded data ID' is rescaled by a spatial up-sampling model. Finally, the up-sampled reconstructed data I' is fed to the machine vision model for machine task.

Fig. 1 Spatial resampling framework for videos with high resolutions.

The down-sampling model consists of three convolutional layers and the activation function is ReLU. The up-sampling model consists 20 convolutional layers with one residual connection from the network input to the last convolutional layer output. ReLU is adopted as the activation function of the first 19 convolutional layers and there is no activation function of the last convolutional layer. Detailed structure is shown in Fig. 2.

Fig. 2 The network structure of the downsampling (a) and upsampling (b) models. Specifically, the input channel cin, the output channel number cout, the kernel size k and the stride s of every convolutional layer are denoted as k-s-cin-cout.

For the video sequences with resolution lower than or equal to 416×240, it was noted that some objects tend to be too small for the machine task models to detect. Therefore, up-sampling is applied to the reconstruction after decoding, before feeding the videos to the machine vision model. The up-sampling ratio is set to 2 and Bicubic interpolation is used.

Experimental results

The experiment is conducted with VTM-22.2 under the VCM CTC [1]. The per-class performance on SFU-HW dataset for object detection is shown in Table 1. (Please note “/” in the tables indicates the performance cannot be calculated due to the nonmonotonic machine task performance). Obvious performance improvement under various configurations could be observed.

Table 1 Performance of adaptive spatial resampling for machine vision.

| RA | Object Detection | End-to-End BD-Rate [%] |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | mAP | mAP (low4) | Pareto mAP | End-to-End BD-mAP |
| SFU-HW | Class A | 35.58% | / | 35.58% | -2.66 |
|  | Class B | / | -20.29% | -21.96% | 3.73 |
|  | Class C | 0.00% | 0.00% | 0.00% | 0.00 |
|  | Class D | / | -5.37% | -5.35% | 0.27 |
| LD | Object Detection | mAP | mAP (low4) | Pareto mAP | End-to-End BD-mAP |
| SFU-HW | Class A | -9.43% | -9.54% | -9.43% | 1.96 |
|  | Class B | / | -22.22% | -23.01% | 5.85 |
|  | Class C | 0.00% | 0.00% | 0.00% | 0.00 |
|  | Class D | / | / | -7.03% | 0.55 |
| AI | Object Detection | mAP | mAP (low4) | Pareto mAP | End-to-End BD-mAP |
| SFU-HW | Class A | -14.38% | -25.83% | -14.38% | 2.04 |
|  | Class B | -12.62% | -10.65% | -12.62% | 5.13 |
|  | Class C | 0.00% | 0.00% | 0.00% | 0.00 |
|  | Class D | -2.13% | -3.61% | -2.13% | 0.15 |

Conclusion

In this contribution, an adaptive spatial resampling algorithm is proposed for machine vision. Simulation results show that the proposed algorithm can achieve obvious performance improvement for object detection with CTC model.

Thus, it is suggested to further study the adaptive spatial resampling algorithm for other machine tasks.

References

- S. Liu, C. Hollmann, “Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 output document JVET-AF2031, 32nd Meeting, Hannover, DE, 13–20 October 2023 (available online at https://jvet-experts.org/).

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).