| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0294 |
| --- | --- |

| Title: | Crosscheck of JVET-AH0244 (EE2-2.10c: MR-SAD based IntraTMP BV search with signalling) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Saverio Blasi | Email: | saverio.blasi@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution provides the crosschecking results of EE2-2.10c: MR-SAD based IntraTMP BV search with signalling.

# Introduction

This document reports the crosscheck results of EE2-2.10c: MR-SAD based IntraTMP BV search with signalling. This test proposes to introduce a CU-level flag in IntraTMP, to alternate between SAD and MR-SAD cost metric to decide the best IntraTMP BV candidate. The encoder performs two template searches to generate two BV candidate lists (SAD- and MR-SAD based), while the decoder performs SAD or MR-SAD template search depending on the CU flag. In addition, the test proposes reducing redundancies between the two lists. Specifically, the MR-SAD template search is shifted by half of the subsampling factor that is used for the other list.

It is confirmed that the partial simulation results match with those provided by the proponent. Please notice runtimes are not provided as simulations were run inconsistently from the anchors.

# Crosscheck results

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | 0.04% | 0.13% | #NUM! | #NUM! | 101.1% | #NUM! |
| Class A2 | -0.05% | -0.03% | -0.06% | #NUM! | #NUM! | 100.3% | #NUM! |
| Class B | -0.07% | -0.03% | 0.04% | #NUM! | #NUM! | 98.5% | #NUM! |
| Class C | -0.01% | -0.09% | 0.08% | #NUM! | #NUM! | 100.7% | #NUM! |
| Class E | -0.11% | -0.10% | -0.17% | #NUM! | #NUM! | 100.8% | #NUM! |
| Overall | -0.05% | -0.04% | 0.01% | #NUM! | #NUM! | 100.1% | #NUM! |
| Class D | -0.01% | 0.14% | 0.04% | #NUM! | #NUM! | 101.2% | #NUM! |
| Class F | -0.05% | 0.02% | -0.03% | #NUM! | #NUM! | 100.8% | #NUM! |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-12.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0.02% | -0.06% | 0.49% | #NUM! | #NUM! | 100.5% | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

# Patent rights declaration(s)

Nokia does not have any current or pending patent rights relating to the technology described in this contribution.