| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0311-v1 |
| --- | --- |

| Title: | AHG13: demonstration of FGC SEI editing, and live VVC web-player with grain synthesis |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Information |  |  |  |
| Author(s) or Contact(s): | P. de Lagrange | Email: | philippe.delagrange@interdigital.com |
| Source: | InterDigital |  |  |  |

_____________________________

# Abstract

This contribution describes a demonstration planned to be presented during the 34th JVET meeting, exercising an interactive FGC SEI editing tool as described in JVET-AG0324, and transparently decoding a VVC bitstream with film grain synthesis in real time through a web page.

# FGC SEI editing

The demonstration of interactive editing of FGC SEI is based on the VFGS tools from InterDigital [1][2], and while those tools have been slightly improved, the demonstration is still similar to the tutorial presented in JVET-AG0324:

- A graphical representation of the film grain characteristics helps understanding the model and capabilities of the FGC SEI message: essentially a modulation of grain strength and/or shape as a function of light intensity. Several example configurations tuned for real cases can be loaded.

- The user can interact with this graphical representation (moving interval boundaries, gain levels, frequency cutoffs), and save the results as a configuration file, that can be used to insert FGC SEIs in an HEVC or VVC bitstream with tools available in the HM and in the VTM.

- The user can have an immediate feedback on the changes made when a test video sequence is loaded (e.g. a bitstream where the grain has been removed or altered by encoding), making use of a film grain synthesizer provided in the same package, and reflecting what can be actually deployed in hardware

The tool currently exercises limited features of the FGC SEI, thus leaving room for improved grain quality, however the demonstration is useful to understand and exercise the FGC SEI model, and can even be used to generate useful FGC SEI messages.

# Web-based real-time VVC decoding and film grain synthesis

This part of the demonstration is based on VVdeC [3] and VVdeCWebPlayer [4] from HHI, integrating VFGS in the output stage of VVdeC. The patch on VVdeC is attached to this contribution.

The video sequences shown were prepared by encoding them using the VTM at different QPs, without priori denoising. For each sequence, different rates are chosen where film grain is just preserved, badly impaired, completely removed but keeping a good video quality, and at even lower quality.

Variants containing FGC SEI message are also prepared (using the tools described in the previous section), and allow:

- Comparing the rate difference and grain similarity when either encoding the grain natively, or making use of film grain synthesis (while letting the encoder remove grain).

- Assessing the visual improvement from film grain synthesis when the grain is impaired by encoding in an unpleasing fashion.

- Assessing the visual benefit of film grain synthesis when video quality is decreased further (hiding coding artefacts).

# Conclusion

A demonstration of a practical toolchain involving FGC SEI generation and live VVC decoding with grain synthesis is presented. Such a demonstration has been presented by InterDigital at DVB World Conference in Munich on March 19 and 20.

Important points seem to be the hardware implementation of grain synthesis, and fidelity of grain synthesis to the original: since the target use case is the distribution of high value movie content, where film grain can be part of the storytelling (giving value to grain fidelity), and where a secure path is key (giving value to a hardware implementation in the display pipeline).

# References

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-fgt/vfgs

- https://github.com/InterDigitalInc/VersatileFilmGrain

- https://github.com/fraunhoferhhi/vvdec

- https://github.com/fraunhoferhhi/vvdecWebPlayer