| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH0312-v1 |
| --- | --- |

| Title: | EE1-4 related: On training data of super resolution |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chaoyi Lin1, Yue Li2, Junru Li2, Kai Zhang2, Li Zhang2 1Bafangcheng 9th building F14 Yuhang District, 311100, Hangzhou City, Zhejiang Province, China 28910 University Center Lane San Diego, CA 92122, USA | Email: | linchaoyi.cy@bytedance.com yue.li@bytedance.com lijunru@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In the NNVC 8.0 software, the training data for the super resolution model is generated with NN tools disabled. However, in the inference stage, the NN tools are requied to be enabled. Thus, this contribution propose to train the SR model with the training data where NN tools are enabled. The experimental results are reported as:

The SR model (LOP SR + LOP2 LF + NN intra) reportedly shows on average {-1.04%, 1.15%, 1.21%} and {-0.79%, 0.95%, 1.59%} BD-rate changes for {Y, U, V} under RA and AI configurations, respectively.

# Introduction

The unified super resolution (SR) model is definded in [1] and it has been adopted into the NNVC 8.0 software in JVET-AG meeting. For this model, the NN tools (LOP2 and NN-intra) are disabled when generating the training data. However, in the inference stage, the NN tools are requied to be enabled. This contribution is proposed to align the training and inference process by generating the training data with NN tools enabled.

# Proposed Method

The network architecture of SR model is the same as [1]. When generate the training data, the LOP2 filter and NN-intra is enabled.

Figure 1 The SR architecture.

The network information in the inference stage is provided in Table 1.

Table 1 Network information for the proposed LOP SR model in inference stage.

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | Intel(R) Xeon(R) Platinum 8260 CPU @ 2.40GHz |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 0.08 M |
|  | Total Parameter Number | 0.08 M |
|  | Parameter Precision (Bits) | Int 16 |
|  | Memory Parameter (MB) | 0.16 MB |
|  | Multiply Accumulate (MAC) | 20.3 kMAC/pixel |

The network information in the training stage is provided in Table 2.

Table 2 Network information for the proposed LOP SR model in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Tesla-V100-SXM2-32GB |
|  | Framework: | PyTorch v1.11 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 45 |
|  | Batch size: | 16 |
|  | Training time: | 120h/model |
|  | Training data information: | DIV2K, BVI_DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | Turn on LOP2 and NN-intra |
|  | Loss function: | L1, L2 |

# Experimental results

The proposed SR model is tested on top of NNVC 8.0, according to the common test conditions [2]. The tested QP values are {22, 27, 32, 37, 42}.

The SR model (LOP SR + LOP2 LF + NN intra) reportedly shows on average {-1.04%, 1.15%, 1.21%} and {-0.79%, 0.95%, 1.59%} BD-rate changes for {Y, U, V} under RA and AI configurations, respectively.

Table 3 test results for integer model under RA configuration

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC 8.0 anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -3.73% | -0.69% | -0.48% | 84% | 70% |
| Class A2 | -1.45% | 6.43% | 6.54% | 87% | 81% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -1.04% | 1.15% | 1.21% | 94% | 89% |

Table 4 test results for integer model under AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -1.79% | 1.76% | 2.60% | 114% | 74% |
| Class A2 | -2.94% | 3.95% | 6.96% | 93% | 64% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | -0.79% | 0.95% | 1.59% | 101% | 88% |

# Conclusions

This contribution proposed to train the SR model with the training data where NN tools are enabled. The results show that the gain of the SR model can be further improved. It is suggested to adopted it in the NNVC software.

# References

[1] C. Lin, Y. Li, J. Li, K. Zhang, L. Zhang, “EE1-4.1: Unified CNN-based super resolution for resampling-based video coding”, JVET-AG0130, Jan. 2024.

[2] E. Alshina, R.-L. Liao, S. Liu, A. Segall, “Common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-AG2016, Jan. 2024.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).