| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes,FR, 17–24 April 2024 | Document: JVET-AH0326-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AH0142 (AHG4: HDR 4K content for JVET CTCs) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Fangjun Pu | Tel: Email: | fpu@dolby.com |
| Source: | Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

JVET-AH0142 proposes to include 4K HDR PQ content in the HDR common test conditions (CTCs). In total 6 HDR 4K clips are proposed. The current HDR PQ test set only comprises 7 clips with HD resolution. To be more in line with current commercial deployment of HDR, it is proposed to add 4 4K clips among the 6 proposed ones, and to reduce the HD PQ set to 4 or 5 clips. This contribution reports the crosschecking results of the proposed 6 HDR 4K clips with HM18.0 versus VTM23.0 for AI and RA configurations. First, it is confirmed that the MD5sum of the source YUV files after conversion match with that provided by the proponent. It is also confirmed that the crosschecking results of HM18.0 vs VTM23.0 for AI and RA configurations match with the proponent’s test results.

# Introduction

The HDR PQ content from the current JVET HDR CTCs (JVET-AC2011) is made of 7 clips of 1080p (HD) or lower resolution. Considering that HDR 4K is more in line with current commercial deployment of HDR, it is proposed to consider updating the HDR PQ test set by adding 4K clips. Some of the 7 HD clips could be removed.

The following 6 clips are proposed for consideration. They have been selected from JVET or 3GPP SA4 content repositories. It is suggested to consider 300 frames for each clip. Table 1 provides the comment parameters for all 6 clips. Detailed information for each clip is listed in Table 2.

Table 1 Common parameters for proposed 6 HDR 4K clips

| Parameter | Value |
| --- | --- |
| Resolution | 3840 x 2160 |
| Scan | progressive |
| Frame Rate | 60/1.001 |
| Bit Depth | 10 |
| Length | 300 |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT.2020 |
| Colour space | HDR PQ |

Table 2 Detailed information for proposed 6 HDR 4K clips

| Clip name | Original location / Md5sum of YUV | Snapshot |
| --- | --- | --- |
| Chimera7 | jvet@ftp.ient.rwth-aachen.de Md5sum of YUV: 3bbe6a5790ad6aed6995420a1473b3ae |  |
| Meridian1 | jvet@ftp.ient.rwth-aachen.de Md5sum of YUV: 063c3f98b5ef94c9e25a9f89dac16bd1 |  |
| Meridian2 | jvet@ftp.ient.rwth-aachen.de Md5sum of YUV: 0ff0affb5d3fe8cc1c98991d6584636b |  |
| Life Untouched | https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences Md5sum of YUV: e01292d0db0d74dc931506acb61dbcac |  |
| Meridian | https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences Md5sum of YUV: d055a94f35f7594776186fc5d09a9fa4 |  |
| Nocturne | https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences Md5sum of YUV: 9b8b72753926f11a7c7fc8545b5c78b7 |  |

# Crosscheck results

This crosscheck report tests the compression performance of the proposed 6 HDR 4K clips coded with HM18.0 and VTM23.0 for all intra (AI) and random access (RA) configurations under the common test conditions (CTCs). Results are provided in Table 3 and per sequence results are provided in Table 4.

Table 3 Compression performance of proposed 6 HDR 4K clips for AI and RA

|  | VTM23 Over HM18 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| AI | -41.29% | -23.76% | -23.37% | -47.48% | -68.42% | -22.64% | -44.28% | -67.12% | 707% | 118% |
| RA | -47.28% | -37.83% | -37.13% | -52.33% | -65.70% | -36.14% | -50.21% | -64.46% | 268% | 140% |

Table 4 Per sequence performance for AI and RA (VTM23 over HM18)

| Per sequnce for AI | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| --- | --- | --- | --- | --- | --- |
| ChimeraHDR7_3840x2160 | -36.93% | -29.58% | -28.07% | -35.12% | -50.41% |
| MeridianHDR1_3840x2160 | -53.64% | -31.62% | -30.92% | -62.25% | -73.33% |
| MeridianHDR2_3840x2160 | -32.24% | -17.50% | -17.82% | -42.91% | -70.43% |
| life_untouched | -47.35% | -22.28% | -21.46% | -32.50% | -66.59% |
| meridian_22412-22738 | -45.35% | -19.27% | -19.67% | -70.21% | -88.30% |
| nocturne | -32.22% | -22.28% | -22.29% | -41.91% | -61.46% |
| Overall | -41.29% | -23.76% | -23.37% | -47.48% | -68.42% |
| Per sequence for RA | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| ChimeraHDR7_3840x2160 | -44.85% | -41.66% | -40.88% | -44.34% | -51.00% |
| MeridianHDR1_3840x2160 | -47.99% | -42.08% | -41.23% | -52.64% | -53.30% |
| MeridianHDR2_3840x2160 | -49.17% | -46.28% | -45.56% | -44.42% | -66.13% |
| life_untouched | -55.86% | -36.10% | -33.94% | -44.18% | -68.27% |
| meridian_22412-22738 | -39.32% | -28.14% | -28.79% | -53.65% | -87.12% |
| nocturne | -46.49% | -32.70% | -32.39% | -74.73% | -68.38% |
| Overall | -47.28% | -37.83% | -37.13% | -52.33% | -65.70% |

From the compression results, it is observed that the performance for the new 4K PQ content is of same range as for current CTCs HD PQ content.

To pick up 4 HDR 4K clips among the proposed 6 clips, it is suggested that Meridian1, Meridian2, life_untouched and Nocturne may be a good selection set to cover the full range of coding complexity.

To reduce the current HDR HD test set, it is suggested to remove some of the following clips: EBU_Starting, SunRise (for those clips which provide lower HDR experience than the other clips), Cosmos1_TreeTrunk (which shows weird chroma samples statistics, and also is not true HD resolution).

# Patent rights declaration(s)

Dolby Laboratories, Inc. does not have any current or pending patent rights relating to the technology described in this contribution.